3GPP TSG-WG SA2 Meeting #164	S2-2407647

# Maastricht, Netherlands, August 19th – August 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1128 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | QoS and policy assistance service by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In FS_AIML_CN study phase, it is agreed that NWDAF can provide assistance information to help PCF to determine QoS and Policy parameters. Whether to define a new service or define a new analytics ID will be determined in the normative phase. We argue that defining a new service is the better way to go, due to the following reasons: 1, The assistance information is different from the conventional network analytics: In clause 5.1 in TS 23.288, analytics is defined as either statistical information of the past events, or predictive information. However, the assistance information is the candidate values of QoS parameters which can be used in the PCC rules in order to fulfil the target assistance goal, e.g., an high QoE. 2, The input parameters are different from the conventional analytics: the service consumer needs to provide candidate QoS parameters and the assistance target to NWDAF, which are not needed for the conventional analytics. It is not a good software design to impact existing analytics semantics to support QoS/policy assistance feature. 3, Easy extensibility: The new service can be easily extended to future use cases which require assistance information from NWDAF to fultil a target assistance goal or maximize the optimization goals. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clause for general description of Assistance to Parameter Determination functionality and specific description of assistance to QoS and Policy control Add service definition about Parameter Determiniation assistance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF will not provide assistance information to QoS and Policy parameter determinations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(new), 6.1D (new), 7.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# 5X	Assistance for Parameters Determiniation Functional Description

## 5X.1 General

Besides the Analytics information (which is either statistical information of the past events, or predictive information), NWDAF may also provide assistance information to help NWDAF service consumers to determine system parameters, e.g., QoS and policies.

The assistance information may include candidate parameters according to the request from the NWDAF service consumer, as well as the expected target (e.g., QoE) that may be achieved by applying those parameters.

## 5X.2 Assistance for QoS and Policy control

When PCF needs assistance information to determine QoS or policy parameters, it may send an assistance information request or subscription to NWDAF with one or multiple sets of QoS parameters and target service experience (e.g. QoE), detailed parameters is defined in clause 7.X. NWDAF may provide the candidate set(s) of QoS parameters and the corresponding expected QoE can be achieved by the candidate set(s) of QoS parameters (the candidate set(s) of QoS parameters are within the sets of QoS parameters provided by the PCF). The expected QoE(s) corresponding to the candidate set(s) of QoS parameters should fulfil the target service experience provided by the PCF.

PCF may take the assistance information into account to determine QoS parameters for a QoS flow, based on the operator's policy. Furthermore, according to the subscription, NWDAF may provide updated assistance information to PCF, then PCF may determine to modify the QoS parameters for a QoS flow and provide the modified QoS parameters to the SMF.

To provide the assistance information, NWDAF may consume service data and performance data collected from the AF (including the service data collected from the UE through the AF), the network data from other 5GC NFs and the network data from OAM as defined in Table 6.4.2-1, 6.4.2-1a, Table 6.4.2-2, Table 6.4.2-3, Table 6.4.2-4, Table 6.4.2-5 and Table 6.14.2-1. Furthermore, NWDAF may also consume analytics from other NWDAF(s), e.g., Observed Service Experience Analytics, Network Performance Analytics, User Data Congestion Analytics, QoS Sustainability Analytics and DN Performance Analytics.

# * * * * Next Change * * * *

## 6.1D	Assistance Information Provisioning for Parameters Determination

The following procedure is used for the NWDAF service consumer to subscribe the assistance information for parameters determination, e.g., QoS parameters and policies.

Figure 6.1D-1: Procedure for Assistance Information Provisioning for Parameters Determination

1.	NWDAF service consumer (e.g., PCF) sends a request to NWDAF to require assistance information for parameters determination by invoking Nnwdaf_ParameterDeterminationAssistance_Subscribe service. In the request, the service consumer may include an Assistance ID=QoS and Policy assistance which indicates what kind of assistance it needs, Assistance Filter (e.g., the assistance information is for which network slice, which area, etc.), Target of Assistance Information (e.g., for which UE it wants the assistance information), one or multiple sets of QoS parameters, the target QoE. Details of the parameters included in the request are defined in clause 7.X.2.

2.	In order to provide assistance information, NWDAF may collect service data and performance data from the AF (including the service data collected from the UE through the AF) as defined in clause 6.2.2.3, the network data from other 5GC NFs as defined in clause 6.2.2.2 and the network data from OAM as defined in clause 6.2.3. The details of the collected data are defined in Table 6.4.2-1, 6.4.2-1a, Table 6.4.2-2, Table 6.4.2-3, Table 6.4.2-4, Table 6.4.2-5 and Table 6.14.2-1. Furthermore, NWDAF may collect executed PCC rules and QoS parameters from PCF, it may also consume analytics from other NWDAF(s), e.g., Observed Service Experience Analytics, Network Performance Analytics, User Data Congestion Analytics, QoS Sustainability Analytics and DN Performance Analytics.

3.	The NWDAF provides the assistance information to the NWDAF service consumer via Nnwdaf_ParameterDeterminiationAssistance_Notify service. In the notification message, NWDAF may include candidate set(s) of QoS parameters which are within the sets of QoS parameters provided by the service consumer in Step 1 and the expected QoE(s) that may be achieved by applying those parameters. The details of the parameters in the notification message are defined in clause 7.X.4.

NWDAF may keep evaluating the provided assistance information based on the observed service experience and the target service experience. The NWDAF will provide the updated assistance information if needed by invoking the Nnwdaf_ParameterDeterminiationAssistance_Notify service operation. NWDAF may also collect information (e.g., executed PCC rules and QoS parameters) from NF consumer PCF, which can be used by NWDAF to evaluate the performance of assistance information generated. Furthermore, NWDAF may provide the evaluation (e.g., accuracy information) on the assistance information to service consumer if requested.

# * * * * Next change * * * *

## 7.X	Nnwdaf_ParameterDeterminiationAssistance service

### 7.X.1	General

Service Description: This service enables the consumer to subscribe assistance information for parameter determination, e.g., QoS and Policy parameters.

When the subscription is accepted by the NWDAF, the consumer NF, receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

### 7.X.2	Nnwdaf_ParameterDeterminiationAssistance Subscribe service operation

Service operation name: Nnwdaf_ParameterDeterminiationAssistance Subscribe

Description: Subscribes to assistance information for parameter determination.

Inputs, Required: Assistance ID (e.g., QoS parameter assistance), Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the subscription), Filter Information, i.e., AoI, DNN/S-NSSAI, Application Identifier, Assistance Goal (e.g., the target QoE to be achieved), one or multiple sets of QoS parameters, target period indicating when the assistance information is used.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

### 7.X.3	Nnwdaf_ParameterDeterminiationAssistance _Unsubscribe service operation

Service operation name: Nnwdaf_ParameterDeterminiationAssistance _Unsubscribe

Description: unsubscribe to assistance information for parameter determination.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

### 7.X.4	Nnwdaf_ParameterDeterminiationAssistance _Notify service operation

Service operation name: Nnwdaf_ParameterDeterminiationAssistance _Notify

Description: NWDAF notifies the (updated) assistance information for parameter determination.

Inputs, Required: Notification Correlation ID, Assistance ID.

Inputs, Optional: If the Assistance ID is QoS parameter assistance, NWDAF may provide candidate set(s) of QoS parameters and the corresponding expected QoE(s) can be achieved by applying the candidate set(s) QoS parameters (the candidate set(s) of QoS parameters are within the sets of QoS parameters provided by the PCF), Priority order indication.

NOTE:	Priority order indication indicates that the list of values of each QoS parameters may be ordered based on priority.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *