3GPP TSG-WG SA2 Meeting #164	S2-2407651

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0451 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support ProSe U2N Multihop Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-03 has conclusion that multihop U2N relay will be supported. Therefore, TS 23.304 shall be updated according to the conclusion of TR 23.700-03. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the Policy/Parameter provisioning of maximum number of hops for 5G ProSe UE-to-Network Relay and Remote UE. - Update QoS handling for 5G ProSe Layer-3 UE-to-Network Relay in terms of multi-hop. - Update the identifiers for multi-hop U2N Discovery with Model B. - New clause for multi-hop UE-to-Network Relay discovery with Model B. - New clause for Layer-2 link management over PC5 reference point for multi-hop Layer-3 UE-to-Network Relay after Model B Discovery with and without N3IWF support. - New clause for 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay after Model B Discovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | U2N multihop relay feature will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4, 5.6.2, 6.1.1.7.1, 6.1.2.3.1, 5.8.3.1, 6.3.2.3.1, 6.3.2.3.3a(new), 6.4.3.6a(new), 6.5.1.1a(new), 6.5.1.2a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.1.4.1	Policy/Parameter provisioning for 5G ProSe UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-Network Relay or a 5G ProSe Intermediate Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 and/or Layer-2 Remote UEs.

The authorisation for a UE to act as a 5G ProSe UE-to-Network Relay also authorizes the use of 5G ProSe UE-to-Network Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, optional Control Plane Security Indicator per RSC, optional Maximum Number of Hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. If the Control Plane Security Indicator is provided for a RSC, then the Control Plane based security procedure as described in clause 5.1.4.3.2 is performed for UE-to-Network Relay Communication for that RSC, otherwise if it is not provided for a RSC, then the User Plane based security procedure as described in clause 5.1.4.3.3 is performed for that RSC. The Maximum Number of Hops indicates the default hop limit when using the associated RSC for discovery procedure. RSC dedicated for emergency service may also be provisioned.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

-	Includes security related content for 5G ProSe UE-to-Network Relay, see TS 33.503 [29].

NOTE 2:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe UE-to-Network Relay, except for the Security parameters in bullet 2).

3)	For 5G ProSe Layer-3 UE-to-Network Relay, QoS mapping(s):

-	Each QoS mapping entry includes:

-	a mapping between a 5QI value and a PQI value;

-	a PQI PDB adjustment factor, for the PC5 communication for the 5G ProSe Layer-3 UE-to-Network Relay operation;

-	optional a hop adjustment factor, for the PC5 communication for multi-hop 5G ProSe UE-to-Network Relay operation;

-	optional the Relay Service Code(s) associates with the QoS mapping entry.

4)	For 5G ProSe Layer-3 UE-to-Network Relay to relay Ethernet or Unstructured traffic from 5G ProSe Layer-3 Remote UE by using IP type PDU Session:

-	Mapping of ProSe Service(s) to ProSe Application Server address information (consisting of IP address/FQDN and transport layer port number).

5)	Parameters to broadcast warning messages:

-	Configured Destination Layer-2 ID(s);

-	PC5 QoS parameters defined in clause 5.6.1;

-	the NR Tx Profile based on the configuration as specified in clause 5.1.3.1.

NOTE 3:	It is up to inter operator alignment and network configuration whether a common Destination Layer-2 ID or IDs are used for all UEs cross PLMNs in a particular region for Public Warning Notification Relaying.

6)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Remote UE and thereby enabling the use of a 5G ProSe UE-to-Network Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 and/or Layer-2 UE-to-Network Relay:

-	For 5G ProSe Layer-3 Remote UE, indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-Network Relay.

-	For 5G ProSe Layer-2 Remote UE, indicates the PLMNs in which the UE is authorized to use a 5G ProSe Layer-2 UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Remote UE also authorizes the use of 5G ProSe UE-to-Network Relay discovery with Model A and Model B.

NOTE 4:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe UE-to-Network Relay Discovery:

-	Includes the parameters for 5G ProSe Relay Discovery and for enabling the UE to connect to the 5G ProSe UE-to-Network Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer indicator per RSC, optional Control Plane Security Indicator per RSC, optional Maximum Number of Hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. If the Control Plane Security Indicator is provided for a RSC, then the Control Plane based security procedures as described in clause 5.1.4.3.2 is performed for UE-to-Network Relay Communication for that RSC, otherwise if it is not provided for a RSC, then the User Plane based security procedures as described in clause 5.1.4.3.3 is performed for that RSC. The Maximum Number of Hops indicates the default hop limit when using the associated RSC for discovery procedure. RSC(s) dedicated for emergency service may be provisioned to enable the support of emergency services for UE-to-Network Relaying.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement and Relay Discovery Additional Information messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic without using N3IWF access, or an indication of N3IWF access, for each ProSe Relay Service Code;

-	For 5G ProSe Layer-3 UE-to-Network Relay, optionally the ProSe application Traffic Descriptor(s) (as defined in TS 23.503 [9]) to be used for the relayed traffic for each ProSe Relay Service Code;

-	Includes security related content for 5G ProSe UE-to-Network Relay, see TS 33.503 [29].

3)	Policy/parameters for N3IWF selection for 5G ProSe Layer-3 Remote UE:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

NOTE 5:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Remote UE, except for the Security parameters in bullet 2).

4)	Parameters to receive warning messages:

-	Configured Destination Layer-2 ID(s);

-	PC5 QoS parameters defined in clause 5.6.1;

-	the NR Tx Profile based on the configuration as specified in clause 5.1.3.1.

NOTE 6:	It is up to inter operator alignment and network configuration whether a common Destination Layer-2 ID or IDs are used for all UEs cross PLMNs in a particular region for Public Warning Notification Relaying.

5)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe UE-to-Network Relay or a 5G ProSe Intermediate Relay as well as in the UE in support of the UE assuming the role of a 5G ProSe Remote UE and thereby enabling the use of a 5G ProSe UE-to-Network Relay:

1)	Radio parameters for 5G ProSe UE-to-Network Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

2)	Radio parameters for 5G ProSe UE-to-Network Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 7:	The validity time of these radio parameters is the same as the validity time of the Policy/Parameter listed above for 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE.

3)	Policy/parameters related to privacy:

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

# * * * * Next change * * * *

### 5.6.2	QoS handling for 5G ProSe UE-to-Network Relay operations

#### 5.6.2.1	QoS handling for 5G ProSe Layer-3 UE-to-Network Relay without N3IWF

For a 5G ProSe Layer-3 Remote UE accessing network via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF, the QoS requirement of the relay traffic between 5G ProSe Layer-3 Remote UE and UPF can be satisfied by the corresponding QoS control for the PC5 link between 5G ProSe Layer-3 Remote UE/intermediate relay and 5G ProSe Layer-3 UE-to-Network Relay/intermediate relay (PC5 QoS control) and the QoS control for the PDU session established between 5G ProSe Layer-3 UE-to-Network Relay and UPF (i.e. Uu QoS control). The PC5 QoS is controlled with PC5 QoS rules and PC5 QoS parameters (e.g. PQI, GFBR, MFBR, PC5 LINK-AMBR) as specified in clause 5.4 of TS 23.287 [2]. The QoS for the PDU session established between the 5G ProSe Layer-3 UE-to-Network Relay and UPF (i.e. Uu QoS control) is controlled with QoS rules and 5G QoS parameters (e.g. 5QI, GFBR, MFBR) as specified in clause 5.7 of TS 23.501 [4].

As shown in figure 5.6.2.1-1 below, the end-to-end QoS can be met only when the QoS requirements are properly translated and satisfied over the legs respectively.

Remote UE

Intermediare Relay(s)

NG-RAN

UE-to-NW Relay

PC5

PC5

Uu

N6

AS

PC5 QoS (PQI)

E2E QoS for a relay service

PC5 QoS (PQI)

Uu QoS (5QI)

5GC

Figure 5.6.2.1-1: End-to-End QoS translation for 5G ProSe Layer-3 UE-to-Network Relay operation

To achieve this, the QoS mapping can be pre-configured or provided to the 5G ProSe Layer-3 UE-to-Network Relay by the PCF using Prose Policy as specified in clause 5.1.4.1. The QoS mapping includes combinations of the 5QIs and PQIs mapping as entries. The PQI shall have standardized values as defined in Table 5.6.1-1 and in Table 5.4.4-1 of TS 23.287 [2]. The 5QI shall have standardized values as defined in TS 23.501 [4] clause 5.7.4. The QoS mapping also includes an adjustment factor for the PQI's PDB, e.g. 1/5 of the standardized PDB value in Table 5.6.1-1 and Table 5.4.4-1 of TS 23.287 [2].

If the QoS Flows setup are initiated by network, the SMF can base on the PCC rules or its local configuration to generates the QoS rules and QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR) and signal to the 5G ProSe Layer-3 UE-to-Network Relay using PDU Session Establishment/Modification procedure. For the PDU sessions used for relaying, the SMF always provides the QoS Flow level QoS parameters to the 5G ProSe Layer-3 UE-to-Network Relay when establishes a QoS Flow. Then the 5G ProSe Layer-3 UE-to-Network Relay decides the PC5 QoS parameters for the corresponding PC5 QoS Flow by determining the PQI based the QoS mapping and the GFBR and MFBR values for the PC5 GBR QoS Flow are set equal to the GFBR and MFBR values for the GBR QoS Flow respectively. The PCF differentiates the relay traffic based on either local configuration, e.g.by a dedicated DNN or S-NSSAI used for relay traffic or by the traffic filters.

For the case of multi-hop UE-to-Network Relay, the SMF generates the QoS rules and QoS Flow level QoS parameters and signals to the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay decides the PC5 QoS parameters for the corresponding PC5 QoS Flow by determining the PQI based on the QoS mapping as described above. These PC5 QoS parameters correspond to the QoS between the Remote UE and 5G ProSe Layer-3 UE-to-Network Relay. According to the received QoS info, the Intermediate Relay(s) split the QoS parameters based on implementation or hop adjustment factor (if available), into the QoS parameters of the previous hop and the QoS parameters from the Intermediate Relay to the Remote UE.

NOTE:	Separate QoS mappings can be configured for different RSCs.

If the 5G ProSe Layer-3 Remote UE initiates PC5 QoS Flows setup or modification during the Layer-2 link establishment or modification procedure, the 5G ProSe Layer-3 Remote UE provides the QoS Info as described in clause 6.4.3.6 to the 5G ProSe Layer-3 UE-to-Network Relay. The received PC5 QoS parameters of the QoS Info (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) are interpreted as the end-to-end QoS requirements by the 5G ProSe Layer-3 UE-to-Network Relay for the traffic transmission between 5G ProSe Layer-3 Remote UE and UPF. If the end-to-end QoS requirements can be supported by an entry in QoS mapping, the 5G ProSe Layer-3 UE-to-Network Relay uses the 5QI of the entry for the Uu QoS control and uses the PQI of the entry for the PC5 QoS control. If the end-to-end QoS requirements cannot be supported by any entries in QoS mapping, the 5G ProSe Layer-3 UE-to-Network Relay, based on its implementation, decides the 5QI for the Uu QoS control and PQI for the PC5 QoS control. The 5G ProSe Layer-3 UE-to-Network Relay provides the QoS Info (including PQI value chosen by the 5G ProSe Layer-3 UE-to-Network Relay) as part of the Accept message to the 5G ProSe Layer-3 Remote UE. If the 5G ProSe Layer-3 Remote UE performs the Layer-2 link modification procedure to add new PC5 QoS Flow(s) or modify the existing PC5 QoS Flow(s) for IP traffic or Ethernet traffic over PC5 reference point, the 5G ProSe Layer-3 Remote UE may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay may generate the Packet Filters used over Uu reference point based on the received PC5 QoS Rule(s).

For the case of multi-hop UE-to-Network Relay, the 5G ProSe Layer-3 Remote UE provides the QoS Info (including PQI) as described in clause 6.4.3.6 to the 5G ProSe Layer-3 Intermediate Relay. According to the received QoS Info, the Intermediate Relay(s) split the QoS parameters based on implementation or hop adjustment factor (if available), into the QoS parameters of the previous hop and the QoS parameters of rest of the path. The Intermediate Relay(s) send the PQI and the rest QoS parameters to the next hop, which is a 5G ProSe Layer-3 Intermediate Relay or a 5G ProSe Layer-3 UE-to-Network Relay. The 5G ProSe Layer-3 UE-to-Network Relay decides the Uu QoS and the PC5 QoS with the previous hop based on the entries in QoS mapping or implementation, as described above. The 5G ProSe Layer-3 UE-to-Network Relay includes the accepted QoS Info and the accumulated QoS Info in the Accept message, the accumulated QoS Info includes the Uu QoS and the PC5 QoS with the previous hop decided by the UE-to-Network Relay. The Intermediate Relay(s) decide the PC5 QoS parameters with its down stream Intermediate Relay/ Remote UE based on the rest QoS parameters previously split by itself and the received accumulated QoS Info. The Intermediate Relay(s) send the accepted QoS Info and accumulated QoS Info in the Accept message, the accumulated QoS Info includes Uu QoS and the PC5 QoS from the Intermediate Relay itself to the UE-to-Network Relay.

The 5G ProSe Layer-3 UE-to-Network Relay performs the UE requested PDU session Modification as defined in TS 23.502 [5], clause 4.3.3 for authorizing the requested QoS including the 5QI and the Packet Filters. If the PCF authorizes the requested QoS with a different 5QI value, the 5G ProSe Layer-3 UE-to-Network Relay may further update the PQI value based on the authorized 5QI value and the 5G ProSe Layer-3 UE-to-Network Relay performs the Layer-2 link modification procedure as defined in clause 6.4.3.6 to update the corresponding PC5 QoS Flow with the updated PQI value.

Alternatively, reflective QoS control over Uu as defined in TS 23.501 [4], clause 5.6.5.3 can be leveraged for dynamic QoS handling of 5G ProSe Layer-3 Remote UE to save on signalling between SMF and 5G ProSe Layer-3 UE-to-Network Relay. Upon reception of a DL packet with RQI on the Uu for the 5G ProSe Layer-3 Remote UE, based on the indicated QFI, the 5G ProSe Layer-3 UE-to-Network Relay creates a new derived QoS rule or updates existing derived QoS rule corresponding to the remote UE, as defined in TS 23.501 [4]. The derived QoS rule is for UL packets from the 5G ProSe Layer-3 Remote UE at Uu interface.

Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the 5G ProSe Layer-3 UE-to-Network Relay may generate the Packet Filters used over PC5 reference point and use the L2 Link Modification procedures as defined in clause 6.4.3.6 to either update existing PC5 QoS Flow(s) or to set up new PC5 QoS Flow(s) (when the QFI to PC5 QoS Flow mapping does not exist). The 5G ProSe Layer-3 UE-to-Network Relay may also provide the PC5 QoS Rule(s) for the PC5 QoS Flow(s) to be added or modified to the 5G ProSe Layer-3 Remote UE.

When the 5G ProSe Layer-3 UE-to-Network relay deletes the derived QoS rule e.g. after the RQ Timer expires, the 5G ProSe Layer-3 UE-to-Network Relay may perform L2 Link Modification procedures defined in clause 6.4.3.6 accordingly using the PQI mapped from the 5QI of the currently used QoS rule after the deletion of the derived QoS rule(s).

#### 5.6.2.2	QoS handling for 5G ProSe Layer-3 UE-to-Network relay with N3IWF

When accessing 5GS via a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the 5G ProSe Layer-3 Remote UE can request for PDU Session establishment or handover an existing PDU session to the N3IWF using UE requested PDU Session Establishment procedure defined in TS 23.502 [5] clause 4.12.5.

Remote UE

Layer-3 Intermediare Relay(s)

NG-RAN

Layer-3 UE-to-Network Relay

Relay UE

5GC

PC5

Uu

N6

UPF

(Relay)

PC5

N3IWF

Remote UE

5GC

UPF

(Remote)

End-to-End QoS for a relay service

IPSec security association

Figure 5.6.2.2-1: End-to-End QoS support via Layer-3 UE-to-Network Relay with N3IWF

For the 5G ProSe Layer-3 Remote UE's PDU session(s) established via N3IWF, QoS differentiation can be provided on per-IPsec Child Security Association basis. N3IWF determines the IPsec child SAs as defined in TS 23.502 [5] clause 4.12. The N3IWF is preconfigured to allocate different IPsec child SAs for QoS Flows with different QoS profiles.

The 5G ProSe Layer-3 Intermediate Relay neither selects N3IWF nor connects to N3IWF.

Based on configuration, the N3IWF can use one of the options below for QoS support in 5G ProSe Layer-3 UE-to-Network Relay UE's serving PLMN:

-	a static QoS mapping mechanism;

-	a dynamic QoS signalling based mechanism.

For the static QoS mapping mechanism, a SLA is established to govern the QoS handling between the 5G ProSe Layer-3 Remote UE's 5GC and the 5G ProSe Layer-3 UE-to-Network Relay UE's 5GC, e.g. when the RSC is configured. The SLA can include the mapping between the DSCP markings for the IPsec child SAs with the Remote UE and the corresponding QoS and N3IWF IP address(es). The non-alteration of the DSCP field between N3IWF and the 5G ProSe Layer-3 UE-to-Network Relay UE's UPF is also assumed to be governed by an SLA and by transport-level arrangements that are outside of 3GPP scope. The packet detection filters at the 5G ProSe Layer-3 UE-to-Network Relay UE's UPF can be based on the N3IWF IP address and the DSCP markings.

When the dynamic QoS signalling based mechanism is used by N3IWF, it works as follows:

-	When the 5G ProSe Layer-3 Remote UE establishes or handovers a PDU session via the N3IWF as described in clause 4.12.5 of TS 23.502 [5], the PCF serving the PDU Session in the 5G ProSe Layer-3 Remote UE's 5GC detects need for specific QoS and provides corresponding PCC rules to SMF in the 5G ProSe Layer-3 Remote UE's 5GC. The resulted QoS information is provided to N3IWF in step 2b of clause 4.12.5 of TS 23.502 [5]. The N3IWF determines the IPSec Child SA(s) and signals to the 5G ProSe Layer-3 Remote UE, as in step 4 of clause 4.12.5 of TS 23.502 [5] via IKE signalling including the PDU Session ID, the QFI(s), optionally a DSCP value and optionally the Additional QoS Information specified in clause 4.12.5 of TS 23.502 [5]. The PDU Session Establishment Accept message will be sent to the 5G ProSe Layer-3 Remote UE as in step 5 of clause 4.12.5 of TS 23.502 [5].

-	Based on Additional QoS Information received from the N3IWF, the 5G ProSe Layer-3 Remote UE determines whether it is necessary to request for QoS session modification for the dedicated QoS Flows toward the 5G ProSe Layer-3 UE-to-Network Relay as described in clause 5.6.2.1. The 5G ProSe Layer-3 Remote UE also provides the N3IWF address, DSCP and the SPI as the traffic filter to enable filtering and mapping of DL traffic towards the right PDU Session/QoS Flow within the 5G ProSe Layer-3 UE-to-Network Relay UE's 5GC.

NOTE:	This mechanism allows to communicate GBR related parameters such as GFBR and MFBR from the PCF of the 5G ProSe Layer-3 Remote UE via the N3IWF and the 5G ProSe Layer-3 Remote UE to the 5G ProSe Layer-3 UE-to-Network Relay UE. The 5G ProSe Layer-3 UE-to-Network Relay UE would be able to request the GBR resources from its serving network using UE requested PDU session modification as in clause 4.3.3. of TS 23.502 [5].

-	If the 5G ProSe Layer-3 UE-to-Network Relay performs the PDU Session Modification procedure, the PCF in the 5G ProSe Layer-3 UE-to-Network Relay UE's 5GC authorizes the QoS parameters. If the PDU Session Modification procedure authorized the requested QoS parameters, the 5G ProSe Layer-3 UE-to-Network Relay acknowledges the 5G ProSe Layer-3 Remote UE over PC5. The 5G ProSe Layer-3 UE-to-Network Relay also provides the traffic filter provided by the 5G ProSe Layer-3 Remote UE to the SMF during the PDU Session Modification procedure and the SMF updates the PSA UPF with DL Packet Detection Rules.

-	The PSA UPF in the 5G ProSe Layer-3 UE-to-Network Relay UE's 5GC maps the DL traffic from IPSec Child SA tunnel to appropriate PDU Session/QoS Flow considering SPI and N3IWF address (filters provided by the 5G ProSe Layer-3 Remote UE).

-	The 5G ProSe Layer-3 Remote UE's or the 5G ProSe Layer-3 Remote UE's 5GC may initiated PDU Session Modification procedures as specified in clause 4.12.6 of TS 23.502 [5]. When the 5G ProSe Layer-3 Remote UE received QoS information from the N3IWF, the same interactions between the 5G ProSe Layer-3 Remote UE and 5G ProSe Layer-3 UE-to-Network Relay and between the 5G ProSe Layer-3 UE-to-Network Relay and its 5GC as described above apply.

#### 5.6.2.3	QoS handling for 5G ProSe Layer-2 UE-to-Network Relay

For a 5G ProSe Layer-2 Remote UE accessing network via 5G ProSe Layer-2 UE-to-Network Relay, the existing 5G QoS control is reused between the 5G ProSe Layer-2 Remote UE and the 5G ProSe Layer-2 Remote UE's core network. The 5G ProSe Layer-2 Remote UE's SMF provides QoS profiles to NG-RAN, how NG-RAN performs QoS enforcement for PC5 interface (between the 5G ProSe Layer-2 Remote UE and 5G ProSe Layer-2 UE-to-Network Relay) and Uu interface (between the 5G ProSe Layer-2 UE-to-Network Relay and RAN) is specified in TS 38.300 [12].

# * * * * Next change * * * *

##### 6.1.1.7.1	5G ProSe Layer-3 UE-to-Network Relay

The UE-UE protocol stacks for discovery and PC5 link management as defined in clause 6.1.1.2 apply to 5G ProSe Remote UE, 5G ProSe Intermediate Relay and 5G ProSe Layer-3 UE-to-Network Relay. There could be zero, one or multiple 5G ProSe Intermediate Relay(s) between the Remote UE and UE-to-Network Relay. If there is no Intermediate Relay, the IP connection and PC5 link is from the remote UE to the UE-to-Network Relay.

Additionally, when N3IWF is supported by the 5G ProSe Layer-3 UE-to-Network Relay, the following control plane protocol stack apply.

EAP-5G

NAS

IP

PC5

IKEv2

EAP-5G

IP

PC5

Remote UE

Intermediate Relay(s)

PC5

U2N Relay

IP

Uu

Uu

RAN

N3 Stack

.

Relay

N3

stack

U2N Relay UPF

L2/L1

IP

PC5

PC5

Uu

N3

N6

Lower Layers

IP

N3IWF

N2

Stack

.

Relay

NAS

N2

N2

Stack

AMF

IKEv2

Legend:

-	NAS, EAP-5G and IKEv2 between the Remote UE and the N3IWF are defined in clause 8.2.4 of TS 23.501 [4].

Figure 6.1.1.7.1-1: Control plane protocol stacks between 5G ProSe Layer-3 Remote UE and N3IWF over 5G ProSe Layer-3 UE-to-Network Relay before the signalling IPSec SA is established

NAS

IP

PC5

IPSec

(tunnel mode)

Inner IP

TCP

IP

PC5

Remote UE

Intermediate Relay(s)

PC5

U2N Relay

IP

Uu

Uu

RAN

N3 Stack

.

Relay

N3

stack

U2N Relay UPF

L2/L1

IP

PC5

PC5

Uu

N3

N6

Lower Layers

IP

N3IWF

IPSec

(tunnel mode)

Inner IP

TCP

N2

Stack

.

Relay

NAS

N2

N2

Stack

AMF

Legend:

-	NAS, TCP and IPsec between the Remote UE and the N3IWF are defined in TS 23.501 [4] clause 8.2.4.

Figure 6.1.1.7.1-2: Control plane protocol stacks between 5G ProSe Layer-3 Remote UE and N3IWF over 5G ProSe Layer-3 UE-to-Network Relay after the signalling IPSec SA is established

# * * * * Next change * * * *

##### 6.1.2.3.1	5G ProSe Layer-3 UE-to-Network Relay

Legend:

-	GPRS Tunnelling Protocol for the user plane (GTP-U): This protocol tunnels user data between NG-RAN node and UPF as well as between the UPFs in the backbone network (not shown in the figure). GTP-U shall encapsulate all end user PDU packets.

-	SMF controls the user plane tunnel establishment and establishes User Plane Bearers between NG-RAN node and UPF.

-	UDP/IP: These are the backbone network protocols used for routing user data and control signalling.

-	Uu: The NR Uu radio protocols of NG-RAN between the UE-to-Network Relay and the NG-RAN node are specified in TS 38.300 [12].

-	PC5-U: The radio protocols between the UE and the UE-to-Network Relay are specified in clause 6.1.2.2.

Figure 6.1.2.3.1-1: User plane protocol stack for Layer-3 UE-to-Network Relay

PDU Layer

IP

PC5

IPSec

(tunnel mode)

Inner IP

GRE

IP

PC5

Remote UE

Intermediate Relay(s)

PC5

U2N Relay

IP

Uu

Uu

RAN

N3 Stack

.

Relay

N3

stack

U2N Relay UPF

L2/L1

IP

PC5

PC5

Uu

N3

N6

Lower Layers

IP

N3IWF

IPSec

(tunnel mode)

Inner IP

GRE

N3

Stack

.

Relay

PDU Layer

N3

Stack

N9

Stack

.

Relay

N3

N9

N9

Stack

UPF(PSA)

Legend:

-	IPSec, Inner IP and GRE between the UE and the N3IWF are defined in TS 23.501 [4] clause 8.3.2.

Figure 6.1.2.3.1-2: User plane protocol stacks for Layer-3 UE-to-Network Relay with N3IWF support

There could be zero, one or multiple 5G ProSe Intermediate Relay(s) between the Remote UE and UE-to-Network Relay. If there is no Intermediate Relay, the IP connection and PC5 link is from the Remote UE to the UE-to-Network Relay.

# * * * * Next change * * * *

#### 5.8.3.1	Common identifiers for 5G ProSe UE-to-Network Relay

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Announcer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Announcer Info: provides information (i.e. User Info ID) about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Solicitation message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoverer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe Remote-UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Discoverer Info: provides information (i.e. User Info ID) about the discoverer user.

-	Target Info: provides information (i.e. User Info ID) about the targeted discoveree user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the 5G ProSe Remote UEs interested in related connectivity services.

-	(Optional) Hop count: indicates the number of hops that the message is already relayed. It will be increased by 1 per hop.

-	(Optional) Maximum number of hops: a constant value that indicates the hop limit of the message.

-	(Optional) Path information: an (ordered) list of User Info ID of Intermediate Relays that indicates the transmitted path of the message.

The following parameters are used in the 5G ProSe UE-to-Network Relay Discovery Response message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoveree Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-Network Relay Discovery Solicitation message.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-Network Relay provides to 5G ProSe Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information (i.e. User Info ID) about the discoveree.

-	(Optional) Hop count: indicates the number of hops between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay on the selected path by the the 5G ProSe UE-to-Network Relay.

-	(Optional) Path information: an (ordered) list of User Info ID of Intermediate Relays that indicates the transmitted path of the message.

The following parameters may be used in the Relay Discovery Additional Information message (using Model A) based on the procedure defined in clause 6.5.1.3 for 5G ProSe UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID to send the Relay Discovery Additional Information message.

-	Destination Layer-2 ID: the Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

-	Announcer Info: provides information about the announcing user.

-	Additional parameters: the additional parameters for 5G ProSe Layer-3 UE-to-Network Relay (when applicable) are defined in clause 5.8.3.2.

NOTE 1:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication) in clause 6.4 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) with a different discovery model.

NOTE 2:	If a 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE from different PLMNs discover each other, it means that the Relay Service Code is associated with the same connectivity service, and the same Relay Service Code is provisioned based on Service Level Agreement among PLMNs.

NOTE 3:  The Hop count, Maximum number of hops and Path information are used for multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B. The multi-hop UE-to-Network Discovery message has no impact on 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE that only supports single-hop UE-to-Network Relay as, e.g., single-hop and multi-hop UE-to-Network Relay can be distinguished based on RSC.

# * * * * Next change * * * *

#### 6.3.2.3	5G ProSe UE-to-Network Relay Discovery

##### 6.3.2.3.1	General

5G ProSe UE-to-Network Relay Discovery is applicable to both 5G ProSe Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services. To perform 5G ProSe UE-to-Network Relay Discovery, the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay are pre-configured or provisioned with the related information as described in clause 5.1.

In 5G ProSe UE-to-Network Relay Discovery, the UEs use pre-configured or provisioned information for the relay discovery procedures as defined in clause 5.1.4.1.

The Relay Service Code (RSC) is used in the 5G ProSe UE-to-Network Relay discovery, to indicate the connectivity service the 5G ProSe UE-to-Network Relay provides to the 5G ProSe Remote UE. The RSCs (including the dedicated one(s) for emergency service) are configured on the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE as defined in clause 5.1.4. The 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE are aware of whether a RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service and whether a RSC is for emergency service, based the policy as specified in clause 5.1.4. A 5G ProSe UE-to-Network Relay supporting multiple RSCs can advertise the RSCs using multiple discovery messages, with one RSC per discovery message.

To support multi-hop 5G ProSe UE-to-Network Relay Discovery, a maximum number of hops is used in the 5G ProSe UE-to-Network Relay Discovery, which could be decided per RSC or by Remote UE based on QoS requirements. Hop count, maximum number of hops may be included in the discovery message. When relaying the discovery message, the 5G ProSe Intermediate Relay includes in the message its own User Info ID.

NOTE:	When the 5G ProSe UE-to-Network Relay can advertise the emergency RSC(s) is specified in clause 5.4.4.

Additional information not directly used for discovery can also be advertised using the PC5-D protocol stack in single or separate discovery messages of type "Relay Discovery Additional Information" as defined in clause 5.8.3.1.

##### 6.3.2.3.2	Procedure for 5G ProSe UE-to-Network Relay Discovery with Model A

Depicted in Figure 6.3.2.3.2-1 is the procedure for 5G ProSe UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: 5G ProSe UE-to-Network Relay Discovery with Model A

1.	The 5G ProSe UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The UE-to-Network Relay Discovery Announcement message contains the Type of Discovery Message, Announcer Info and RSC and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe Layer-3 UE-to-Network Relay shall only include a RSC in the UE-to-Network Relay Discovery Announcement when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the UE-to-Network Relay.

The 5G ProSe Remote UE (1 to 3) determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

5G ProSe Remote UE (1 to 3) monitors announcement messages with the 5G ProSe UE-to-Network RSC corresponding to the desired services.

Optionally, the 5G ProSe UE-to-Network Relay may also send Relay Discovery Additional Information messages as defined in clause 6.5.1.3. The parameters contained in this message and the Source Layer-2 ID and Destination Layer-2 ID used for sending and receiving the message are described in clause 5.8.3.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 1.

NOTE:	Access Stratum layer information used for 5G ProSe UE-to-Network Relay selection is specified in RAN specifications.

##### 6.3.2.3.3	Procedure for 5G ProSe UE-to-Network Relay Discovery with Model B

Depicted in Figure 6.3.2.3.3-1 is the procedure for 5G ProSe UE-to-Network Relay Discovery with Model B.

Figure 6.3.2.3.3-1: 5G ProSe UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Discovery Solicitation message contains the Type of Discovery Message, Discoverer Info, RSC and optionally Target Info and is send using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3. The 5G ProSe Remote UE discovering a 5G ProSe UE-to-Network Relay sends a solicitation message with the RSC which is associated to the desired connectivity service. The RSC is based on the Policy/Parameters specified in clause 5.1.4.1.

How the 5G ProSe UE-to-Network Relays (1 to 3) determine the Destination Layer-2 ID for signalling reception is specified in clause 5.8.3. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

2.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of a 5G ProSe UE-to-Network Relay, and the Target Info contained in the solicitation message, if any, matches the 5G ProSe UE-to-Network Relay, the 5G ProSe UE-to-Network Relays (e.g. 1 and 2) respond to the 5G ProSe Remote UE with a UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to-Network Relay Discovery Response message contains the Type of Discovery Message, Discoveree Info and RSC and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe UE-to-Network Relay shall only respond to a matching RSC in the UE-to-Network Relay Discovery Solicitation message when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the 5G ProSe UE-to-Network Relay.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 2.

##### 6.3.2.3.3a	Procedure for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B

intermediate Relay1

intermediate Relay3

intermediate Relay2

Remote UE1

……

U2N Relay

11. choose relay

PCF

0. ProSe Policy

1.decides max hop num

2a. U2N Relay Discovery Solicitation message

4a. U2N Relay Discovery Solicitation message

3a. decides to send Solicitation or Response

3b. decides to send Solicitation or Response

4b. Solicitation message

8a. Response

9a. Response

7. choose Relay

6b. Solicitation message

2b. U2N Relay Discovery Solicitation message

9b. Response

10b. Response

8b. Response

Figure 6.3.2.3.3a-1: General procedures for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE decides the maximum number of hops for discovery based on policy configuration (i.e. a mapping between maximum number of hops and RSC) or QoS parameters.

If the maximum number of hops is decided based on RSC or is decided as the same value as that based on RSC, the 5G ProSe Remote UE may not include the maximum number of hops in the Solicitation message. Otherwise the 5G ProSe Remote UE includes the maximum number of hops in the Solicitation message.

2a.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Discovery Solicitation message additionally contains following IEs compared with that in clause 6.3.2.3.3: an indication that multi-hop relay is supported, hop count and the maximum number of hops.

The Target Info may contain the User Info ID of UE-to-Network Relay and Intermediate Relay(s).

3a.	If the RSC contained in the Solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of a 5G ProSe Intermediate Relay, and the Target Info matches the User Info ID of the 5G ProSe Intermediate Relay (if any), the 5G ProSe Intermediate Relay may decide to send a 5G ProSe UE-to-Network Relay Discovery Solicitation message.

The 5G ProSe Intermediate Relay should drop the received Solicitation message when its own User Info ID is contained in the received Solicitation message, or if the hop count (corresponding to the number of Relays included in the message) has reached the maximum number of hops of the received Solicitation message. The maximum number of hops may be obtained from the Solicitation message or may be decided based on RSC if it is not obtained from the message.

The 5G ProSe Intermediate Relay may send a Response message when it has already found or established PC5 link with 5G ProSe UE-to-Network Relay(s), without sending Solicitation message. i.e. steps 4a-8a are skipped and step 9a is performed directly. The response message additionally contains the User Info ID of UE-to-Network Relay, path information to the UE-to-Network Relay which is an (ordered) list of User Info ID of intermediate Relay(s).

If the same Direct Discovery Set is received from different ProSe UEs, the 5G ProSe UE-to- Network Relay may select a Solicitation message to be sent to the next hop based on various criteria (e.g. hop count, delay, channel quality of received messages, etc.).

NOTE 1:	If the Source End UE does not receive any response after a timeout, based on application requirement, it may increase the maximum number of hops and send the discovery message again.

4a.	A 5G ProSe Intermediate Relay sends a Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID of Relays in the path that has relayed the Solicitation message. The hop count is increased by 1.

2b.-6b.	Another discovery path of the Solicitation message.

7-8. If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of the 5G ProSe UE-to-Network Relay, and the Target Info matches the User Info ID of the 5G ProSe UE-to-Network Relay (if any), then the 5G ProSe UE-to-Network Relay responds to the 5G ProSe Intermediate Relay with a 5G ProSe UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to- Network Relay Discovery Response message additionally contains the path information compared with that in clause 6.3.2.3.3.

The 5G ProSe UE-to-Network Relay may choose the path based on e.g. the PC5 signal strength of each message received, hops to the Remote UE, the path information, etc.

9-10.	A 5G ProSe Intermediate Relay sends a 5G ProSe UE-to-Network Relay Discovery Response message. The Response message additionally contains the path information.

11.	The Remote UE may perform relay path selection based on e.g. the PC5 signal strength and the number of hops to the 5G ProSe UE-to-Network Relay.

# * * * * Next changes * * * *

#### 6.4.3.6a	Layer-2 link management over PC5 reference point for Multi-hop 5G ProSe UE-to-Network Relay after Model B Discovery

The Layer-2 link procedures over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.6 can be used for the PC5 reference point among 5G ProSe Remote UE, 5G ProSe Intermediate Relay and UE-to-Network Relay after Model B Discovery, with the following differences and clarifications:

For the UE oriented Layer-2 link establishment as described in the clause 6.4.3.1,

-	The 5G ProSe Remote UE determines the destination Layer-2 ID for PC5 unicast link establishment based on the unicast source Layer-2 ID of the selected 5G ProSe Intermediate Relay (as specified in clause 5.8.3) during UE-to-Network Relay discovery as specified in clause 6.3.2.3.

-	The 5G ProSe Intermediate Relay determines the destination Layer-2 ID for PC5 unicast link establishment based on the unicast source Layer-2 ID of the selected 5G ProSe Intermediate Relay or UE-to-Network Relay (as specified in clause 5.8.3) during UE-to-Network Relay discovery as specified in clause 6.3.2.3.

-	5G ProSe Remote UE sends a unicast Direct Communication Request message to the selected 5G ProSe Intermediate Relay. The destination Layer-2 ID used to send the Direct Communication Request message shall be unicast Layer-2 ID. The Direct Communication Request message additionally includes:

-	Path information: an (ordered) list of User Info ID of 5G ProSe Intermediate Relays and the UE-to-Network Relay selected by the 5G ProSe Remote UE based on the path information provided to the 5G ProSe Remote UE during 5G ProSe UE-to-Network Relay Discovery procedure.

-	QoS Info: indicates the End to End QoS Info.

-	5G ProSe Intermediate Relay sends a unicast Direct Communication Request message to the next 5G ProSe Intermediate Relay or the UE-to-Network Relay according to the path information in the received Direct Communication Request message. The destination Layer-2 ID used to send the Direct Communication Request message shall be unicast Layer-2 ID. The Direct Communication Request message additionally includes:

-	Path information: an (ordered) list of User Info ID of 5G ProSe Intermediate Relays and the UE-to-Network Relay selected by the 5G ProSe Remote UE based on the path information provided to the 5G ProSe Remote UE during 5G ProSe UE-to-Network Relay Discovery procedure.

-	QoS Info: indicates the remaining QoS Info of hops from the Intermediate Relay to the network.

-	In step 4 and step 5, step 4a and step 5a are performed if the 5G ProSe Intermediate/UE-to-Network Relay's identity matches the Target Info (if any) and the Relay Service Code is one of the Relay Service Codes included during UE-to-Network Relay discovery as specified in clause 6.3.2.3.

For the Layer-2 link release as described in the clause 6.4.3.3,

-	If the Layer-2 link release procedure is initiated by the 5G ProSe Intermediate Relay, the Disconnect Request message may indicate the 5G ProSe UE-to-Network Relay is temporarily not available as described in clause 5.12.

-	If the service authorization for acting as a 5G ProSe Intermediate Relay is revoked, the 5G ProSe Intermediate Relay should initiate the release of the layer-2 link that the revoked authorization affects.

Each PC5 unicast link for 5G ProSe UE-to-Network Relay is associated with a Unicast Link Profile, which additionally includes:

-	Path Information: which contains the User Info ID of Intermediate Relay at the next hop to the Remote UE.

Each PC5 unicast link for 5G ProSe Intermediate Relay is associated with a Unicast Link Profile, which additionally includes:

-	Path Information: which contains the User Info ID of Intermediate Relay or UE-to-Network Relay at the adjancent hop.

# * * * * Next changes * * * *

#### 6.5.1.1a	5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay without N3IWF after Model B Discovery

Remote UE

Intermediate Relay

U2N Relay

NG-RAN

AMF

SMF

UPF

1. 5GS Registration, authorization and provisioning for U2N Relay and Intermediate Relays

1. 5GS Registration, authorization and provisioning for Remote UE

3. Multi-hop U2N Relay Discovery Procedure

4. Establishment of connection for unicast PC5 communication

4. U2N Relay may establish new PDU session(s) for relaying

5. IP address/prefix allocation

6. Layer-2 link modification

2. PDU Session Establishment

6. U2N Relay may modify existing PDU Session for relaying

7. Remote UE Report (Remote UE ID, Remote UE info)

Relayed traffic

Figure 6.5.1.1a-1: Connection establishment over Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support

Connection establishment over Multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF support as described in clause 6.5.1.1 can be used for multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

1.	Service authorization and provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay, Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE.

2.	The 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying.

3.	The 5G ProSe Layer-3 Remote UE performs Model B discovery of a 5G ProSe Layer-3 UE-to-Network Relay. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure.

4.	The 5G ProSe Layer-3 Remote UE selects a path and establishes a connection for unicast mode communication.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, the UE-to-Network Relay assigns IPv6 prefix or IPv4 address to Intermediate Relay(s) and the Remote UE. The Intermediate Relay act as DHCP proxy or relay the IP allocation message (e.g. Router Solicitation, Advertisement).

6.	The 5G ProSe Layer-3 Remote UE may provide PC5 QoS Info and PC5 QoS rule(s) to the 5G ProSe Layer-3 UE-to-Network Relay using Layer-2 link modification procedure via Intermediate Relay(s). Intermediate Relays may split the PC5 QoS hop-by-hop as described in clause 5.6.2.

7.	For the Remote UE Report, the Remote User ID is an identity of the 5G ProSe Layer-3 Remote UE.

For IP PDU Session Type, the Remote UE info is IP info related to Remote UE.

-	for IPv4, the 5G ProSe Layer-3 UE-to-Network Relay shall report IP address and TCP/UDP port ranges that were assigned to 5G ProSe Layer-3 Remote UE(s) by the UE-to-Network Relay;

-	for IPv6, the 5G ProSe Layer-3 UE-to-Network Relay shall report IPv6 prefix(es) assigned to individual 5G ProSe Layer-3 Remote UE(s).

For Ethernet PDU Session Type, the Remote UE info is Remote UE MAC address which is detected by the 5G ProSe Layer-3 UE-to-Network Relay.

NOTE:	Intermediate relays forward the Downlink packet to the correct PC5 links based on destination IP address in the packet.

#### 6.5.1.2a	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay with N3IWF after Model B Discovery

5G ProSe Communication via 5G ProSe Layer-3 UE-to-Network Relay with N3IWF as described in clause 6.5.1.2 can be used for multi-hop UE-to-Network Relay after Model B Discovery with the following differences and clarifications:

Remote UE

Intermediate Relay

U2N Relay

NG-RAN

AMF

SMF

UPF

N3IWF

1. 5GS Registration and/or PDU Session connectivity (UE PRoSe Policy)

1. 5GS Registration, authorization and provisioning (UE ProSe Policy, URSP)

2. Multi-hop U2N Relay Discovery Procedure

3. Establishment of connection for one-to-one PC5 communication session

3. U2N Relay may establish new PDU session(s) for relay

4. IP address/prefix allocation

5. Remote UE selects an N3IWF and obtains its IP address

6. Remote UE performs NAS Registration and establishes IPSec tunnel using IKE procedures with N3IWF as provided in Figure 4.12.2.2-1 of TS 23.502

7. Additional Child SA, configuration and QoS policies are exchanged as specified in TS 23.304

Figure 6.5.1.2a-1: 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support

1.	5G ProSe Layer-3 UE-to-Network Relay performs Registration procedures and obtains the ProSe Policy that corresponds to the operation supporting the access to N3IWF. The 5G ProSe Layer-3 Remote UE is configured with the corresponding ProSe Policy and URSP rules.

Supporting of the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF is preconfigured or provisioned to the Intermediate Relay by the ProSe Policy.

2-4. A 5G ProSe Layer-3 UE-to-Network Relay, 5G ProSe Intermediate Relay(s) and 5G ProSe Layer-3 Remote UE perform multi-hop discovery using the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF. The Remote UE obtains the path information to the UE-to-Network Relay(s) from the discovery procedure.

5.	The 5G ProSe Layer-3 Remote UE that connects to a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support selects an N3IWF and determines the N3IWF IP address. The 5G ProSe Layer-3 Intermediate Relay neither selects N3IWF nor connects to N3IWF.

6.	The 5G ProSe Layer-3 Remote UE establishes a signalling IPsec tunnel using IKE procedures with a N3IWF via Intermediate Relays and UE-to-Network Relay performs NAS Registration.

7.	Based on Additional QoS Information received from the N3IWF, the 5G ProSe Layer-3 Remote UE determines whether it is necessary to request for QoS session modification for the dedicated QoS Flows toward the 5G ProSe Layer-3 UE-to-Network Relay, taking the number of hops into account.

# * * * * End of changes * * * *