SA WG2 Meeting SA2#164	S2-2407687

19 - 23 August, 2024, Maastricht, Netherlands	(revision of S2-240XXXX)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2130 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6, Support of ECN marking for 5G RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per WT#3.1 (SP-240633), L4S is supported in 5G-RG: WT#3.1 KI#6 Further enhancement to support XR based on non-3GPP access. - Support L4S in wireline access (W-AGF, 5G-RG). and conclusion of KI#6 in TS 23.700-70: 2. For wireline access: b. ECN marking for L4S is supported in 5G-RG in UL. It is controlled via N1 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s). L4S (Low Latency, Low Loss and Scalable Throughput) is described in IETF RFC 9330, IETF RFC 9331 and IETF RFC 9332. It exposes congestion information by marking ECN bits in the IP header of the user IP packets between the UE and the application server to trigger application layer rate adaptation. As the congestion information may change dynamically, if the Indication of ECN marking for L4S is included in the QoS rule, it may lead to a frequent update of QoS rule due to Indication of ECN marking for L4S. Hence it is proposed to send this Indication of ECN marking for L4S using a separate indicator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support of ECN marking for L4S in 5G-RG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of ECN marking for L4S in 5G-RG |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 23.501 CR 5440 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

4.x	Support of ECN marking for L4S

To enable ECN marking for L4S by a 5G-RG, an ECN marking for L4S indication is sent associated with a QoS rule in PDU session establishment procedure or PDU session modification procedure as defined in clause 5.37.3.x of 3GPP TS 23.501[2]. Based on the ECN marking for L4S indication, 5G-RG maps the L4S-enabled QoS Flow(s) to L4S-enabled wireline QoS resources(s) and perform ECN bits marking for L4S in UL direction.

NOTE 1:	How the congestion information is converted to ECN markings is 5G-RG implementation specific.

To enable ECN marking for L4S by a W-AGF, an ECN marking for L4S indication is sent to W-AGF as defined in clause 5.37.3.x of 3GPP TS 23.501[2]. Based on the ECN marking for L4S indication, W-AGF maps the L4S-enabled QoS Profile(s) to L4S-enabled wireline QoS resources(s) and perform ECN bits marking for L4S in corresponding direction.

NOTE 2:	How the congestion information is converted to ECN markings is W-AGF implementation specific.

End of Changes