SA WG2 Meeting #164	S2-2407699

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-240xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0232 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | I-SMF insertion in EAS discovery for local service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-07-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in conclusion, I-SMF based approach is considered to reduce the impact on central 5GC NFs. The following contents are agreed in TR conclusion and are related to this CR changes: - I-SMF selection shall be performed by AMF. I-SMF selection mechanism shall be enhanced to support I-SMF for local offloading management, e.g. I-SMF selection for local offloading management is based on the subscription data. - EASDF configuration and DNS message handling is performed by I-SMF (instead of SMF). - EASDF discovery and selection is performed by I-SMF (instead of SMF). When the I-SMF approach is used the SMF does not discover and select an EASDF. - I-SMF retrieves EDI from NEF to configure EASDF. EDI management in the I-SMF shall be supported. I-SMF can also be preconfigured with the EAS deployment information. - I-SMF can be configured with or receive the local offloading policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this paper, following contents are introduced: - Add related abbreviations - Add the figures that reflecting the I-SMF insertion for EAS discovery - New procedure for I-SMF related to support EAS discovery - I-SMF retrieves EDI from NEF to configure EASDF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support the local area network configuration is not clear, and how to I-SMF to reduce the impact to 5GC NFs is FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2, 6.x(all new text), 7.1.1, 7.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [2].

C-DNS	Central DNS

C-NEF	Central NEF

C-PSA UPF	Central PSA UPF

EAS	Edge Application Server

EASDF	Edge Application Server Discovery Function

ECS	Edge Configuration Server

EDC	Edge DNS Client

EDI	Edge Deployment Information

EEC	Edge Enabler Client

EES	Edge Enabler Server

EHE	Edge Hosting Environment

I-SMF	Intermediate SMF

L-DN	Local part of DN

L-DNS	Local DNS

L-NEF	Local NEF

L-PSA UPF	Local PSA UPF

HR-SBO	Home Routed Session BreakOut

* * * Start of Change 2 * * *

4.2	Reference Architecture for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501 [2]. The following reference architectures for non-roaming, LBO roaming and HR with Session Breakout (HR-SBO) roaming scenarios further depict the relationship between the 5GS and a DN where Edge Application Servers (EASs) are deployed in an EHE.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

Figure 4.2-5 depicts 5GS architecture for HR-SBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-5: 5GS providing access to EAS with UL CL/BP for HR-SBO roaming scenario

Figure 4.2-x depicts 5GS architecture for I-SMF deployment managing local area scenario supporting Edge Computing with UL CL/BP insertion for non-roaming scenario.

Figure 4.2-x: I-SMF deployment managing local area scenario supporting Edge Computing for non- roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

NOTE 3:	For the HR-SBO roaming scenario, there can be other UPF(s) located in VPLMN between the UPF acting UL CL/BP and the UPF acting as remote PSA in HPLMN.

* * * Start of Change 3(all new text) * * *

6.X	Support the local traffic offloading with I-SMF deployment

6.x.1	General

In order to support to reduce impact on central 5GC NFs, when UE establishes the PDU session and the subscription data indicates possible local offloading or the I-SMF selection to reduce the central 5GC NFs load, the the I-SMF is selected for EAS discovery and local offloading management.

If the subscription data indicate the possible local offloading or the I-SMF selection to reduce the central 5GC NFs load, during the EAS discovery procedure, the I-SMF replacing the SMF, is selected by AMF. During the SM policy association procedure, the PCF generates the local offloading policy and provide the local offloading policy to SMF. The SMF further provides the local offloading policy to I-SMF transparently. The I-SMF, replacing the SMF, is responsible for selecting EASDF and configuring DNS handling rules to EASDF. The I-SMF reuses the procedure of EDI management procedure to retrieve the EDI from UDR.

When receiving the DNS response from EASDF, if the EAS IP address or FQDN is in the scope of local offloading policy, the I-SMF inserts the local PSA and configures the local PSA with the traffic rule. The I-SMF performs the perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

6.x.2	Procedure

6.x.2.1	General

This clause describes the local offloading management procedure using I-SMF insertion during EAS discovery procedure.

6.x.2.2	I-SMF insertion for local service in EAS discovery

If supports local offloading to reduce impact on central 5GC NFs, the following procedure applies.

Figure 6.x.2.2-1: Procedure of I-SMF deployment to support EAS discovery

1.	The same procedure as defined from step 1 in section 6.2.3.2.2 with the following differences:

The SMF retrieves the Session Management Subscription data from UDM to indicate the possible local offloading or the I-SMF selection to reduce the central 5GC NFs load.

The SMF replies the indication of possible local offloading or the I-SMF selection to AMF.

2.	The AMF selects the I-SMF section 4.23.2 of TS 23.502[3] with the following changes:

The AMF may select the I-SMF according to the subscription data which indicates the supporting local handling of edge computing.

3-4.	The AMF performs the Nsmf_PDUSession_CreateSMContext Request/Response, the same as in step 3 and step 5 in section 4.3.2.2.1 of TS 23.502[3]. The only difference is the SMF ID is included in Nsmf_PDUSession_CreateSMContext Request.

5.	The I-SMF performs the Nsmf_PDUSession_Create Request to request the local offloading policy.

6-7.	The SMF performs the procedure from step 1 to step 5 as indicated in section 4.16.4 of TS 23.502[3] to retrieve the local offloading policy from PCF. The SMF provides the IP address of the I-SMF to PCF. The PCF provides the local offloading policy to SMF by Npcf_SMPolicyControl_Create response.

The local offloading policy indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN, and indicates the supporting local handling of edge computing.

8.	The SMF performs the Nsmf_PDUSession_Create Response to deliver the local offloading policy transparently to I-SMF.

9.	The I-SMF retrieves the EDI from UDR as indicated in step 1-4 in section 6.2.3.4.3 and 6.2.3.4.4, which replaces the SMF to I-SMF. The EDI is consumed by I-SMF to generate DNS handling rules for EASDF.

10.	The same procedure as defined from step 2 to step 19 in section 6.2.3.2.2, which replaces the SMF to I-SMF. I-SMF selects the EASDF and configures the DNS handling rule to EASDF and with the following changes:

Step 16: if the EAS IP address and FQDN in DNS response are included in the IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN, the I-SMF perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

6.x.2.3	EDI management

If supports local offloading to reduce impact on central 5GC NFs, the I-SMF is responsible for managing the EAS Deployment Information instead of SMF.

The procedure of EAS Deployment Information Provision from AF via NEF is reused the procedure in clause 6.2.3.4.2.

The procedure of EAS Deployment Information Management is reused the procedure in clause 6.2.3.4.3 that replace the SMF to I-SMF.

* * * Start of Change 5 * * *

7.1.1	General

The following table illustrates the EASDF Services and Service Operations.

Table 7.1.1-1: NF services provided by the EASDF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Neasdf_DNSContext | Create | Request/Response | SMF/I-SMF |
|  | Update | Request/Response | SMF/I-SMF |
|  | Delete | Request/Response | SMF/I-SMF |
|  | Notify | Subscribe/Notify | SMF/I-SMF |
| Neasdf_BaselineDNSPattern | Create | Request/Response | SMF/I-SMF |
|  | Update | Request/Response | SMF/I-SMF |
|  | Delete | Request/Response | SMF/I-SMF |

* * * Start of Change 5 * * *

7.1.2.1	General

Service description: This service enables the consumer to create, update, or delete DNS context in EASDF and to Subscribe to DNS message related reporting from EASDF.

DNS contexts in EASDF include rules on how EASDF is to handle DNS messages.

This service also can be supported by V-EASDF in VPLMN for HR scenario supporting HR-SBO.

If supports local offloading to reduce impact on central 5GC NFs, I-SMF is the consumer of service. The I-SMF is not supported to consume the service for HR scenario supporting HR-SBO.

* * * End of Change * * *