3GPP TSG-WG SA2 Meeting #164	S2-2407700

Maastricht, Netherlands, August 19 - 23, 2024	(revision of S2-2407196)

Source:	China Mobile, China Unicom, Huawei, HiSilicon, Ericsson, Interdigital, Xiaomi, vivo

Title:	TR 23.700-70 KI#2: Conclusion for Supporting PDU Set Info. Identification for E2E Encrypted XRM Traffic

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract: This pCR proposes to conclude the KI#2.

1	Discussion

This paper provides conclusion of KI#2: Support PDU Set information identification for end-to-end encrypted XRM traffic.

2	Proposal

It is proposed to include the below changes into TR 23.700-70.

First Changes

- 8. Conclusions

8.X	Conclusions for Key Issue #2

The following principles are recommended to be included in the conclusions:

Media over QUIC (MoQ) [9] and Proxy-UDP-in-HTTP/3[38]+QUIC-Aware Proxying[40] should be supported in normative work.

-	For MoQ, the PDU set information is accommodated via Metadata in MoQ. UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata. The detailed information include:

-	AF provides the Protocol Description and the requirement for MoQ in order to support the encryption information identification from the PDUs in PDU Set following the AF session with QoS procedure. The request may include AS address.

-	PCF determines PCC rules based on AF request, to instruct SMF/UPF to identify PDU Set information for MoQ traffic.

-	SMF provides N4 rule to UPF to activate the MoQ relay functionality.

-	UPF supports the MoQ relay functionality and identifies the PDU Set information from the MoQ metadata. For DL, UPF includes the identified PDU Set information into GTP-U header as specified in Rel-18.

NOTE: There is no 3GPP specification impact for UE. The application in UE uses MoQ protocol defined in [9] to obtain a MoQ relay address via DNS procedure and establish MoQ connection with MoQ reply in UPF.

-	For Proxy-UDP-in-HTTP/3 + QUIC-Aware proxying, PDU set information is included in HTTP datagram via a new datagram mode or via packet transforms in QUIC-Aware proxying[40]. UPF supports the HTTP/3 Client functionality. The detailed information include:

-	AF provides the Protocol Description for PDU Set Information in order to support the identification of PDU Sets for encrypted traffic, and the AS address to enable an encapsulation protocol connection between the UPF and AS. This is done with using the AF session with QoS procedure, and meant to supportthe establishment of a UDP tunnel with the connect-UDP upgrade token.

-	PCF generates a PCC rule including the indication of UDP tunnel establishment and AS address.

-  SMF provides N4 rules to the UPF including the indication of UDP tunnel establishment and AS address.

-	UPF supports the HTTP/3 Client functionality, which establishes a UDP tunnel to the AS (where HTTP/3 Proxy operates) with the connect-UDP upgrade token. For DL, UPF identifies the PDU Set Information from the HTTP datagrams, and includes the PDU Set Information into GTP-U header as in Rel-18.

NOTE: There is no 3GPP specification impact for UE. The application in UE is assumed to use QUIC-based (for QUIC-aware proxying) or other UDP-based encrypted media delivery protocol(e.g. RTP cryptex) (for Proxy-UDP-in-HTTP/3). For QUIC-aware proxying, in case the connection ID from UE side is changed, the packets with new connection ID will be forwarded in tunnel model according to Proxy-UDP-in-HTTP/3[38].

End of Changes