3GPP TSG-WG SA2 Meeting #164	S2-2407705

# 19 - 23 August, 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3790 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Store and Forward Satellite operation for EPC on-board |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In conclusion of KI#2, it is agreed to include Store and Forward operation with a full CN onboard for EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding EPC on-board Store and Forward operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Store and Forward Satellite operation of whole EPC on-board is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.X.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.13.X.3	EPC on-board

The on-board EPC including eNB, MME, SGW, PGW, HSS etc for each satellite. Proxies are deployed on the satellite and the ground for protocol conversion and traffic transmission, including support of MT traffic, MO traffic, SMS, etc.

When EPC onboard the satellite, an SSF indication is broadcast in SIB1 along with a PLMN ID to indicating the SSF capability. A UE initiates the attach procedure to the E-UTRAN in SSF mode by indicating its capability to support the SSF operation in the Attach Request message. UEs that do not support SSF mode should not access to the satellite or rejected.

The attach request may be rejected due to the lack of UE subscription and credential in onboard HSS. A timer may be provided in the rejection response. The timer indicates the time the UE should wait before re-attempting. Besides, the onboard HSS can store the IMSI and current TAI for the UE whose attach request is rejected, and the onboard HSS can retrieve the subscription and credentials for the rejected UE in the ground HSS when the feeder link is available, and then the UE can attach again when the service link become available again.

Optionally the MME provides the UE with a SSF monitoring list of satellites IDs, during attach/TAU. The MME determines a SSF monitoring list of satellites IDs, based on the UE location, the subscription and credentials information provided by on-board HSS, and satellite ephemeris.

# * * * * End of changes * * * *