SA WG2 Meeting #164	S2-2407708

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-240xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1130 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of UE data collection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the R19 UPEAS_ph2 conclusion, the following aspects are concluded as principles for normative work: - UPF event exposure service is enhanced to allow NF consumer to obtain one NATed UE public IP address and Port for a particular PDU Session from the SMF and UPF, based on the private UE IP address allocated by 5GC. - The remote end IP address is mandatory input for the above to avoid exposing the full NAT mapping for a UE. For the current UE data collection, it only applies to no NAT between UE and UPF. After introducing the NAT functionality, there is a lack of a process to provide public UE IP address to AF to identify the target UE for data collection. Also, there is a lack of a process in the NWDAF correlates procedure that the NWDAF should indicate the public UE IP address to AF via NEF, to indicate the target UE for data collection. This paper focus on to introduce a procedure to let the AF or NWDAF to obtain the public UE IP address from 5GC, that to identify the target UE and consequently perform data collection from the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the service for UPF to expose the public UE IP address. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to handle the NAT deployment in UE data collection is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.2.4.x (all new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1(all new text) * * *

6.2.8.2.4.x	Procedure of UE data collection supporting NAT deployment

The procdure in this section applies to the case of NAT deployment between UE and UPF.  And this procedure covers the trusted AF, untrusted AF and the NWDAF correlation UE data collection.

Figure 6.2.8.2.4.x-1: UE data collection procedure supporting NAT deployment

1.	The same procedure as Step 1-2 in cluases 6.2.8.2.4.2 or 6.2.8.2.4.4, or step 1-3 in cluase 6.2.8.2.4.3.

2.	The same procedure as Step 1-4 in clause 4.15.4.5 to get the public UE IP address and port number. The consumers of SMF service to get the public UE IP address/port number are trusted AF, NEF and NWDAF respectively.

3a.	The same procedure as Step 5 in clause 6.2.8.2.4.2 for trusted AF correlation.

3b.	The same procedure as Step 6-7 in clause 6.2.8.2.4.3 for untrusted AF correlation.

3c.	The same procedure as Step 5 in clause 6.2.8.2.4.4-1 for NWDAF correlation.

* * * End of Change * * *