3GPP TSG-WG SA2 Meeting #164	S2-240xxxx

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-240xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | DRAFT | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription of UPF event via I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19_SUEEI |  |  |  |  |  | Date: |  |  | 2024-07-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To get exposure data from UPF, NF consumer may subscribe to the UPF indirectly via SMF. But, if the UPF is inserted by I-SMF that SMF cannot serve the target DNAI for the traffic routing for local access to the DN, the SMF can’t directly subscribe the event in UPF. The SMF should send the subscription to UPF via I-SMF. For this case, the SMF should sent the subscription to I-SMF, and the I-SMF represents the SMF to subscribe to UPF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If I-SMF is used for this PDU session and the UPFs controlled by the I-SMF, the SMF sends the the UPF event subscription related information to I-SMF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to SMF subscribe the event to the UPF/DNAI that SMF not control is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.34.x (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 (all new text) * * *

5.34.X	Support for UPF event subscription

This clause defines how to supports the UPF event subscription.

The consumer NF (e.g.: NWDAF) of UPF event may subscribe the UPF event in target DNAI. The UPF in the target DNAI may be inserted by I-SMF that SMF cannot serve the target DNAI for the traffic routing for local access to the DN. So, when the SMF receives the subscription request from NF consumer, the SMF can’t directly subscribe the event in UPF in target DNAI.

For certain UE subscription, if the SMF receives the subscription request from NF consumer including the target DNAI that not served by the SMF, the SMF is determines the I-SMF and impacted PDU session according to the target DNAI. And the SMF sends the subscription request to I-SMF on behalf of the NF consumer. And the I-SMF sends the subscription request to UPF on behalf of SMF.

For any UE subscription, according to the subscribed UPF event, the NF consumers discover the SMFs/UPFs respectively. If the subscribed UPF events needs the SMF(s) to do a third-party subscription onto UPF, the same procedure similar as single UE subscription. And if the several SMFs determines the target DNAI is not served, the several SMFs sends the subscription request to I-SMFs on behalf of the NF consumer. And the I-SMFs send the subscription request to UPFs on behalf of SMFs.

* * * End of Change * * *