SA WG2 Meeting #164	S2-2407729

Maastricht, Netherlands, 19 – 23 August, 2024

Title:	[Draft] Reply LS on RAN2 agreements and assumptions for Ambient IoT

Response to:	S2-2407447/R2-2406150

Release:	Rel-19

Work Item:	FS_AmbientIoT

Source:	NEC [to be SA2]

To:	RAN2

Cc:	RAN3, SA3

Contact Person:

Name:	Chunhui Zhu

E-mail Address:	zhu_chunhui@nec.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks RAN2 for the LS on RAN2 agreements and assumptions for Ambient IoT.

SA2 has discussed the SA2 related aspects and would like to provide the following feedback:

RAN2 agreements and assumptions: RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2.

SA2 feedback: SA2 has not reached agreements on the general QoS requirements for AIoT service, which may be provided later.

RAN2 agreements and assumptions: As baseline, the “inventory and command” case is supported by the procedure:

- Step A: A-IoT paging;

- Step B: Device ID transmission (via Random Access or without using RA).  Details are FFS

- Step C: reader to device data transmission (e.g. the R2D command), and

- Step D: corresponding device to reader data transmission (e.g. the feedback).  FFS whether this is optional, pending other WG discussions.

SA2 feedback: In Step D, the data transmission (e.g. the feedback) from device to reader is required. If due to some reason e.g. the device energy is exhausted, the device does not send the data, or the reader does not receive the data, in these cases the reader should consider Step C fails.

RAN2 agreements and assumptions: From RAN2 point of view we will study “Command only” use case.

FFS the options on how to support it:

•	A-IoT paging message from the reader contains the command.  Final feasibility depends on SA2 and SA3 work/conclusions.

•	Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

SA2 feedback: A-IoT paging message from the reader does not include the command. The Command is sent after the successful paging. SA2 prefers ‘Use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command)’.

RAN2 agreements and assumptions: RAN2 will study the following cases for A-IoT paging message:

•	a message containing multiple IDs of A-IoT devices.  Need to confirm the need for this use case based on SA2 discussion.

What device ID and group ID and scenarios is depending on SA2 discussion.

SA2 feedback: the case to send message(s) to multiple AIoT devices simultaneously exists. It is up to RAN2 to decide “a message containing multiple IDs of A-IoT devices” or multiple messages with each message containing one AIoT device ID. SA2 defines device ID, group ID and scenarios in TR23.700-13.

2. Actions:

To RAN2 group

ACTION: SA2 kindly asks RAN2 to take the above feedback into account for the future work.

3. Date of Next SA2 Meetings:

SA2#165	14th – 18th October 2024 		Hyderabad, IN

SA2#166	18th – 22nd November 2024 		Orlando, USA