SA WG2 Meeting #164	S2-2407730

# Maastricht, Netherlands, 19 – 23 August, 2024                 (revision of S2-240xxxx)

Source: 	NEC

Title: 	New solution: Simple AIoT device and AIoT operation in the certain deployment

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes a new solution for Simple AIoT device and AIoT operation in the certain deployment.

1	Discussion

By relying on energy harvested from ambient sources, AIoT aims to develop low-cost, small, and maintenance-free devices.

RAN1 defines the AIoT device as following:

- The overall objective of the SI is to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices:

i. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

ii. ≤ a few hundred µW peak power consumption has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

Since the AIoT device has only very limited energy storage and capabilities with computing, storage and communication, it should be possible for AIoT device to have simpler functions and less interactions with the network than NB-IoT or other existing 3GPP defined UE/devices.

One typical AIoT use case is inventory in the warehouse, where the environment is closed and controllable by the warehouse owner (as the 3rd party) and the AIoT network operator.

In this paper, the simple AIoT device and AIoT operation are proposed.

2	Proposal

It is proposed to update TR 23.700-13 as following.

# * * * * First change* * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #x | X | X | X |

# * * * * Next change（ALL NEW TEXT）* * * *

## 6.X	Solution #X: Simple AIoT device and AIoT solution in the certain deployment

### 6.X.1	Description

One typical AIoT use case is inventory in the warehouse, where the environment is closed and controllable by the warehouse owner (as the 3rd party/AF) and the AIoT network operator.

For the AIoT device with very limited capabilities and power can access to the network, the AIoT device operation including security and privacy can be compromised if it is agreed by the AIoT network operator and the 3rd party.

In addition to the architectures in Figure 6.3.1.4-1 and Figure 6.3.1.4-2, the architecture is enhanced with a core network function AIoT device Management, which can manage the device including the indication that this AIoT device can be operated without security and privacy requirements on the device and the related Reader(s).

### 6.X.2	Procedures

In this solution, the AIoT device does not need to have security or privacy capabilities internally. From the AIoT network and 3rd party’s perspectives, the device only needs to store its device ID, and responds to a reader with its device ID when the reader requests for inventory. Based on the agreement between AF and AIoT device management, AIoT controller authorizes the simple AIoT operation without security and privacy.

AIoT device

AIoT Reader

AIoT controller

7. Inventory Response (Device ID, Reader ID)

1. Inventory Request (Reader ID, Device ID, No security and privacy operation allowed indication)

AIoT device Management

2. Device information request (Device ID)

3. Device information response (Device ID, No security and privacy operation allowed indication, No security and privacy operation Allowed Reader ID)

5. Inventory Request (Device ID)

6. Inventory Response (Device ID)

AF

8. Inventory Response (Device ID, Reader ID)

4. Inventory Request（Device ID, No security and privacy operation allowed indication)

Figure 6.X.2 Inventory procedure

- AF sends Inventory request to AIoT controller, which includes Reader ID, Device ID and No security and privacy operation allowed indication. The Reader ID indicates the reader requested for Inventory. Multiple Reader IDs can be included. The Device ID indicates the AIoT device for Inventory. Multiple Device IDs can be included.

- AIoT controller sends Device information request to AIoT device management, which includes Device ID received in Step 1. If multiple Device IDs are received in Step 1, AIoT controller can repeat this step per Device ID for multiple times, or AIoT controller can send the Device ID list to AIoT device management in the same message.

- AIoT device management sends Device information response to AIoT controller, which includes the Device ID, No security and privacy operation allowed indication, No security and privacy operation allowed Reader ID(s). If Multiple Device IDs are received in Step 2, this message may include all device information corresponding to the Device IDs.

- Based on the received information in Step 3 and the received request in Step 1, the AIoT controller authorizes the No security and privacy operation for the AIoT device under the Reader(s). The reader ID(s) shall be the common parts both from the Step 1 and Step 3, the differences between them shall be excluded. The AIoT controller sends the inventory Request to the reader(s), which includes Device ID, No security and privacy operation allowed indication.

- The Reader(s) send Inventory Request to AIoT device, which includes the Device ID.

- If the received Device ID is matched with the Device ID stored in the AIoT device, the AIoT device sends Inventory Response to the Reader with its Device ID.

- The Reader sends Inventory Response to the AIoT controller with the Device ID and Reader ID.

- AIoT controller stores the Device ID and the Reader ID, if multiple Inventory Responses are received from different Readers, the AIoT controller stores the Device ID and multiple Reader IDs. AIoT controller sends the Inventory Response to AF with the Device ID and Reader ID(s).

### 6.X.3	Impacts on services, entities and interfaces

New network entities and interfaces are proposed.

# * * * * End of Changes * * * *