SA WG2 Meeting #164	S2-2407731

# Maastricht, Netherlands, 19 – 23 August, 2024                 (revision of S2-240xxxx)

Source: 	NEC

Title: 	New solution: Architecture enhancements for Intermediate node authorization in Topology 2

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes a new solution for Architecture enhancements for Intermediate node authorization in Topology 2.

1	Discussion

For AIoT Topology 2: BS <--> intermediate node <--> Ambient IoT Device, only a UE can act as an intermediate node which is under the network control.

In this solution, the architecture enhancement for the authorization to UE reader in Topology 2 is proposed.

2	Proposal

It is proposed to update TR 23.700-13 as following.

# * * * * First change* * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #x | X |  |  |

# * * * * Next change（ALL NEW TEXT）* * * *

## 6.X	Solution #X: Architecture enhancements for Intermediate node authorization in Topology 2

### 6.X.1	Description

In Topology 2, the UE acting as the intermediate node is responsible for transferring the information between AIoT device and 5GS. It is necessary to study how to authorize the UE for acting as the intermediate node.

In addition to the architectures in other solutions, the architecture is enhanced with a core network function AIoT Reader Management, which can authorize the UE reader as the intermediate node of AIoT service.

### 6.X.2	Procedures

UE

AMF/AIoTF/AIoT controller

4. AIoT Intermediate node authorization response

1. Authorization Request as the AIoT Intermediate node

AIoT Reader Management

2. AIoT Intermediate Node Authorization request

3. AIoT Intermediate Node Authorization response

Figure 6.X.2 Intermediate node authorization procedure

- UE sends Authorization Request as the AIoT Intermediate node to AMF/AIoTF/AIoT controller, which includes UE ID. This request may be included/piggybacked in the Registration Request from UE to AMF, or other interactions between UE and AMF/AIoTF/AIoT controller when the UE wants to be a AIoT Intermediate node.

- AMF/AIoTF/AIoT controller send AIoT Intermediate Node Authorization request to AIoT Reader Management, where the UE ID is included.

- Based on the configuration and policy from the operator, the AIoT Reader Management authorizes the UE as AIoT Intermediate Node. AIoT Reader Management sends AIoT Intermediate Node Authorization response to the AMF/AIoTF/AIoT controller, where the authorization result is included.

- AMF/AIoTF/AIoT controller sends AIoT Intermediate node authorization result to the UE. If the result is authorizing the UE to be AIoT Intermediate node, the UE can perform the AIoT Intermediate node operation.

### 6.X.3	Impacts on services, entities and interfaces

New network entities and interfaces are proposed.

# * * * * End of Changes * * * *