3GPP SA WG2 Meeting #164	S2-2407743

Maastricht, NL, August 19 - 23, 2024														(was S2-240xxxx)

Source:	Ericsson

Title:	KI #1, #2, #3, Sol#30: Update to resolve ENs

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution resolves some ENs in solution #30.

1. Introduction

Solution#30 “Solution #30: Support AIoT Devices using AIOTF for Topology 2” was included in the TR 23.700-13 in SA2#163. In this solution, the system architecture and protocol stacks are described. The procedures include the AIoT Service Authorization for Intermediate UE, inventory and command. There are some ENs left in the solution. This contribution proposes some updates to resolve those ENs.

There is one EN about Intermediate UE registration:

Editor's note:	It is FFS whether to add separate procedure from the UE for establishing the NAS connection for the AIOT UE NAS layer, instead of establishing the connection at registration procedure when the Ambient IoT Capable UEs have been authorized Intermediate UE.

In solution#30, there is a AIOT UE NAS layer proposed. Within this layer, it is possible to introduce registration/deregistration as separate NAS procedures between the UE and the AIOTF.

[Proposal-1] Include the NAS layer option and resolve the EN.

Another EN is about the collection of posture information of Intermediate UEs:

Editor's note:	It is FFS whether RAN has enough information to determine suitable UEs.

The NG-RAN understands the RRC state of the Intermeidate UEs. The intermediate UEs are kept in CM-CONNECTED. However, they may be in RRC_INACTIVE state. And the NG-RAN needs to perform RAN paging to bring those UEs back to RRC_CONNECTED state, and then the UEs can be utilized as intermediate UEs for the AIoT service operation. Otherwise, the DL NAS from the AIOTF cannot be delivered.

For whether NG-RAN can have further information to determine suitable intermediate UEs, it is up to RAN WGs to investigate.

[Proposal-2] Clarify as above to resolve the EN.

Two ENs about the device type:

Editor's note:	It is FFS and up to RAN to determine whether the device type is useful or not e.g. to optimize AS functionality such as spectrum usage, number of repetitions, etc.

Editor's note:	It is FFS whether the external device type can be kept same as the internal device type.

It is up to RAN WGs to determine whether the device type is helpful. Regarding whether the external device type is different from the internal device type, it can be further investigated in the normative phase.

[Proposal-3] Clarify as above to resolve the EN.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v0.4.0:

* * * Start of Change * * * *

## 6.30	Solution #30: Support AIoT Devices using AIOTF for Topology 2

### 6.30.1	Description

#### 6.30.1.0	Introduction

This solution proposes a complementary solution to Solution #8. It enables the support of Ambient IoT devices for topology 2, which addresses KI#1, KI#2 and KI#3. Together with Solution #8 which focus on Topology 1, the solutions can support AIoT devices for both Topologies.

In this solution, it is AIOTF who is in charge of the intermediate UE for the Ambient IoT operations, including determining intermediate UEs, sending operation commands to and receiving results from intermediate Ues. As the licensed spectrum is owned by MNO, it is proposed to let network provide the radio resource information towards the intermediate Ues about the spectrum information for the over-the-air interface between Intermediate Ues and AIoT devices.

#### 6.30.1.1	Reference Architecture

The Figure 6.30.1.1-1 illustrates the architecture for supporting AIoT Devices using AIOTF for Topology 2:

Figure 6.30.1.1-1: System Architecture of Supporting AIoT Devices using AIOTF for Topology 2

This solution focuses on Topology 2, which works together with Solution #8 addressing Topology 1.

The functional entities defined in Solution #8 and TS 23.501 [4] are reused with the exception for the following additions:

-	UDM/UDR: The authorization information of Intermediate UE for AIoT is stored in UE subscription data.

-	AMF: Receive AIoT capability information from UE and authorize based on the subscription data in UDM/UDR.

-	NG-RAN: Determine intermediate UEs and provide spectrum information towards authorized intermediate UE.

-	Intermediate UE:

-	Provide Ambient IoT capability information to AIOTF and receive the authorization information.

-	Receive the instruction from AIOTF and perform Ambient IoT operations (e.g. inventory, command, etc.) on the proper spectrum. The radio resource information is received from NG-RAN.

-	Register/Deregister towards AIOTF (optional)

-	Ambient IoT Function (AIOTF): AIOTF is introduced to support AIoT services, with some AMF’s functionalities integrated, which includes:

-	Inventory handling and device context management.

-	Command delivery.

-	Authentication and authorization for the access, which triggers interaction with AUSF/UDM.

-	Collect charging data and interact with CHF for charging.

-	Routing the request from AF (via NEF) to Intermediate Ues, for DO-DTT/DT traffic types.

-	Routing the response from Intermediate Ues to AF (via NEF) for DO-DTT traffic type.

-	Receive registration/deregistration from UE.

#### 6.30.1.2	Protocol Stack

The Figure 6.30.1.2-1 illustrates the protocol stack:

Figure 6.30.1.2-1: Protocol Stack for Supporting AIoT Devices using AIOTF for Topology 2

Within the protocol stack, the AIOT Device NAS layer, AIOT AS layer and App layer are defined as Solution #8.

As UE reader is used in Topology 2, which is different from the RAN reader in Topology 1, the following layer is proposed specific in this solution:

-	AIOT UE NAS layer: The NAS protocol between AIOTF and UE readers. Besides the 5GMM NAS and 5GSM NAS, it can be defined as a new NAS (e.g. 5GAIOT NAS) which is piggybacked in 5GMM NAS transport messages. The AIOT UE NAS layer is used to communicate the AIOT specific information to the UE reader e.g. inventory and command.

### 6.30.2	Procedures

NOTE:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

#### 6.30.2.1	AIoT Service Authorization for Intermediate UE

The Registration procedure for UE is performed as defined in clause 4.2.2.2 of TS 23.502 [5] with the following additions:

-	UE includes the AIoT Intermediate node capability as part of "5GMM capability" in Registration Request message.

-	The AMF obtains the AIoT Subscription data as part of the user subscription data from UDM using Nudm_SDM service

-	The AMF determines whether the UE is authorized to work as Intermediate UE for AIoT based on UE’s AIoT Intermediate node capability and the AIoT Subscription data. The AMF includes the authorization information as part of UE context in NGAP message sent to NG-RAN.

In Service Request procedure, N2 Handover procedure, Xn Handover procedure, and when receiving Subscriber Data Update to AMF, the AMF includes the authorization information in NGAP message sent to NG-RAN.

After NG-RAN receives the authorization information in the NGAP message from the AMF that the UE is authorized as an Intermediate UE, it does not release the UE to RRC_IDLE state afterwards. The UE can be released to RRC_INACTIVE when needed, in which case the NG-RAN assigns a suitable RNA and the NG-RAN need to perform RAN paging before the UE can act as a Intermediate UE.

#### 6.30.2.2	AIOTF Keep Track of Intermediate UE

For authorized Intermediate Ues (Those Ambient IoT Capable Ues which have been authorized by the AMF as described in clause 6.30.2.1), in Initial Registration procedure as defined in clause 4.2.2.2 of TS 23.502 [5] with the following additions:

-	The AMF sends Create UE Context Request to the AIOTF for the Intermediate UE to establish the usage of the AIOT UE NAS layer, so that the AIOTF may select the Intermediate UE for AIoT operations.

In Deregistration procedure as defined in clause 4.2.2.3 of TS 23.502 [5], the following additions are performed:

-	The AMF sends Release UE Context Request to the AIOTF for the Intermediate UE, so that the Intermediate UE will no longer be used.

An alternative option is to establish NAS connection between the UE and the AIOTF in AIOT UE NAS layer:

-	After register towards the network, the UE sends Registration Request towards the AIOTF.

-	Before de-register from the network, the UE send Deregistration Request towards the AIOTF.

The Registration/Deregistration request is 5GAIOT NAS request, which is transferred from the UE to the AIOTF via the AMF.

The AIOTF can subscribe the UE mobility event of the Intermediate UE towards the AMF to be updated for the location information.

#### 6.30.2.3	Application Inventory Procedure

The Inventory procedure can be initiated by the AF to discover one or more AIoT devices in a specific area via Intermediate Ues.

Figure 6.30.2.3-1: Inventory Procedure

1.	AF sends Inventory Request to the AIOTF, which is the same as step 1 - 4 in Figure 6.8.2.1-1 in Solution #8.

2a.	The AIOTF generates the candidate UE list for the Intermediate UEs to be used, based on the location information for the Intermediate UEs, as well as area information provided by the AF.

2b.	The AIOTF sends Determine UE Request to the AMF with the candidate UE list and the area information.

2c.	The AMF sends N2 message: Determine UE Request to the NG-RAN with the candidate UE list and the area information. The AMF may send to multiple NG-RANs within the area.

2d.	The NG-RAN selects the reachable UEs based on the candidate UE list and the area info.

NOTE 1:	It’s up to RAN WGs to determine whether the NG-RAN can determine suitable UEs to ensure the coverage with less interference.

2e.	The NG-RAN sends N2 Message: Determine UE response to the AMF with the determined UE list.

2f.	The AMF aggregates the responses and send Determine UE response to the AIOTF with the determined UE list.

3.	The AIOTF creates and sends the 5GAIOT NAS message to request each of the Intermediate UE(s) in the determined UE list to perform the Inventory. The 5GAIOT NAS message is delivered via the AMF and the NG-RAN.

4.	The Intermediate UE determines radio resource or requests NG-RAN to determine radio resource, which is used between AIoT Devices and Intermediate UE.

5.	The Intermediate UE initiates inventory based on device information as well as the inventory strategy information provided by the AF. The Intermediate UE may provide reader identity information to enable the AIoT devices to understand they are read by which Intermediate UE. Considering the mobility of the Intermediate UE, the reader identity information can be a combination of a UE ID and the location information.

6.	The AIoT Device reports the device ID to the Intermediate UE. If the Inventory procedure indicates delta inventory only, and the AIoT Device has performed the inventory procedure towards this reader, it should skip the reporting. The device capability index can be provided by AIoT device optionally.

7.	The Intermediate UE creates the 5GAIOT NAS message and sends it towards the AIOTF, including the AIoT Device ID and optional device capability index. The Intermediate UE may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy. The Intermediate UE may further provide an end indicator to inform the AIOTF whether it is the last inventory response for the inventory round, e.g. based on timeout in Intermediate UE.

NOTE 2:	It's up to RAN WG3 to determine the details and enhancements of the location information, and the ability to provide an end indicator from a reader is assumed to be aligned between UE as reader and RAN as reader.

8.	The AIOTF validates the concealed device ID via interacting with AUSF and UDM. The AIOTF may further get the device capability information from the device capability profile stored in the UDM based on the device capability index provided by the device.

9.	The AIOTF together with AUSF and UDM, triggers authentication and authorization procedures (i.e. like the authentication request/response between UE and network) towards the AIoT device. In Topology 2, the message between the AIOTF and the AIOT device is transmitted via the Intermediate UE over 5GAIOT NAS.

Based on the device capability information from UDM, if the AIoT device is capable of storing received parameters for a longer period, step 10 - step 11 are executed i.e. the AIOT device is then considered registered:

10.	The security mode negotiation and security parameter exchanges are performed (i.e. like the security mode command/complete between UE and network). In Topology 2, the message between the AIOTF and the AIOT device is transmitted via the Intermediate UE over 5GAIOT NAS.

NOTE 3:	Further details of the security procedures is to be defined by SA WG3.

11.	The AIOTF may further allocates CN device ID and sends to the AIoT device.

12.	The AIOTF registers with the UDM (UECM) for the device access.

13.	The AIOTF may further aggregate the reported device IDs and send to AF via the NEF, as step 14 - 16 in Figure 6.8.2.1-1.

#### 6.30.2.4	Periodic Inventory

The periodic inventory may be requested by the AF, so that network can trigger the inventory periodically without the request from the AF directly.

Considering the movement of the Intermediate UE, and the inventory is based on area information, the periodic inventory should be initiated by the CN (i.e. the AIOTF), which follows the instructions from the AF. When the period expires in the AIOTF, the AIOTF determines the Intermediate UEs based on the area information from the AF and the location information for the Intermediate UEs. Step 2-10 in Figure 6.30.2.3-1 are executed.

#### 6.30.2.5	Command Procedure

The command procedure is initiated by the AF to request one or more specific AIoT devices or a group of AIoT devices in an area to execute the command. The command can be read, write, enable, disable, or execution request from the AF. The command result from the devices may or may not be required.

NOTE 1:	The command for enable/disable is to be studied by SA WG3, and to be aligned with SA WG3.

Figure 6.30.2.5-1: Command Procedure

1.	The AF sends Command Request to the NEF, containing the command to be executed, area information, device information, optional inventory strategy information, and optional report aggregation info.

-	The command is to be executed in the AIoT devices. It contains the command (e.g. read, write, execute, etc.) together with the command parameters. The command specific parameters (e.g. what to be executed) are to be included in an app layer container between AF and AIOT devices.

-	The area information could be the external geographical area information.

-	The device information could be device ID, device group ID, and/or external device type.

-	The location required indicates whether the AF requests the location information of the AIoT devices provided.

-	The report aggregation info indicates whether the reports need to be aggregated or not for a specific aggregation period, and whether the reports are needed after the aggregation period.

2.	The NEF authorizes the request from the AF and perform the area translation to translate external area information to the internal area information. Within the authorization, the NEF further check whether the AF is authorized to get the location information of the device. The NEF map the external device type to internal device type (i.e. device 1, device 2a, device 2b, etc.).

NOTE 2:	It’s up to RAN WGs to determine whether the device type is useful or not.

Depending on whether area information is provided, step 3a or step 3b is performed:

3a.	If area information is provided, the NEF sends NRF query with internal area information to query AIOTFs serving the area.

3b.	If area information is not provided, and device ID(s) are provided, the NEF query the serving AIOTF from UDM.

4.	The NEF sends the Command Request to the AIOTFs with the command, internal area information, device information, device type, and optional location required information.

5.	The AIOTF allocates a Transaction ID for the command, which is unique per command. The AIOTF creates the AIOT Device NAS request message for the devices, based on the command information from the AF. The AIOTF get the registered devices, which match the device information. If no registered devices, step 6 - 13 are skipped.

6.	The AIOTF creates the 5GAIOT NAS request message and sends to the Intermediate UE to deliver the Command. The 5GAIOT NAS request message contains the AIOT Device NAS message, CN allocated device IDs for the registered devices, the Transaction ID, and optional location required information. The 5GAIOT NAS message is delivered via the AMF and the NG-RAN.

7.	The Intermediate UE determines radio resource or requests NG-RAN to determine radio resource, which is used between AIoT Devices and Intermediate UE.

8.	The Intermediate UE initiates command delivery for the AIOT Device NAS request message based on CN allocated device IDs, using dedicated signalling. Transaction ID is also included.

NOTE 3:	RAN signalling details, including how the command is sent, are to be further studied by RAN WGs.

9.	The AIoT Device executes the command, creates an AIOT Device NAS response message containing the command result and delivers the AIOT Device NAS response message, if the result needs to be sent back. The application specific result can be included in an app layer container.

10.	The Intermediate UE creates the 5GAIOT NAS response message, including the AIOT Device NAS response message, and sends to the AIOTF. The Intermediate UE may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy. The 5GAIOT NAS message is delivered via the NG-RAN and the AMF.

NOTE 4:	It's up to RAN WG3 to determine the details and enhancements of the location information.

11.	The AIOTF gets the command result from the AIOT Device NAS response message. The AIOTF may perform aggregation for the device ID, based on the report aggregation information provided by the AF. Within the aggregation period, the AIOTF will buffer the command results reported from the AIoT devices. The AIOTF may stop buffering and send report immediately, if it receives end indicator from the Intermediate UE in step 10. When the aggregation period expires, the AIOTF sends the report. For those command result after the aggregation period, if it is needed by the AF, the AIOTF sends the report. Otherwise, it will be dropped.

12.	The AIOTF sends Command Response or Notification Request towards the NEF for the command result or the aggregated command result information.

13.	The NEF sends Command Response or Notification Request towards the AF for the command result or the aggregated command result information.

If the AIOTF determines all required devices have executed the command (e.g. when AF provide the device ID(s) and those devices have executed the command in step 7 - 8), the following steps are skipped.

14.	For those unregistered devices(s), the AIOTF initiate Inventory procedure as step 3 - 12 in Figure 6.30.2.3-1 (step 11 - 12 will be skipped, if the device is not able to persistent received information). In the inventory the Transaction ID is delivered, so that the registered devices which have executed command do not respond this inventory request from the Intermediate UE.

15.	The AIOTF creates the 5GAIOT NAS request message and sends to the Intermediate UE to deliver the Command to the responded device. The 5GAIOT NAS request message contains the AIOT Device NAS request message, AIoT device ID and optional location required information. The 5GAIOT NAS request message is delivered via the AMF and the NG-RAN.

16.	The Intermediate UE determines radio resource or requests NG-RAN to determine radio resource, which is used between AIoT Devices and Intermediate UE.

17.	The Intermediate UE initiates command delivery for the AIOT Device NAS request message to the device.

18.	The AIoT Device executes the command, creates an AIOT Device NAS response message containing the command result and delivers the AIOT Device NAS response message, if the result needs to be sent back. The application specific result can be included in an app layer container.

19.	The Intermediate UE creates the 5GAIOT NAS response message, including the AIoT Device NAS response message, and sends to the AIOTF. The Intermediate UE may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy. The 5GAIOT NAS message is delivered via the NG-RAN and the AMF.

20.	As step 11 - 13, the AIOTF get the command result from the AIOT Device NAS response message. It may perform the aggregation and send to NEF. The NEF delivers to the AF.

### 6.30.3	Impacts on services, entities and interfaces

The following NFs are impacted on top of the impacts of solution #8:

-	NG-RAN:

-	Determine Intermediate UEs.

-	Determine radio resources between Intermediate UE and AIoT devices.

-	Keep Intermediate UEs in RRC_CONNECTED or RRC_INACTIVE states

-	Ambient IoT Function (AIOTF):

-	AIOT NAS Registration/Deregistration from UE (optional)

-	Routing the request from AF (via NEF) to Intermediate UE, for DO-DTT/DT traffic types.

-	Routing the response from the Intermediate UE to AF (via NEF) for DO-DTT traffic type.

-	AMF: Authorization of Intermediate UEs and create UE context towards AIOTF (optional).

-	UDM: Enhance authorization information for Intermediate UEs.

-	Intermediate UE:

-	Authorization of Intermediate UEs

-	Request radio resource information from NG-RAN

-	Receive the request from AIOTF and perform Ambient IoT operations (e.g. inventory, command, etc.) on the proper spectrum.

-	Receive responses from AIoT devices and send to AIOTF.

-	AIOT NAS Registration/Deregistration towards AIOTF (optional)

* * * End of Change * * * *