3GPP TSG-SA2 Meeting #164	S2-2407754

# 19 - 23 August, 2024, Maastricht, Netherlands									revision of S2-xxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1132 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of QoS and policy assistance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, vivo, OPPO, SK Telecom, China Mobile, Tencent, Tencent Cloud, InterDigital Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As it has been agreed in clause 8.3 of TR 23.700-84: - The PCF may send a analytics request or subscription to NWDAF for QoS and policy assistance information analytics. The PCF may include an analytics ID, Filter Information, one or multiple sets of QoS parameters, the candidate value list(s) for each QoS parameter, requesting expected service experience (e.g. QoE), etc. - Based on the PCF request, the QoS and policy assistance information analytics provided by NWDAF may include: expected QoE can be achieved by the candidate QoS parameters (the values of the QoS parameters are within the candidate value list provided by the PCF). In order to support the above conclusions for FS_AIML_CN KI#3, a new NWDAF-based analytics is introduced to provide QoS and Policy Assistance Analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new NWDAF-based analytics to provide QoS and Policy Assistance Analytics to consumer to support QoS and policy enhancement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusions for FS_AIML_CN KI#3 cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new), 6.x.4 (new), 7.1, 7.2.2, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes (new text) * * *

## 6.x	QoS and Policy Assistance Analytics

### 6.x.1	General

Clause 6.x describes how NWDAF can provide the QoE of candidate QoS based on the consumer request.

QoS and policy assistance analytics can be used to assist consumer (i.e. PCF) with deciding policies, i.e. PCC rules and QoS parameters. The PCF may take the QoE of candidate QoS into account for QoS determination and modification for a service flow.

The consumer may provide the list of target QoS parameter set(s) and corresponding PCC rules of a service flow to the NWDAF to request the NWDAF to work out the QoE(s). Based on the target QoS parameter set(s) provided by the consumer, the NWDAF generates the QoE(s) of the corresponding QoS parameter set(s) in the list and notify the consumer.

In addition, the service consumer may also provide the allowed values of individual QoS parameters for the different QoS parameter set(s) in the list, and the target service experience (i.e. QoE) of the service flow. Using the list of target QoS and the allowed values of individual QoS parameters, the NWDAF generates the candidate QoS parameter set(s) that can fulfil the QoE indicated by the service consumer. The NWDAF informs the consumer of the candidate QoS parameter set(s) that can fulfil the QoE, the PCC rules and the associated QoE(s).

In order to derive the QoS and policy assistance analytics, the NWDAF collects data from AF, OAM, 5GC NFs (e.g. SMF, PCF, SMF, AMF, LCS, etc.). The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. a Request-Response model).

The service consumer may be an NF (e.g. PCF or AF).

The consumer of these analytics indicates in the request or subscription:

-	Analytics ID = " QoS and Policy Assistance".

-	Target of Analytics Reporting as defined in clause 6.1.3.

-	Analytics Filter Information optionally includes:

-	DNN;

-	Application ID;

-	Area of Interest (AOI(s)): restricts the scope of the QoS service experience analytics to the provided area;

-	A list of analytics subsets that are requested (see clause 6.x.3), e.g. QoS requirements (e.g. 5QI, QoE).

-	A list of target QoS parameter set(s), the associated PCC rules, and optional the candidate values of one or more individual QoS parameters in each QoS parameter set(s).

-	The target QoE of the service.

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred level of accuracy of the analytics.

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.x.3).

-	Optionally, preferred order of results for the list of QoE associated to QoS:

-	ordering criterion: ”QoE” (i.e. QoE that is associated to QoS),

-	order: ascending or descending.

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets (see clause 6.x.3).

### 6.x.2	Input Data

The NWDAF supporting analytics on candidate QoS shall be able to collect information from AF, OAM and 5GC NFs.

The input data collect from AF defined for Observed Service Experience related network data analytics in Table 6.4.2-1 and Table 6.4.2-1a is reused. In addition to the data in Table 6.4.2-1 and Table 6.4.2-1a, other information collected by the NWDAF from the AF is defined in the Table 6.x.2-1.

The information collected by the NWDAF from relevant 5GC NFs (i.e. UPF, SMF, AMF, PCF) is defined in Table 6.x.2-2.

The input data collected from OAM for E2E data volume transfer time analytics in Table 6.18.2-1 is reused.

Table 6.x.2-1: Input data from AF related to QoS and policy assistance inforamtion

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifier of the application at the AF. |
| UE ID | AF | The list of UE IDs that are associated to the service flows/ candidate QoS with the Service Experience value(s). When the AF is untrusted, GPSIs will be provided. When the AF is trusted SUPIs will be provided. |
| QoS reference | AF | The (pre-defined) QoS information of the service flow associated to the application ID to present the service requirement of the AF. |
| QoS parameters | AF | Individual QoS parameters of the flow associated to the application ID to present the service requirement of the AF. |
| QoS duration | AF | The active duration of the QoS. |
| Alternative QoS | AF | Indicate a list of preference of the QoS parameters sets in prioritized order with which the service can operate. |
| Expect service experience | AF | The expecting or required service experience (e.g. MOS, QoE) of the service flow associated to the application ID. |
| Averaging Window | AF | The value of the averaging window for deriving individual QoS parameters for GBR QoS Flows. |
| Time stamp | AF | The time stamp associated to the collected data. |

Table 6.x.2-2: Input data from 5GC NF related to QoS and policy assistance information

| Information | Source | Description |
| --- | --- | --- |
| PCC rule | PCF | The content of PCC rule/ QoS policy, as defined in Table 6.3.1 of TS 23.503 [4]. |
| PCC rule identifier | PCF | The identifier of the PCC rule. |
| QoS parameter set(s) | PCF, SMF | The QoS parameter set(s) including individual QoS parameters and QoS characteristics as defined in TS 23.501 [2]. |
| Alternative QoS | PCF | The alternative QoS of a service flow. |
| QoS profile | SMF | The QoS profile associated to the PCC rule and QoS parameter set(s). |
| 5QI | PCF, SMF | The 5QI as reference of the QoS parameters and QoS characteristics, e.g. standardised and non-standardised 5QI. |
| Active duration of QoS | SMF | The active/ applicable duration of the QoS policy, including the potential active duration of QoS and alternative QoS. |
| PDU session ID | SMF | PDU session ID |
| PDU Session Type | SMF | The type of the PDU session, i.e. Ethernet PDU Session Type and IP PDU Session Type. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| QFI | SMF | The QFI associated to the PCC rule/ QoS parameter set/ profile. |
| Application ID | SMF | The identifier of the applications with traffic flow associated to the corresponding QoS flow. |
| Congestion information | UPF | The UL and/or DL QoS Flow congestion information. |
| Jitter information | UPF | The N6 and/or N3 jitter information for UL/DL collected from UPF. |
| Data rate/flow bit rate | UPF | UL and/or DL data rate per QoS flow/UE at the PSA UPF and it can be applied to a Non-GBR or GBR QoS flow. |
| Traffic delay | UPF | QoS flow Packet Delay for UL/DL/Round trip collected from UPF. |
| Data volume | UPF | Measured UL/DL/overall data traffic volume for the flow of the UE. |
| Packet loss rate | UPF | The maximum/ average packet loss rate of UL and/or DL. |
| Averaging time window | UPF | The averaging window used to measure the parameters, e.g. data rate, data volume, packet loss rate. |
| UE location | AMF, GMLC | The location of the corresponding UE, e.g. TA or cells that the UE enters, or the location in a finer granularity than cell level. |
| Time stamp | PCF, SMF, UPF, AMF | The time stamp associated to the collected data. |
| UE identifier | PCF, SMF, AMF | The identifier of UE, e.g. SUPI, UE IP address, etc. |

### 6.x.3	Output Analytics

The NWDAF supporting analytics on service experience associated to QoS results to consumer NFs, e.g. PCF or AF, as defined in Table 6.x.3-1.

Table 6.x.3-1: QoS and Policy Assistance analytics

| Information | Description |
| --- | --- |
| UE ID or list of UE IDs (1..SUPImax) | Identifies the UE(s) for which the statistic applies by a list of SUPIs. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > QoS and Policy Assistance information (1…max) (NOTE 1) | Service experience information of a list of candidate QoS parameter set(s). The candidate QoS could be the combination of the target QoS and allowed values of individual QoS parameters in the consumer request. Max. is the number of the candidate QoS parameter set(s), if applicable. |
| >> DNN | DNN for the PDU Session which contains the QoS flow. |
| >> S-NSSAI | Identifies the Network Slice used to access the application. |
| >> RAT Type | Indicate the list of RAT types for which the Service experience information of candidate QoS applies. |
| >> Access Type | Access Type when the UE establishes a PDU Session for the AF(s). |
| >> UE location | Indicate the UE location information when the service is delivered. |
| >> PDU session ID | The PDU session of the QoS flow associated to the PCC rule/QoS parameters belongs to. |
| >> PDU Session Type | The type of the PDU session, i.e. Ethernet PDU Session Type and IP PDU Session Type. |
| >> QFI | QoS Flow Identifier. The QoS flow that associated to the PCC rule/QoS parameters. |
| >> Application ID(s) | Identifies the application(s) that associated to the PCC rule or QoS. |
| >> PCC rule identifier | The identifier of the PCC rule associated to the QoS parameters. |
| >> QoS service experience | The service experience (e.g. QoE, MOS) of the corresponding QoS parameter set (e.g. average, maximum, minimum, variance). |
| >> Applicable duration of QoS and Policy Assistance information | The applicable duration/ time window of the QoS and Policy Assistance information. |
| >> Data volume | The UL/DL/overall data volume of the QoS flow within the valid duration. |
| >> Candidate QoS parameter set and characteristics | The values of one or more individual QoS parameters of each set and corresponding characteristics. |
| >>> 5QI | The reference to 5G QoS characteristics and QoS parameters. |
| >>> ARP | The QoS parameter ARP contains information about the priority level, the pre-emption capability and the pre-emption vulnerability, as defined in TS 23.501 [2]. |
| >>> RQA (NOTE 4) | Reflective QoS Attribute (RQA) only applies to Reflective QoS. The Reflective QoS Attribute (RQA) is an optional parameter which indicates that certain traffic (not necessarily all) carried on this QoS Flow is subject to Reflective QoS, as defined in TS 23.501 [2]. |
| >>> RQA timer | Associate RQA timer. |
| >>> Alternative QoS parameters | The alternative QoS parameters of the QoS flow. |
| >>> Flow Bit Rates (NOTE 2) | The flow bit rates only applies to GBR QoS Flow. |
| >>>> GFBR | Guaranteed Flow Bit Rate (GFBR) for UL and/or DL. |
| >>>> MFBR | Maximum Flow Bit Rate (MFBR) for UL and/or DL. |
| >>> Aggregate Bit Rates (NOTE 2) | The aggregate bit rate of PDU session, UE, or slice. |
| >>> Packet Loss Rate (NOTE 2) | The Maximum Packet Loss Rate (UL, DL) indicates the maximum rate for lost packets of the QoS Flow that can be tolerated in the uplink and downlink direction. This is provided to the QoS Flow if it is compliant to the GFBR |
| >>> Resource type | The resource type of the corresponding QoS flow, e.g. GBR QoS flow, non-GBR QoS flow, delay-critical QoS flow. |
| >>> Packet Delay Budget | Packet Delay Budget (PDB) indicates the upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [2]. |
| >>> Packet Error Rate | Packet Error Rate (PER) defines an upper bound for a rate of non-congestion related packet losses, as defined in TS 23.501 [2]. |
| >>> Averaging Window (NOTE 2) | The Averaging window is applied when the resource type is GBR QoS. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE), as defined in TS 23.501 [2]. |
| >>> Maximum Data Burst Volume (NOTE 3) | The Maximum Data Burst Volume (MDBV)applies to GBR QoS Flow with Delay-critical resource type. The MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| >>> Packet Filter Set | The Packet Filter Set is used in the QoS rule and the PDR to identify one or more packet (IP or Ethernet) flow(s), e.g. IP Packet Filter Set (as defined in clause 5.7.6.2 of TS 23.501 [2]) and Ethernet Packet Filter Set (as defined in clause 5.7.6.3 of TS 23.501 [2]). |
| >>>> PDU Set QoS Parameters | As defined in clause 5.7.7 of TS 23.501 [2], the PDU Set QoS Parameters are used to support PDU Set based QoS handling in the NG-RAN, including PDU Set Delay Budget (PSDB), PDU Set Error Rate (PSER), PDU Set Integrated Handling Information (PSIHI). |
| >> Validity period | The validity period within the time slot for the analytics on service experience associated to QoS in clause 6.1.3. |
| >> Spatial validity | Area where the analytics on service experience associated to QoS applies. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 2: The output analytics only applies to GBR QoS Flow. NOTE 3: The output analytics only applies to GBR QoS Flow with Delay-critical resource type. NOTE 4: The output analytics only applies to Reflective QoS. |  |

### 6.x.4	Procedures

The NWDAF may provide the QoS and policy assistance analytics to a consumer 5GC NF (e.g. PCF).

Figure 6.x.4-1: Procedure for QoS and Policy Assistance Analytics

1.	The Consumer NF, e.g. PCF or AF, requests or subscribes to QoS and policy assistance analytics from NWDAF (possibly via NEF in case the consumer NF is an untrusted AF) and provides the input information as specified in clause 6.x.1 to 5GC.

2a.	The NWDAF may subscribe to the service data from AMF as defined in Table 6.x.2-2 using Namf_EventExposure_Subscribe service operation for collecting UE location(s) for a UE or a group of UEs.

If NWDAF the required UE location information is finer granularity than TA/cell level, then NWDAF collects the location data from GMLC instead of AMF by invoking the Ngmlc_Location service as defined in TS 23.273 [39] and TS 29.515 [48].

2b.	The NWDAF may subscribe to service data from PCF as defined in Table 6.x.2-2 by invoking Npcf_EventExposure_Subscribe service operation (Event ID, PCC rule ID, SUPI(s) or Application ID).

2c-1.	The NWDAF may subscribe to service data from SMF as defined in Table 6.x.2-2 by invoking Nsmf_EventExposure_Subscribe service operation (Event ID, QoS profile ID, PCC rule ID, SUPI(s) or Application ID).

2d.	In order to provide the QoS flow Packet delay to NWDAF the SMF subscribe to QoS Monitoring information from UPF, as defined in Table 6.x.2-2, using the N4 Session level Reporting procedure defined in TS 23.502 [3].

2e.	The subscribed event is triggered in the UPF.

2c-2.	The SMF notifies the NWDAF of the subscribed service data.

2f.	Alternatively, the UPF may notify the subscribed event report directly to the NWDAF.

2g.	The NWDAF may subscribe to the service data from as defined AF in the Table 6.x-2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe (Event ID = Service Experience Associated to QoS, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)) service as defined in TS 23.502 [3].

2h.	The NWDAF may subscribe to the input data from the OAM as defined in the Table 6.18.2-1 according to the data collection principles described in clause 6.2.3.

3.	The NWDAF derives the requested analytics on Service Experience Associated to QoS.

4.	The NWDAF provides the requested Service Experience Associated to QoS to the consumer NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If the consumer NF subscribed to Service Experience Associated to QoS in step 1, once the NWDAF generates new analytics for Service Experience Associated to QoS, it provides a notification using Nnwdaf_AnalyticsSubscription_Notify to the Consumer NF.

* * *Next Change * * *

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| QoS and Policy Assistance | Analytics ID: QoS and Policy Assistance | Analytics on candidate QoS parameter set(s) and the associated QoE(s). |

* * *Next Change * * *

### 7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe

Description: Subscribes to NWDAF analytics and optionally Analytics Accuracy Information with specific parameters.

Inputs, Required:

-	(Set of) Analytics ID(s) as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional:

-	Analytics Filter Information;

-	Time window for historical analytics;

-	Subscription Correlation ID (in the case of modification of the analytics subscription);

-	Preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset;

-	Reporting Thresholds;

-	Maximum number of objects requested (max);

-	Preferred order of results;

-	Maximum number of SUPIs requested (SUPImax);

-	Time when analytics information is needed;

-	Analytics Metadata Request;

-	(Set of) NWDAF identifiers used by the NWDAF service consumer when aggregating multiple analytic subscriptions;

-	Dataset Statistical Properties;

-	Output strategy;

-	Data time window;

-	Consumer NF's serving area or NF ID;

-	Information of previous analytics subscription, i.e. NWDAF identifier (i.e. Instance ID or Set ID), Analytics ID(s) (including SUPIs and Analytics Filter Information for UE-related Analytics) and Subscription Correlation ID;

-	Use case context.

-	Analytics Accuracy Request information.

-	Analytics Feedback Information.

NOTE 2:	Analytics Filter Information, Reporting Thresholds, maximum number of objects requested (max), maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

NOTE 3:	Analytics Feedback Information only can be included in modification request for the existing analytics subscription.

NOTE 4:	Other input parameters specific for different analytics ID can be also needed, as specified in the corresponding clause for each analytics ID.

-	Target QoS parameter set(s), corresponding PCC rules, allowed values of individual QoS parameters, target QoE.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: First corresponding Analytics report is included, if available and if analytics consumer requested immediate reporting (see clause 4.15.1 of TS 23.502 [3]).

NOTE 5:	When the Target of Analytics Reporting is a SUPI or a GPSI then the subscription may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSIs or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

* * *Next Change * * *

### 7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request

Description: The consumer requests NWDAF operator specific analytics and optionally Analytics Accuracy Information.

Inputs, Required: (Set of) Analytics ID(s) as defined in Table 7.1-2, Target of Analytics Reporting, Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional: Analytics Filter Information, preferred level of accuracy of the analytics, preferred level of accuracy per analytics subset, time when analytics information is needed, maximum number of objects requested (max), preferred order of results, maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window, Use case context, Time window for historical analytics and Analytics Accuracy Request information, target QoS parameter set(s), corresponding PCC rules, allowed values of individual QoS parameters, target QoE..

NOTE 2:	Analytics Filter Information, Maximum number of objects requested (max), Maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

Outputs, Required: If the request is accepted, then set of the tuple (Analytics ID, Analytics specific parameters). When the request is not accepted, an error response.

Outputs, Optional: Timestamp of analytics generation (required when ADRF is deployed), validity period, confidence, revised waiting time, Analytics Metadata Information, Analytics Accuracy Information. See clause 6.1.3.

NOTE 3:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

NOTE 4:	When the Target of Analytics Reporting is a SUPI or a GPSI then the request may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSIs or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

NOTE 5:	Other input or output parameters specific for different analytics ID can be also needed, as specified in the corresponding clause of each analytics

* * *End of Changes * * *