3GPP TSG-WG SA2 Meeting #164	S2-2407762

Maastricht, Netherlands, August 19 - 23, 2024

Source:	T-Mobile USA

Title:	Interim Conclusions for Simple AIoT Devices

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: This pCR proposes some preliminary interim conclusions.

# 1	Discussion

This paper provides some preliminary interim conclusions to the TR based on the strong operator need to support very simple type A and type B devices based on both technical and business requirements. These very simple devices will likely use topology 2 based on use cases and the fact these very simple devices have limited capabilities.

The Ambient IoT study covers a range of device types A, B, and C. The interim conclusions here are only directed at the most simple devices of types A and B. This paper suggests to agree upon some “principles” that can be adopted from several solutions currently in the TR. How to incorporate them into the specifications can be determined during normative phase.

This paper places no restrictions on normative solutions for type C devices and other topologies. It is expected that the final normative specification might support different approaches for low vs. high capability devices. Further, this paper recognizes that any security related aspects will need to be coordinated with SA3.

# 2	Proposal

It is proposed to include the below changes into TR 23.700-13.

First Changes

# 8	Conclusions

## 8.X	General Principles

The following interim conclusions are agreed for normative work regarding Ambient IoT with type A and type B AIoT devices:

- Support of Topology 2.

- Support of AIoT device type A and type B.

- Support bulk out of band provisioning and MNO network on-boarding of AIoT devices.

- Support AIoT devices to be pre-provisioned with:

  - Device IDs.

  - Security credentials that allow secure identification of AIoT devices and communications between AIoT device and network.

- Allow efficient scaling for transmitting downlink triggers/commands and data to a large number of AIoT devices.

- Allow efficient scaling for receiving uplink data from a large number of AIoT devices.

- Depending on operator policies, allow the possibility of avoiding an over the air network registration procedure for AIoT devices.

- The AIoT device ID shall not be permanently tied to a specific MNO.

End of Changes