SA WG2 Meeting #164	S2-2407773

19th August – 23rd August, 2024, Maastricht, Netherland

Source:	Lenovo

Title:	Key Issue #4, Conclusions

Document for:	Approval

Agenda Item: 	19.3.1

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes conclusion principles for KI#4

1	Discussion

After SA2#163 meeting, the following interim conclusions were agreed for KI#4. Some parts of the current interim conclusions still need to be modified, which are listed as follows:

- The EN about a new UE capability indication. Both the UE and the UPF should be able to support handling the additional Packet Filter to further identify the multiplexed media streams. For UE, it is recommended that the UE can report its capability of supporting the additional Packet Filter to the 5GC, e.g., via 5GSM capability during PDU session establishment/modification procedure. The SMF and PCF will take this into consideration when generating the QoS rules for UL handling. For UPF, the SMF can choose the UPF that supports the additional Packet Filter for DL handling. The support of the additional Packet Filter depends on the current tranport protocols being used, e.g., RTP, QUIC etc since the content of the additional Packet Filter may change accordingly.

- Include the encryption situation. Right now, there is no wording related to the encryption media streams in the interim conclusions. For encryption scenario, the logic to identify the multiplexed media stream is basically the same as the unencrypted scanrio, which relies on introducing a new/additional packet filter that has the identifier to identify the encrypted media streams. The content of the additional packet filter can be further decided based on the conclusion in KI#2 about which end-to-end traffic will be adopted, such as MoQ and Proxy-UDP-in-HTTP/3+QUIC-Aware Proxy.

- Refine the wordings to better reflect the core points.

2	Proposal

It is proposed to include the following changes in TR 23.700-70.

* * * * Start of Changes – All New Text * * * *

8.4	Conclusions for Key Issue #4

The following aspects are concluded as principles for normative work:

1. The AF may provide QoS requirements for media flows and additional packet filters as part of "AF session with required QoS". The detectable media flows that require different QoS treatment are mapped to distinct QoS Flows. If the AF does not provide any QoS requirement for the media flows or it provides the same QoS requirement, then the media flows can be mapped to the same QoS Flow. The additional Packet Filter will be provided in the “AF session with required QoS” procedure to the NEF/PCF and used if the multiplexed media flows cannot be differentiated using the legacy Packet Filter.

2. In order to unique identify each media flow, the additional packet filter is used together with the legacy packet filter. The additional Packet Filter is used to differentiate the media flow among multiple media flows that share the same legacy packet filter both in uplink (in UE) and downlink (in UPF) in the following case:

a) For the media flows that are transported in (S)RTP, (S)RTCP and other associated protocols that are multiplexed into a single UDP/IP traffic flow as specified in IETF RFCs 5761, 5764, 7983, 8872, 8843 and 9443. The additional Packet Filter SHALL be the Synchronization Source (SSRC), Payload Type (PT) and RTP-M header fields.

NOTE: For encrypted media flows, the components of the additional Packet Filter will be determined based on the conclusion of KI#2, e.g., which end-to-end encrypted XRM traffic is adopted.

3. The PCC rule and N4 rules are enhanced to support the additional Packet Filter along with legacy packet filter.

4. The SMF may provide the received additional Packet Filter along with legacy packet filter to the UPF (that supports the additional packet filter) in the N4 rules. The PDI in PDR is extended to support the additional Packet Filter.

5. The SMF provides QoS rules for the uplink traffic for the UE. QoS rules are enhanced to include additional Packet Filter when required (additional packet filter is not expected to be interpreted by a legacy UE; in case of legacy UE, it will use legacy packet filter). If supported by the UE, the UE uses both the legacy packet filter and the additional packet filter for uplink QoS Flow mapping.

NOTE: UE may report the capability of supporting the additional Packet Filter to the 5GC to help determine the PCC rules and QoS rules, e.g., whether to include the additional packet filter and provide to the UE.

NOTE: It is recommended that CT1 specifies additional packet filter in a backward compatibility manner so that legacy UE can match the XRM traffic to a QoS flow with the QoS requirement defined in r18 XRM.

* * * * End of Changes * * * *