3GPP TSG SA WG2 Meeting #164	S2-2407781

Maastricht, Netherlands, August 19 – August 23, 2024

Source: 	Samsung

Title:	Discussion on URSP evaluation for a PDN connection

Document for:	Approval

Agenda Item:	9.25.2

Work Item / Release:	eUEPO

Abstract of the contribution: It is proposed to discuss how to handle URSP rules for a PDN connection in 5GS with EPS interworking.

# Discussion

Based on current specifications, during establishment of non-emergency PDN connection in EPC, if the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C selects a S-NSSAI and provides it to the UE, as specified in the first and the sixth paragraph of clause 4.11.0a.5 in TS 23.502.

[Clause 4.11.0a.5 in TS 23.502 V18.6.0]

During establishment of non-emergency PDN connection in the EPC, the UE and the SMF+PGW-C exchange information via PCO as described in clause 5.15.7 of TS 23.501 [2]. For UE with 5GC NAS capability disabled (i.e. N1 mode is disabled), the UE may also allocate a PDU Session ID and send it to the SMF+PGW-C via PCO. If the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the SMF+PGW-C provides an S-NSSAI to the UE, the SMF+PGW-C should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs which supports interworking with EPC and that the S-NSSAI is not subject to Network Slice-Specific Authentication and Authorization, by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the SMF+PGW-C discovers and selects a UDM as described in clause 6.3.8 of TS 23.501 [2]). If the SMF+PGW-C is in a VPLMN, the SMF+PGW-C uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values. If the S-NSSAIs supported by the SMF+PGW-C are all subject to NSSAA, then the SMF+PGW-C should reject the PDN connection establishment. If the selected S-NSSAI is subject to NSAC and EPS counting is required for the S-NSSAI, the SMF+PGW-C uses the Nnsacf_NSAC_NumberOfUEsUpdate services operation and/or the Nnsacf_NSAC_NumberOfPDUsUpdate services operation to check if the selected S-NSSAI is available as described in clause 4.11.5.9. The SMF+PGW-C uses the Nudm_SDM_Subscribe service operation to subscribe the change of the Session Management Subscription data. If the SMF+PGW-C is notified from UDM with subscription data change, the SMF+ PGW-C takes actions for the PDN connection as described in clause 5.17.2.1 of TS 23.501 [2].

(…)

During establishment of non-emergency PDN connection in the EPC, if SMF+PGW-C is selected for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN and if the SMF+PGW-C supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C+PGW-C may proceed as specified in first paragraph of this clause or select any S-NSSAI associated with the APN of the PDN connection. The SMF+PGW-C shall not provide any 5GS related parameters to the UE.

When the UE has established a PDN connection in EPC, and if application traffics occurred in the UE, the UE checks whether there is an existing PDN connection for the APN which is the same APN that the application traffics should be routed. If it is existing, then the UE will proceed the corresponding procedures such as UE requested bearer resource modification towards the existing PDN connection.

However, this will cause the mismatch with URSP rule for that application. For example, the SMF+PGW-C has selected an S-NSSAI (S-NSSAI#1) for the existing PDN connection, and the application (Application#2), which is executed later is associated with the different S-NSSAI (S-NSSAI#2) but using the same APN/DNN according the URSP rule, will still use the existing PDN connection.

Example:

- PDN Connection#1 (existing one) connects to the APN#1(DNN#1), and S-NSSAI#1 is selected. Application#1 is routed to PDN Connection#1.

- Application#2 (occurred later) is routed to PDN Connection#1, because Application#2 is connected to APN#1(DNN#1). But the URSP rule says it is connected to the combination of APN#1(DNN#1) and S-NSSAI#2.

This becomes an issue when the UE moves from EPS to 5GS, and moves the PDN connection (PDN Connection#1) for the application (Application#2) that has been executed and routed to the existing PDN connection. Because, the UE will request PDU Session establishment for that application with the S-NSSAI (S-NSSAI#1) which does not match with the URSP rules.

Example (continued):

- When the UE moves from EPS to 5GS, the UE will request a PDU Session establishment to move the PDN Connection#1 for the Application#2 and the S-NSSAI#1 will be selected in 5GS for this PDU Session. However, this does not match with the URSP rule for the Application#2.

Issue: When the UE moves from EPS to 5GS, the S-NSSAI which does not match with the URSP rule may be selected for the PDU Session corresponding to the existing PDN Connection.

To resolve this issue, a number of options can be considered as following:

Option#1 (URSP evaluation with S-NSSAI): If the UE receives the URSP rules with S-NSSAI(s) over EPS, the UE evaluates the URSP rule with considering S-NSSAI. If there is an existing PDN connection with the same APN/DNN but different S-NSSAI, the UE initiates establishment of an additional PDN connection for the same APN/DNN with the S-NSSAI via PCO to the SMF+PGW-C, so that the SMF+PGW-C selects the S-NSSAI for this PDN connection considering the S-NSSAI value received from the UE.

Option#2 (URSP re-evaluation on EPS to 5GS Mobility): During EPS to 5GS Mobility procedure, the UE re-evaluates the URSP rules. Based on the URSP re-evaluation result, the UE detects that there is no existing PDU Session matching the URSP rule (for the case when the S-NSSAI is different) and the UE initiates a request for PDU Session Establishment with the S-NSSAI corresponding to the URSP rule. As a result, if the UE detects that an existing PDN connection/PDU Session has no associated application, the UE decides to release the PDN connection/PDU Session.

Based on above, it is proposed as below.

Proposal: It is proposed to resolve the issue based on the option(s) discussed above. To avoid the misuse of network slices when the UE is in EPS, Option#1 is more needed and preferred as described in 23.502 CR. See S2-2407782.

# Proposal

This authors of this paper propose below.

Proposal: It is proposed to resolve the issue based on the option(s) discussed above. To avoid the misuse of network slices when the UE is in EPS, Option#1 is more needed and preferred as described in 23.502 CR. See S2-2407782.