3GPP SA WG2 Meeting #164	S2-2407788

Maastricht, NL, 19-23 August 2024

Source:	China Telecom

Title:	A review of Rel-18 study on multi-modal services to help reply the LS

Document for:	Discussion

Agenda Item:

Work Item / Release:

Abstract: This paper reviews the Rel-18 study on multi-modal services so that we can reach consensus on how to reply the LS received from RAN2.

# 1. Summary

This paper reviews Rel-18 study on the scope of multi-modal services and synchronization thresholds related discussions so that we can reach consensus on whether/what information can be provided to support multi-modality awareness at RAN. This paper is related to S2-247789, which is the LS out to RAN2.

# 2. Review of Rel-18 Study on Multi-modal Services

During Rel-18 study phase of FS_XRM, we experienced difficult discussions on the scope of multi-modal services, which are as following:

At SA2#148e, S2-2109360 was agreed and this is what was sent to the workshop before SA#94e. The text of WT#1 from this version of the SID is shown below. The RAN Dependency indicated “Maybe”.

Study whether and how to enable delivery of related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) to the user at a similar time.

At SA#94e, SP-211646 was approved and the modified WT#1 text is shown below, and the RAN Dependency still indicates “Maybe”.

Study whether and how to enable delivery of related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) with an application to the user at a similar time, focusing on the need for policy control enhancements (e.g. QoS policy coordination).

Unfortunately, some companies still had different understanding about the scope of WT#1, therefore, at SA#96, WT#1 was further revised in SP-220715 and the text is shown below, and the RAN Dependency still indicates “Maybe”.

Study whether and how to enable delivery of related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) with an application to the user at a similar time, using only control plane enhancements (i.e. QoS policy coordination, PCC enhancement for the 5GS). No N3/N9 user plane impact.

NOTE 1: Any parameters provided by 5GC to NG-RAN via the Control plane need to have a clear explanation about how they are intended to be used by NG-RAN.

A few things should be pointed out about the modifications of WT#1 text:

- To support multi-modal services, the enhancements are limited to QoS policy coordination and PCC enhancement in the control plane, not per packet/frame synchronization.

- It is reasonable to provide some parameters from 5GC to NG-RAN, as long as we give a clear explanation about the usage of these parameters.

Based on KI#1 & KI#2 solutions in TR 23.700-60, we made conclusions and further defined Multi-modal Service ID to support the delivery of multi-modal services, the related descriptions in TS 23.501 clause 5.37.2 and TS 23.503 clause 6.1.3.27.3 are shown below:

The Nnef_AFsessionWithQoS service allows the AF to provide, at the same time, for each data flow that belongs to the multi-modal service, a Multi-modal Service ID, the service requirements and the QoS monitoring requirements:

-	The Multi-modal Service ID is an explicit indication that data flows are related to a multi-modal service. The PCF may use this information to derive the correct PCC rules and to apply appropriate QoS policies for the data flows that are part of a specific multi-modal application.

-	The AF may provide QoS monitoring requirements for data flows associated to a multi-modal service to the PCF . The PCF generates the authorized QoS Monitoring policy for each data flow.

Following additional attributes are supported where the AF provides specific information for multi-modal service:

-	Multi-modal Service ID: an identifier that refers to the multi-modal communication service. Data flows belonging to the same multi-modal service share the same Multi-modal Service ID.

Observation 1: When received the Multi-modal Service ID from the AF, the PCF can use this information to derive the correct PCC rules and to apply appropriate QoS rules for the data flows belonging to a specific multi-modal application. It is reasonable that the PCF can further provide Multi-modal Service ID to the SMF, and then to the NG-RAN so that the NG-RAN can identify flows belong to the same multi-modal services and treat these flows jointly (e.g. joint admission control, joint resource reservation).

Proposal 1: It is proposed that 5GC can provide Multi-modal Service ID to the NG-RAN to help identify flows belonging to the same multi-modal services.

# 3. Discussion on the coordinated transmission requirements

Multi-modal flows can transfer different types of data (e.g. audio, video, haptic data, positioning) and may come from different sources. Synchronization between different media components is critical in order to avoid having a negative impact on the user experience. Based on TS 22.261 clause 7.11 and clause 6.43, part of the latency KPI requirements and the typical synchronization thresholds are shown below:

Table 7.11-1: Multi-modal communication service performance requirements

| Use Cases | Characteristic parameter (KPI) |  |  | Influence quantity |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Message size (byte) | UE Speed | Service Area |  |
| Immersive multi-modal VR (UL: device application sever) | 5 ms (note 2) | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) [40] | 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 More DoFs can be supported by the haptic device | Stationary or Pedestrian | typically < 100 km2 (note 5) | Haptic feedback |
|  | 5 ms | < 1Mbit/s | 99.99% [40] | 1500 | Stationary or Pedestrian | typically < 100 km2 (note 5) | Sensing information e.g. position and view information generated by the VR glasses |

Table 6.43.1-1: Typical synchronization thresholds for immersive multi-modality VR applications

| Media components | synchronization threshold (note 1) |  |
| --- | --- | --- |
| audio-tactile | audio delay: 50 ms | tactile delay: 25 ms |
| visual-tactile | visual delay: 15 ms | tactile delay: 50 ms |
| NOTE 1: for each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |

Observation 2:

To guarantee the synchronization of multi-modal flows, there are two kinds of transmission requirements:

1) The delay for specific flow should satisfy the latency KPI requirements for the application. e.g. for immersive multi-modal VR UL, the max allowed end-to-end latency for haptic data is 5ms.

2) The delay difference value between two different media components should be lower than the synchronization thresholds. e.g. for immersive multi-modality VR applications, the synchronization threshold for visual-tactile is less than 15ms (if the visual data is delayed compared to the tactile) or less than 50ms (if the tactile is delayed compared to the visual).

Additionally, based on TS 23.501 clause 5.37.2, for each data flow that belongs to the multi-modal service, the AF may provide the services requirements and the QoS monitoring requirements to the PCF, and then the PCF generates the authorized QoS Monitoring policy for each data flow.

Observation 3: The latency requirements of each multi-modal flow can be introduced as a QoS parameter. When this information is provided from the AF to the PCF, the PCF uses this information to derive correct PCC rules to the SMF, and the SMF provide this QoS parameter to the NG-RAN as part of the QoS profile.

Proposal 2: It is proposed that the latency requirements for specific multi-modal flow can be provided as a QoS parameter to the NG-RAN, how the NG-RAN use this information for coordinated handling is up to implementation.

Observation 4: By executing the QoS monitoring policy for each multi-modal flow, it is feasible to get the end-to-end delays of each flow and calculate the real delay difference between different media components. If the real delay difference value can be kept lower than the synchronization thresholds, the synchronization between multi-modal flows with the same multi-modal service ID can be guaranteed.

NOTE: The NG-RAN and the UPF can schedule the delivery of multi-modal flows to keep the real delay difference between specific couple of flows not to exceed the synchronization thresholds, which is up to implementation.

Proposal 3: It is proposed that the synchronization thresholds between different media components can be provided as a QoS parameter to the NG-RAN, how the NG-RAN use this information for coordinated handling is up to implementation.

# 4. Conclusion

Observation 1: When received the Multi-modal Service ID from the AF, the PCF can use this information to derive the correct PCC rules and to apply appropriate QoS rules for the data flows belonging to a specific multi-modal application. It is reasonable that the PCF can further provide Multi-modal Service ID to the SMF, and then to the NG-RAN so that the NG-RAN can identify flows belong to the same multi-modal services and treat these flows jointly (e.g. joint admission control, joint resource reservation).

Observation 2: To guarantee the synchronization of multi-modal flows, the delay for specific flow should satisfy the latency KPI requirements for the application and the delay difference value between two different media components should be lower than the synchronization thresholds.

Observation 3: The latency requirements of each multi-modal flow can be introduced as a QoS parameter. When this information is provided from the AF to the PCF, the PCF uses this information to derive correct PCC rules to the SMF, and the SMF provide this QoS parameter to the NG-RAN as part of the QoS profile.

Observation 4: By executing the QoS monitoring policy for each multi-modal flow, it is feasible to get the end-to-end delays of each flow and calculate the real delay difference between different media components. If the real delay difference value can be kept lower than the synchronization thresholds, the synchronization between multi-modal flows with the same multi-modal service ID can be guaranteed.

Proposal 1: It is proposed that 5GC can provide Multi-modal Service ID to the NG-RAN to help identify flows belonging to the same multi-modal services.

Proposal 2: It is proposed that the latency requirements for specific multi-modal flows can be provided as a QoS parameter to the NG-RAN, how the NG-RAN use this information for coordinated handling is up to implementation.

Proposal 3: It is proposed that the synchronization thresholds between different media components can be provided as a QoS parameter to the NG-RAN, how the NG-RAN use this information for coordinated handling is up to implementation.