3GPP SA WG2 Meeting #164	S2-2407791

Maastricht, NL, 19-23 August 2024	(was S2-240xxxx)

Source:	China Telecom

Title:	KI#5: Conclusions on Data Burst Transmission Enhancement

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract: This paper proposes KI#5 conclusions on Data Burst Transmission Enhancement.

1.	Discussion

After SA2#163 meeting, Burst Size related enhancements have been concluded for KI#5, while Data Boost related enhancements still need more discussions.

According to TS 23.501, the terminology “Data Burst” defined in Rel-18 is as following:

Data Burst: A set of multiple PDUs generated and sent by the application in a short period of time.

NOTE 1:	A Data Burst can be composed of one or multiple PDU Sets.

According to Sol#16 in TR 23.700-70, the following sentence implies that the concept “Data Boost” is quite similar to “Data Burst”:

Based on application requirements in the AS, AS instructs UE to send a higher resolution data file (e.g. video/jpeg). Because AS is aware that the data size from the UE is expected to be large and timeliness to receive this information is crucial for the application, AS includes the request to expediate this transfer to the 5GS in the metadata.

Observation 1: Both “Data Boost” and “Data Burst” indicate the rapid increasing of data within a short period of time, therefore, Data Boost handling enhancement can be regarded as a kind of Data Burst transmission enhancement. Therefore, we can certainly use the terminology “Data Burst” for KI#5 conclusions.

What’s more, in Sol#16, the UPF behaviors to expedite DL/UL data transfer are actually not the same:

1) When received the “expedited transfer indication” from the AS, the UPF reselects an established QoS Flow with higher quality 5QI to perform the subsequent DL data transfer;

2) When received both the RQI marking instruction from the SMF and the “expedited transfer indication” from the AS, the UPF performs RQI marking to enable the UE to derive QoS rules for UL data transfer.

Observation 2: It is necessary to describe the UPF behaviors for DL and UL Data Burst transmission enhancement respectively.

Therefore, taking the above into consideration, this paper proposes the following KI#5 conclusions on Data Burst transmission enhancement.

2.	Proposal

It is proposed to update TR 23.700-70 as follows.

* Start of change *

## 8.X	Conclusions for Key Issue#5

The following aspects are concluded as principles for the normative work:

-	Support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling.

1.	The AF may indicate to the NEF/PCF to mark the Burst size.

2.	Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size.

3.	Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header.

NOTE:	This conclusion assumes that SA4 will specify Burst size indication in the header extension (i.e. RTP HE).

4.	The NG-RAN may use the received Burst size to assist radio resource management.

-	Support the AS triggered Data Burst transmission enhancement to adapt the changed traffic characteristics:

1.	QoS flows with different 5QIs are established during PDU session establishment procedure, the SMF can configures two QoS flows with one for normal data transmission and the other for Data Burst transmission.

2.	The AS triggers Data Burst Transmission Enhancement by providing “expedited transfer indication” in N6 metadata to 5GS.

3.	The PCF authorizes QoS flow with different 5QIs can be used with reflective QoS to support the dynamic changes in traffic.

4.	The UPF, based on the received “expedited transfer indication” from AS, switching to an established QoS Flow which can support Data Burst transmission to perform the subsequent DL data transmission.

5.	The UPF, based on the received the RQI marking instruction from SMF and the “expedited transfer indication” from AS, performs RQI marking to enable the UE to derive QoS rules and select suitable QoS flow for UL data transmission.

NOTE: “Expedited transfer indication” is expected to be an indication that will be defined along with other metadata for PDU set information in KI#2.

* End of changes *