3GPP TSG-SA2 Meeting #164	S2-2407799

# Maastricht, Netherlands, August 19 – 23, 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3793 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Support for Store and Forward Satellite operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusion of TR 23.700-29, two deployment options are aggreed to support store and forward satellite operation: split MME and full CN. This contribution provides general concept and architecture options to support store and forward satellite operation. An additional point to note is that the UE need to be able to distinguish between the different satellite architecture in order to operate accordingly. E.g. attach and detach with every satellite for full CN onboard architecture or attempt to TAU with other satellite for split MME architecture. For full CN onboard architecture, although the MME in satellite can perform a NAS detach of the UE, the satellite coverage can be lost before the detach procedure. In this case, If the UE knows the architecture option, the UE can perform a local detach. The UE may also trigger detach procedure before coverage lost or when the UE finished data transfer for full CN onboard architecture. Serveral options for the UE to know the S&F architecture: - Option1: Indication in SIB from the eNB onboard satellite - Option2: Based on the Attach or TAU reject cause - Option3: Indication to the UE during Attach or TAU procedure This CR propose to use option 2, based on the new reject cause introduced in the conclusion, the UE can know the architecture option used for S&F operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce general concept and architecture to support store and forward satellite operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS cannot support store and forward satellite operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.X (new), 4.13.X.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.13.X	Support of Store and Forward Satellite operation

4.13.X.1	General

To support Store and Forward satellite operation, eNodeB and core network entities are placed on board the satellite. The following deployment options are supported for store and forward satellite operation:

-	Split MME: MME functionality is splited into two parts, MME-onboard and MME-ground.

-	Full CN onbard: The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite.

For both deployment options, when feeder link is not available and the network support S&F operation, the network shall be able to inform UEs whether Store and Foward Satellite operation is applied (e.g. eNB broadcast support of S&F operation as part of System Information)

During the Attach or TAU procedure, the UE indicates support for S&F mode to the MME following existing NAS capability and the MME may provide a list of Satellite IDs over which the UE may exchange the signalling and data, and a wait-timer that indicates to the UE the time it should wait before attempting signalling and data exchanges with satellites. The Satellite IDs are based on the SIB information broadcasted by eNB.

NOTE X:	A list of satellite IDs can be determined by the CN. How the network determines the list of satellite IDs is outside the scope of 3GPP in this release of specification.

For split MME architecture, the MME may send Attach, TAU, or Service Reject message to the UE if these procedures cannot be completed due to S&F operation. This indicates to the UE that the information contained in Attach, TAU, Service Request message is stored by the MME and the network will be available to the UE after interaction with ground network. This also indicates to the UE that the satellite operates in split MME architecture.

For full CN onboard architecture, The UE attaches, transfers data (e.g. SMS, MO and MT data, etc.) and detaches from each satellite as required and as determined by the list of satellite IDs provided by the network. The UE may be rejected with different cause described above in the split MME archtecture if the satellite cannot support the UE at this time.

In the following, Store and Forward satellite operation with Split-MME architecture is described in clause 4.13.X.2, and Full CN onboard architecture is described in clause 4.13.X.3.

* * * * End of Changes * * * *