3GPP TSG-SA2 Meeting #164	S2-2407800

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1134 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General training procedure for NWDAF initiated Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to specify the general training procedure for vertical federated learning between AF(s) and NWDAF(s) with the NWDAF as the VFL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added to specify the general training procedure for vertical federated learning between AF and NWDAFs, with the NWDAF as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL training procedure not specified with the NWDAF acting as VFL server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

6.2X.Y.Z General training procedure for NWDAF initiated Vertical Federated Learning between NWDAF(s) and AF(s)

The figure 6.2X.Y.Z-1 below shows general training procedure for NWDAF initiated Vertical Federated Learning between NWDAF(s) and AF(s).

9. Model subscription response

Figure 6.2X.Y.Z-1: General training procedure for VFL between NWDAF(s) and AF(s)

1. VFL clients selection

0.	The consumer sends a subscription request to VFL server NWDAF to train an ML model, including Analytics ID, ML model metric (e.g. ML model accuracy).

NOTE 1:	As an example, the consumer could be NWDAF containing AnLF or VFL server NWDAF to trigger VFL server NWDAF to start VFL training.

1.	VFL server NWDAF selects the VFL clients that participate in VFL (e.g. VFL client NWDAF, AF) as described in the clause X.

2.	To start VFL training, VFL server NWDAF sends a request to the selected VFL clients. The request includes initial ML model. The request also includes sample alignment information, e.g. UE ID(s) whose corresponding data is used for the VFL model training, and VFL model correlation ID, which identifies the VFL model training process.

3.	[Optional] Each VFL client collects its local data by using the current mechanism if the VFL client has not local data available already.

4.	During VFL training procedure, each VFL client further trains the ML model based on the initial ML Model provided by the VFL server NWDAF and its own data, and reports the local ML model information (e.g. intermediate training result) to the VFL server NWDAF. The report also includes VFL model correlation ID, and intermediate training result identification information, which identifies the intermediate result in the VFL training service, e.g. time stamp.

5.	The VFL server NWDAF computes the backward local ML model information (e.g. gradient information or loss information) based on all the local ML model information and label. The loss information may include loss function, the value of loss, the type of loss function or type of loss. The backward local ML model information is used for updating the local ML model. Different backward local ML model information may be computed for different VFL clients, respectively.

The VFL server NWDAF may also compute the global ML model metric (e.g. ML model accuracy) based on all the all the local ML model information and the label.

6a.	[Optional] Based on the consumer request in step 0, the VFL server NWDAF sends VFL status report to update the ML model metric to the consumer.

6b.	[Optional] The consumer decides whether the current model can fulfil the requirement, e.g. ML model metric is satisfactory for the consumer and determines to stop or continue the training process. The consumer continues the training process or stops the training process.

6c.	[Optional] Based on the subscription request sent from the consumer in step 6b, the VFL server NWDAF updates or terminates the current VFL training process.

If the VFL server NWDAF received a request in step 6b to stop the Federated Training process, steps 7 and 8 are skipped.

7.	If the VFL procedure continues, VFL server NWDAF sends the backward local ML model information (e.g. gradient information, loss information) to the VFL clients for next round of VFL.

8.	Each VFL client computes gradient of its local model and updates its local ML model based on backward local ML model information distributed by the VFL server NWDAF at step 7.

NOTE 4:	The steps 4-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

NOTE 5:	If untrusted AF is involved in VFL Clients, the message between 5GC NF and the untrusted AF is via NEF.

NOTE 6:	After VFL model training is terminated, VFL server NWDAF may collect trained ML model from VFL clients. This aims to support VFL model inference with participants that have not involved in VFL model training procedure.

9. After the training process is complete, the VFL Server NWDAF responses to the consumer about the ML model information.

# >>>>END OF CHANGES<<<<