3GPP TSG-SA2 Meeting #164	S2-2407801

# Maastricht, Netherlands, August 19 – 23, 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3794 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MO/MT transmission for split MME architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusion of TR 23.700-29, two deployment options are aggreed to support store and forward satellite operation: split MME and full CN. This contribution provides general description of MO/MT data transmission for split MME architecture |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description to support MO/MT data transmission for split MME architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EPS cannot support MO/MT data tranmission in store and forward satellite operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.X.2.4 (new), 4.13.X.2.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.13.X.2.4	MO data transmission

For a UE attached with store and forward satellite operation and in ECM-IDLE, the UE attempts to accesse a satellite which support store and forward operation, it may be one of the satellites included in the list of satellite IDs provided by the MME. The UE may send MO data to the satellite after TAU procedure, Service Request procedure, or using Control Plane Service Request.

The MO data is stored in the MME-onbaord when CP CIoT EPS Optmisation is used, or stored in the eNodeB onboard satellite, during the feeder link is unavailable. Then transferred to the ground when the feeder link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

* * * * Start of 2nd Change * * * *

4.13.X.2.5	MT data transmission

For a UE attached with store and forward satellite operation and in ECM-IDLE, the MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the satellite when the feeder link becomes available. The MME determines a satellite which will serve the UE to deliver MT data. The MME can determine the satellite which will serve the UE in shortest time. It may be one of the satellites included in the list of satellite IDs provided to the UE.

The MT data is stored in the satellite when the feeder link to the satellite is available and service link is unavailable. For a UE using CP CIoT EPS Optimisation, the user data is stored in the MME-onboard, or the user data can be stored in the eNodeB.

The MT data stored in the satellite is transferred to the UE after paging when service link becomes available. All types of data traffic (e.g. IP, etc.) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

Features described for High latency communication in clause 4.3.17.7 may be used to handle mobile terminated (MT) communication with UEs operate in store and forward mode, and the time until the feeder link with the satellite to be available to forward the data may be used when determining the Downlink Buffering Duration time.

* * * * End of Changes * * * *