3GPP TSG-SA2 Meeting #164	S2-2407802

# Maastricht, Netherlands, August 19 – 23, 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1135 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General Description of Vertical Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the conclusion of TR 23.700-84, this contribution provides general description to support vertical federated learning in 5GC |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description to support vertical federated learning in 5GC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support vertical federated learning in 5GC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

DCAF	Data Collection Application Function

RE-NWDAF	Roaming Exchange Network Data Analytics Function

VFL	Vertical Federated Learning

* * * * Start of 2nd Change * * * *

5.X	Vertical Federated Learning (VFL) among multiple NWDAFs and AFs

Vertical Federated Learning (VFL) involving NWDAF and/or AF, where no raw data need to be exchanged but some level of coordination is still required when training and inference are performed on local ML Models. In particular, datasets used for each local ML Model need to share the same samples (e.g. UE IDs) while holding different features. VFL supported by NWDAF and/or AF in the following scenarios:

-	VFL among NWDAFs in a single PLMN.

-	VFL between AF and NWDAF(s) in a single PLMN.

The main functionality includes:

VFL server:

-	An NWDAF or AF that integrates local training results for the local ML Model update in VFL training process.

-	coordinates the VFL training process by discovering and selecting VFL clients.

-	triggers sample alignment and selects samples to be used in the training process

-	aggregates intermediate results from VFL client(s) and computes intermediate training information (e.g. gradient information, loss information) for updating its own local ML Model and the ML Models of VFL clients during the VFL training process and sends the intermediate training information towards VFL clients involved in the joint VFL training process.

-	In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer.

VFL client:

-	An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server.

-	computes the intermediate results for their local ML Models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

-	receives intermediate training information (e.g. gradient information, loss information) and updates local ML Model based on the received information.

The NWDAF as a VFL Server determines based on internal logic and the operator's policy, whether to use VLF to train an ML Model or to provide a particular analytics ID.

VFL server NWDAF or VFL client NWDAF register to NRF with NF profile including VFL capability information as described in clause 5.2. For an untrusted AF as the VFL client, the NEF registers based on configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes as part of the information about the AF an VFL capability information (VFL capability type (i.e. VFL Clients)).

The NWDAF as VFL server will select NWDAF(s) and/or AF(s) (via NEF profile for untrusted AF) as candidate VFL client(s) from NRF for VFL training process. The AF (via NEF in case of untrusted AF) as VFL server will select candidate NWDAF(s) as VFL client(s) from NRF for VFL training process.

The detailed procedure for VFL among Multiple NWDAFs and AFs is described in clause 6.2X.

* * * * End of Changes * * * *