3GPP TSG-SA2 Meeting #164	S2-2407803

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1136 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General inference procedure for NWDAF initiated Vertical Federated Learning between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims for specify the general inference procedure for vertical federated learning between AF(s) and NWDAF(s) with the NWDAF as the VFL server. The consumer requests the VFL inference result to the NWDAF acting as VFL server. The VFL server NWDAF performs the VFL inference with the NWDAF(s) and AF(s) as VFL client, and sends the analytics output to the consumer. Based on the conclusion captured in the TR 23.700-84 clause 8.2 P#2.5.5, this CR also specifies the main procedure of performing VFL model performance monitoring. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X.Y is added to specify the general inference procedure for vertical federated learning between AF(s) and NWDAF(s) with NWDAF as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL inference procedure not specified with the NWDAF acting as VFL server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

6.2X.Y General inference procedure for NWDAF initiated Vertical Federated Learning between NWDAF(s) and AF(s)

Figure 6.2X.Y-1: General inference procedure for VFL between NWDAF(s) and AF(s)

1.	The first step is the same as the existing one captured in clause 6.4.4 since the NF consumer doesn't need to be aware whether the NWDAF performs VFL or normal analytics for the requested analytics ID. Consumer NF sends an Analytics request/subscribe (i.e. Analytics ID = Service Experience, Target of Analytics Reporting = UE IDs, Analytics Filter Information that may include one or more of the following as defined in Table 6.4.1-1 (Application ID, S-NSSAI, DNN, Application Server Address(es), Area of Interest, RAT type(s), Frequency value(s)), Analytics Reporting Information=Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2.	Based on the requested analytics ID, the NWDAF VFL server determines to perform the VFL inference with the NWDAF and AF as VFL client to generate a more accurate analytics result.

NOTE 1:	The NWDAF acts as VFL server could only gather the intermediate inference results but not perform local model inference.

3.	VFL server NWDAF sends the VFL inference request to the VFL client NWDAF including the analytics ID received in step 1, internal UE IDs that are used by the VFL server NWDAF as the data sample for the inference, the feature profile to indicate what are the feature used by the NWDAF VFL client to generate the intermediate inference result, the VFL model correlation ID to indicate which VFL model should be used by the NWDAF VFL client to generate the other part of the intermediate inference result, and an optional maximum response time of local inference. NWDAF VFL client based on the local data and model to generate the intermediate inference result and send the VFL inference response to the VFL server NWDAF including the local inference result and the VFL model correlation ID.

4.	VFL server NWDAF sends the VFL inference request to the NEF including the internal UE IDs that are used by the NWDAF as the data sample for the inference, the feature profile to indicate what are the feature used by the VFL server NWDAF to generate the intermediate inference result, the VFL model correlation ID to indicate which VFL model should be used by the AF to generate the other part of the intermedia inference result, and an optional maximum response time of local inference.

5.	NEF maps the internal UE IDs to the external UE IDs.

6.	NEF sends the VFL inference request to the AF including the external UE IDs, the feature profile and the VFL model correlation ID.

7.	Based on the external UE IDs and the feature profile received from NEF, AF identifies the related sample and feature that will be used as the local dataset. Based on the VFL model correlation ID, AF chooses the local model to generate the local inference result.

8.	AF sends the VFL inference response to the NEF including the local inference result and the VFL model correlation ID. The AF shall send the response before maximum response time elapse.

9.	NEF sends the VFL inference response to the VFL server NWDAF including the local inference result and the VFL model correlation ID.

10.	Based on the VFL model correlation, VFL server NWDAF gathers the inference result from the AF and from VFL client NWDAF to generate the final inference result (i.e. observed Service Experience).

11.	The VFL server NWDAF provides the data analytics, i.e. the observed Service Experience to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

When performing VFL model performance monitoring, inference data can be used for model retraining. The main procedures are as follows:

During the model inference process, the VFL server allocates model inference ID, which is sent to the VFL client(s). After the model inference is completed, the VFL server saves model inference information, e.g., ground truth, the label, local data for Inference, model ID and model inference ID into ADRF, while VFL client(s) save model inference information e.g., the label, local data for Inference, model ID and model inference ID into ADRF.

VFL server may evaluate which inference data should be used for ML Model retraining (e.g., based on the ground truths and the inference results). When it determines that the inference data of an inference session should be kept for model retaining, it will keep the corresponding inference ID in an inference ID list.

When VFL server determines that the VFL model needs to be re-trained, the VFL server sends the inference ID or inference ID list to the VFL client(s). VFL server and VFL client(s) may retrieve the data from the ADRF based on inference ID or inference IDs in the list.

Based on the model inference information acquired from ADRF, VFL server initiates the re-training of the VFL model.

# >>>>END OF CHANGES<<<<