3GPP TSG-WG SA2 Meeting #164	S2-2407807

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 1139 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Performance monitoring for AI/ML based positioning in TS23.273 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions have been reached for KI#1 "Enhancements to LCS to support Direct AI/ML based Positioning” in clause 8.1 of TR 23.700-84: Principle #4: LMF or MTLF performs model performance monitoring for AI/ML based Positioning. - Principle #4.2 The result of model performance monitoring may trigger ML model retraining in the training entity. Based on the above conclusions, the procedures that MTLF-based model performance monitoring for AI/ML positioning are proposed, and clarify that performance monitoring is aligned with TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures that MTLF-based model performance monitoring for AI/ML positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures of performance monitoring for AI/ML based positioning is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *(All new text)

## 6.X	Performance monitoring for AI/ML based positioning

This clause specifies procedures for performance monitoring for AI/ML based positioning.

According to the definition of TS 23.288 [37], an NWDAF may have the accuracy checking capability for Analytics and/or ML Models. The NWDAF may provide the accuracy information to consumers when requested or use it for its internal processes.

Input data is collected from Data Producer NF(s) when there is a request for inference/prediction per analytics ID in NWDAF for a specific time period in future. Ground truth data are collected from those Data Producer NF corresponding to the requested analytic ID at the time to which the prediction refers.

The ground truth data is the actual measured data observed at the time which the prediction refers to.

NOTE 1:	The ground truth data can be impacted when Analytics Feedback Information shows that an action is triggered by the analytics output in the consumer.

The NWDAF (containing AnLF/MTLF) with accuracy checking capability decides to initiate Analytics Accuracy Monitoring based on:

-	A request from an analytics accuracy consumer. The analytics accuracy consumer may be an NWDAF containing AnLF, NWDAF containing MTLF or an analytics consumer NF.

-	Analytics Feedback Information which may be provided by an Analytics Consumer NF.

The MTLF with ML Model accuracy checking capability as defined in clause 6.2E in TS 23.288[37], is able to determine ML Model degradation based on e.g.:

-	comparing/evaluating the data: including input data, analytics output and the ground truth data either collected from various data source NFs, DCCF, AnLF, ADRF or configured by OAM;

-	or AnLF providing notifications of the Analytics Accuracy Information; or

-	AnLF providing Analytics Feedback Information of the analytics generated by the ML Model.

The NWDAF containing MTLF may reselect a new ML Model or retrain the existing ML Model and consequently notify the ML Model accuracy degradation to the ML Model consumer(s). In addition, the NWDAF containing MTLF may consider the rating of untrusted AF(s) when used as data sources.

6.X.1	MTLF-based model performance monitoring for AI/ML positioning

According to the definition of TS 23.288 [37], an NWDAF containing MTLF has the accuracy checking capability for Analytics and/or ML Models.

The LMF (may be co-located with NWDAF containing AnLF for model training/model inference) may perform the AI/ML positioning inference and provide it to consumers when requested by consumer. The LMF may provide inference data to MTLF for model monitoring. The MTLF performs AI/ML positioning model monitoring based on requests from LMF.

The MTLF may collect UE location information from UE, RAN, AMF, GMLC and AF. MTLF trains ML Model and monitors AI/ML positioning Model performance based on newly collected UE data and re-trains or re-provisions the existing AI/ML positioning Model.

Figure 6.X.1-1: Procedure of MTLF-based model performance monitoring for AI/ML positioning

1.	The AF/Consumer NF subscribes to the LMF to request AI/ML positioning.

2.	The LMF performs UE location data collection for AI/ML positioning.

3.	The LMF uses the trained ML Model deriving inference result.

4.	The LMF notifies AI/ML positioning inference to the AF/Consumer NF, and may send the inference to MTLF for monitoring model performance. At the same time, the LMF may provide an AI/ML positioning model performance monitoring threshold which is used as an indicator to execute the model performance report and/or re-training/re-provisioning of ML Model.

The performance monitoring information may including: ML Model monitoring reporting mode, ML Model Accuracy Threshold, ML Model identifier, etc. The detailed description is specified in TS 23.288 [37].

5.	The MTLF performs UE location data collection for monitoring model performance of AI/ML positioning inference.

NOTE:	UE location data collection as ground truth data, e.g. via application layer.

6.	The MTLF obtains model performance based on the collected UE location data and may decide to re-train/re-provision the ML Model for LMF.

7.	The MTLF provides a model performance report to LMF.

When the newly generated or re-trained ML Model is ready, the MTLF sends a new or re-trained ML Model to the LMF.

* * * End of Changes * * * *