3GPP TSG-WG SA2 Meeting #164	S2-2407808

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0542 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for LMF to retrieve ML models in TS23.273 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions have been reached for KI#1 "Enhancements to LCS to support Direct AI/ML based Positioning” in clause 8.1 of TR 23.700-84: Principle #3: MTLF is enhanced to perform model training for AI/ML based Positioning. - Principle #3.1: To retrieve a model to perform location calculation, the LMF discovers a suitable MTLF via NRF. Based on the above conclusions, the procedure that LMF retrieves ML models for AI/ML positioning are proposed in TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedure that LMF retrieves ML models for AI/ML positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of LMF retrieves ML models for AI/ML based positioning is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new) , 6.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *(All new text)

## 6.X	Procedure for LMF to retrieve ML models

### 6.X.1	General

This clause presents the procedure for LMF retrieves ML models.

LMF may be locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML Models or may use the NWDAF discovery procedure specified in clause 5.2 as defined in TS 23.288[37] for discovering NWDAFs containing MTLF. An NWDAF containing MTLF may determine that further training for an existing ML Model is needed when it receives the ML Model subscription or the ML Model request.

The NWDAF containing MTLF determines to train a ML Model either based on the request from LMF, or based on local configuration.

### 6.X.2	LMF Subscribe/Unsubscribe ML Models

The procedure in Figure 6.X.2-1 is used by LMF to subscribe/unsubscribe NWDAF containing MTLF, to be notified when ML Model Information related to the ML Model subscription becomes available, using Nnwdaf_MLModelProvision services. The ML Model Information is used by LMF to derive AI/ML positioning. The service is also used by an NWDAF to modify existing ML Model Subscription(s).

Figure 6.X.2-1: LMF Subscribe/Unsubscribe ML Models

1.	LMF subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the LMF are listed in clause 6.2A.2 as defined in TS 23.288[37]. LMF optionally indicates its support for multiple ML Models if available.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF), UE Application (via AF), to generate the ML Model.

If the service invocation is for a subscription modification or subscription cancelation, LMF includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If LMF subscribes to a (set of) trained ML Model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies LMF by invoking Nnwdaf_MLModelProvision_Notify service operation with:

-	For each Analytics ID requested by the service consumer, a set of pair(s) of unique ML Model identifier and the associated Information.

The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2 as defined in TS 23.288[37].

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify an available re-trained ML Model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML Model different to the previously provided one, or a re-trained ML Model by invoking Nnwdaf_MLModelProvision_Notify service operation.

When the content includes ML Model provide indicator, LMF may triggers ML Model retrieval procedure to retrieve ML Model(s).

* * * End of Changes * * * *