3GPP TSG-SA WG2#164	S2-2407812

Aug 19th – Aug 23th , Maastricht, NL

Source:	China Mobile, NEC, ZTE

Title:	Update architectural assumptions – standalone Ambient IoT core network

Document for:	Approval

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This contribution suggests adding one note about the standalone ambient IoT core network into the architectural assumptions section due to security concerns.

# 1.	Text proposal

It is proposed to agree the following changes to TR 23.700-13:

# >>>>BEGINNING OF CHANGES<<<<

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumptions

-	The following traffic types for Ambient IoT Device are to be studied:

-	DT: Device-terminated; and

-	DO-DTT: Device-originated - device-terminated triggered.

NOTE 1:	The DO-DTT additionally includes traffic from AIoT Devices, which is triggered by RAN/UE as reader, without CN sending traffic towards the AIoT Devices.

NOTE 2:	The final decision for including DO-A (Device-originated - autonomous) in the study depends on RAN decision.

-	The following two connectivity topologies as defined in TR 38.848 [7] are to be studied:

-	Topology 1: BS <--> Ambient IoT Device;

-	Topology 2: BS <--> intermediate node <--> Ambient IoT Device: Only a UE can act as an intermediate node which is under the network control.

-	The communication spectrum is assumed to be licensed.

-	Handover is not supported.

-	RRC states are not supported by AIoT Devices (see TR 38.769 [8])

-	No mobility (i.e. at least no cell selection/re-selection-like function) supported by AIoT Devices (see TR 38.769 [8])

Editor's note:	The meaning of no mobility is to be clarified by RAN in TR 38.769 [8].

NOTE 3: 	Coordination with RAN is required to determine the Ambient IoT Device capabilities in relation to system level of functionality (considering e.g. traffic scenarios, connectivity topologies etc.).

NOTE 4:	The security aspects for Ambient IoT requires coordination with SA WG3.

NOTE 5:	The charging aspects for Ambient IoT will be studied by SA WG5.

NOTE 6:	The NAS based Congestion control is not in the scope of this study.

NOTE 7:  A standalone Ambient IoT network shall be deployed by the operator.

## 4.2	Architectural Requirements

The following architectural requirements are applicable to this study:

-	Support for AIoT Services needs to adhere to the nature of the AIoT Devices (e.g. ultra-low complexity, power, cost and resource-constrained).

-	Support of the security aspects needs to consider the nature of the AIoT Devices (e.g. ultra-low complexity power, cost and resource-constrained) while addressing e.g. confidentiality, integrity, etc.

# >>>>END OF CHANGES<<<<