3GPP SA WG2 Meeting #164 	S2-2407816

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

Source:	China Mobile

Title:	KI#2 Evaluation and Conclusion proposal on Identification Management

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT /Rel-19

Abstract of the contribution: This contribution proposes an evaluation and conclusion for KI#2 to format the identifier of Ambient IoT Device or group of devices.

1. Background

This paper proposes an evaluation on the key aspects of the Identification management and identifier format discussed in solutions that have been adopted in the TR for Key Issue 2. And proposal to add some high level conclusions on KI#2 with the aim to identify those aspects that require normative work in Rel-19.

2. Text Proposal

It is proposed to adopt the following text within the TR 23.700-13.

# * * * * First change (all new text)* * * *

7.X	Identification Management

For Identification management and identifier format in Key Issue #2 " Identification, Subscription, Registration and Connection management", there are solutions mainly as follows:

Solution #1 proposes an AIoT Temporary Identifier to avoid sending the permanent subscriber identifier in clear text over Uu interface. Every time the TempID has been sent over the radio interface and a response from the AIoT device is sent, both CN NF and AIoT device locally generate a new TempID.

Solution #2 formats the identifier of Ambient IoT Device in order to identify specific Ambient IoT Device or a group of Ambient IoT Devices. The Ambient IoT Device ID is a permanent identifier used by the MNO to derive subscription-like data. The Home Network Identifier, Owner Identifier and Instance Identifier are considered necessary to compose the Ambient IoT Device ID, which is defined by 3GPP.

Solution #3 proposes that AIoT Devices are assumed to be pre-provisioned with a Device ID and the EPC defined by GS1 in the EPC Tag Data Standard can be used as the Device ID.

Solution #6 formats a device individual identifier and a group/service identifier. The device ID may be in the format of <hardwareID@MNO-ID> or <hardwareID@manufacturer-ID>. The group IDs may be in the format of <applicaitonID@SP-ID> wherein the SP-ID is the service provide ID or AIoT AF/AS ID.

Solution #8 proposes that the device information could be device ID, device group ID, and/or device type. The device ID allocated by CN is similar as 5G-GUTI.

Solution #9 proposes to allocate a temporary ID to the AIoT Device during the registration procedure.

Solution #10 introduces a new device ID which can be treated as SUPI. The device ID is either allocated by 5GC or the third party. The format of the device ID is Operator ID + Group ID + Instance ID.

Solution #11 proposes that AIoT device ID is defined by the external application. The uniqueness of the AIoT device ID per application is assumed to be guaranteed by the application itself, such as by adhering to standards like EPC (Electronic Product Code) and utilizing EPC global allocation, which manages a global registry for unique prefixes among different companies.

Solution #12 proposes that the AIoT device is able to identify itself, e.g. using its Device Identifier. In addition, a temporary UE identifier may be allocated, e.g. a temporary SUPI, associated credentials, and other necessary configuration and policies (e.g. URSP policy).

Solution #13 proposes that a Permanent ID is expected to enable uniquely identifying Ambient IoT Device by the 3GPP system. Such ID can entail MNO ID, Enterprise (Owner) ID, Instance ID. However, due to its long length, the Permanent ID is expected to be mapped to short temporary IDs across different entities / procedures.

Solution #14 proposes that AIoT Device ID should be used to identify the AIoT Device. The AIoT Device ID and its associated credential can be managed by network (PLMN network operator) or the 3rd party. In the former case, the AIoT Device ID has the information to identify the network and the format can be designed to be “PLMN Network information+AIoT Device Identification Number”. The AIoT Device ID can also be in NAI like format <username> @ operator where the <username> part is assigned by the operator and the operator info can be used to identify the operator. In this format, the <username> part is required to be unique @operator. In the latter case, the AIoT Device ID has the information to identify the 3rd party (service provider or other 3rd party e.g. AIoT Device manufacturer). The format can be NAI like format<username> @ 3rd party (SP or and other 3rd party e.g. AIoT Device manufacturer). The username part is assigned by the 3rd party.

Solution #16 revolves three types of device ID, the device owner ID, the concealed ID and the temporary ID. The AIoT device responds with a Registration Request that includes e.g. device owner ID and concealed device ID. During the registration procedure, the CN NF will provision the AIoT device with e.g. initial TempID or parameters to derive the initial Temp ID i.e. parameters for the Temp ID generation algorithm as discussed in Solution#1.

Solution #17 proposes that an AIoT Device ID is allocated to each AIoT device that has subscription-like information provisioned in the UDM. The AIoT Device ID shall be unique within a mobile network, and it is used to locate the subscription-like information.

Solution #18 proposes that the Ambient IoT device has a unique ambient device ID which consists of MNO operator info, Company info, and instance ID. Device ID will not be exposed to the third Ambient IoT AF.

Solution #22 proposes that the device information could be device ID, device group ID, and/or device type. The device type refers to type 1, 2A or 2B.

Solution #27 assumes that the AIoT Reader (UE or Base station) and AIoT device only send the AIoT ID (Temporary ID) once over the radio interface as described in solution #1. In case of Inventory service, the CN NF derives the permanent AIoT device ID of the registered AIoT device from the Temporary ID received by the AIoT device i.e. the CN NF can authenticate the Temporary ID belongs to the registered AIoT device.

Solution #29 assumes that each Ambient IoT device is identified with a unique internal ID (e.g. EPC ID). In the messages, a list of EPC IDs of AIoT devices is contained.

Solution #33 proposes that the AIoTCF authenticates the AIoT Device and assigns the type B AIoT Device temporary identifier (TempDevID) as part of the authentication procedure. The AIoT Device uses the temporary identifier in subsequent communication.

Solution #34 proposes that AIoT devices use a network-assigned virtual TMSI (VTMSI). If the device is authorized the AMF allocates a virtual temporary identifier (VTMSI) for each AIoT service and/or Service Provider ID wishes to provide assistance for.

In conclusion, there are 19 solutions related to identification management. Nine solutions format the identifier of an Ambient IoT Device or group of devices, and one solution (Solution #34) defines the identifier for an AIoT service and/or Service Provider. The identifier format of an Ambient IoT Device or group of devices is summarized for the key aspects of Identification Management in KI #2.

| Identifier Format of Ambient IoT Device |  |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | Device Identifier |  |  |  |
|  | Permanent Device Identifier | Temporary Device Identifier | EPC | Operator ID |
| 1 |  | √ |  |  |
| 2 | √ |  |  | √ |
| 3 |  |  | √ |  |
| 6 | √ |  |  |  |
| 10 | √ |  |  | √ |
| 11 |  |  | √ |  |
| 13 | √ | √ |  | √ |
| 14 | √ |  |  | √ |
| 18 | √ |  | √ | √ |

Table 7.X-1 compares the candidate Identifier Format of Ambient IoT Device

Solution #2, Solution #6, Solution #10, Solution #13, Solution #14, and Solution #18 define the format of the permanent Device ID. In Solution #2, Solution #10, Solution 13, and Solution #18, the Device Identifier consists of Operator ID, Owner ID or Enterprise ID, and Instance ID. In Solution #14, the format of Device ID is up to the person who manages the device. If the devices are managed by the network operator, the Device Identifier is defined as “PLMN Network information+AIoT Device Identification Number”or <username> @ operator. If the 3rd party manages the devices, the Device Identifier can be designed as <username> @ 3rd party. If the multiple operators’ ambient IoT device access and roaming service has to be supported, at least operator ID information should be stored in the ambient IoT device. Thus, the 5GC can distinguish the owner of the ambient IoT device based on operator ID info. If there is no Operator information in the ambient IoT Device, the operator network can’t determine the location of the credential holder if other operators provide ambient IoT devices.

Solution #1 and Solution #13 propose allocating the Temporary ID to the device. However, it is tough for ambient IoT devices to acquire and store the Temporary ID due to their minimal computing capability and power. In addition, writing the Temporary ID into devices via the network will significantly impact the service coverage range under the same signal strength. Furthermore, it will increase the cost of ambient IoT devices because of introducing additional multiple read and write RAM. Pure read operators are preferred in the Identification management procedure.

Solution #3, Solution #11, and Solution 18 propose to use the EPC as the Device ID to represent a specific ambient IoT device or a group of ambient IoT devices. Maybe leveraging the existing GS1 EPC is a potential method to address the issue of how to flexibly and efficiently identify an ambient IOT device or a group of IoT devices and ensure the global uniqueness of the device ID. However, considering the competent relationship with RFID, and in many application scenarios, the customers don’t need the global uniqueness of device ID; maybe introducing the operator's device ID with a similar format is another potential method, too.

# * * * * Next change (all new text) * * * *

8.X	Identification management and identifier format

Based on the evaluation of solutions, the following conclusions are preferred:

- The 5GS should support a permanent Device Identifier and read only operations are preferred in the Identification management procedure.

- The permanent Device Identifier can flexibly and efficiently identify a specific Ambient IoT Device or a group of Ambient IoT Devices, such as GS1-owned EPC or operator-owned identifier with EPC similar format

- Operator ID information should be contained in the ambient IoT device. Based on this information, 5GC can perform ambient IoT device management, such as determining the credential holder, supporting roaming service, etc.

# * * * * End of changes * * * *