3GPP TSG-WG2 Meeting #164	S2-2407818

# Maastricht, Netherlands, 19 August – 23 August, 2024                   (revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0123 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support assistance of pre-mission flight planning and in-mission flight monitoring for UAV |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR intends to support assistance of pre-mission flight planning and in-mission flight monitoring for UAV based on the conclusion of clause 8.1.1 in TR 23.700-59. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions and procedures for supporting assistance of pre-mission flight planning and in-mission flight monitoring for UAV. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new functionality of assistance of pre-mission flight planning and in-mission flight monitoring for UAV can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

5.X

5.x.1	Functional Description

3GPP network supports the functionality of assistance of pre-mission flight planning and in-mission flight monitoring for UAVs and service exposure towards USS/UTM.

5.x.2	Procedures

5.x.2.1	Procedure for pre-mission Flight Planning with assistance of 5GC

Figure 5.x.2.1-1: Pre-mission Flight Planning with assistance of 5GC

1.	The UAV establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3.

2.	[Optional] The UAV requests Pre-mission flight planning service from USS/UTM. The request message includes the 3GPP UAV ID (e.g. GPSI, CAA-Level UAV ID), information of the flight path (e.g. the 3D location waypoints at specific time interval), and candidate flight path(s) (if available).

3.	[Optional] The USS/UTM derives information for the pre-mission flight planning assistance request based on step 2.

NOTE 1:	How USS/UTM derives information for the pre-mission flight planning assistance information is out of scope of the 3GPP.

4.	The USS/UTM sends pre-mission flight planning assist request to NEF. The request message includes 3GPP UAV ID (e.g. GPSI), information of the flight path (e.g. the 3D location waypoints at specific time interval), candidate flight path(s) (if available), and accuracy level of predictions relevant to the flight planning, QoS requirements.

5.	The NEF maps the geographical area/waypoints into 3GPP identifiers (e.g. a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (e.g. UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (e.g. Movement Behaviour analytics, QoS Sustainability Analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	[Optional] If the NEF cannot satisfy the AF provided requirements in step 4, the NEF sends the Pre-mission flight planning assistance response to reject the request from USS/UTM.

7.	[Optional] If the Pre-mission flight planning request is rejected in step 6, the USS/UTM may also provide a Pre-mission flight planning response to reject the request from UAV.

8.	[Optional] The NEF subscribes/requests Movement Behaviour analytics provided by NWDAF as specified in clause 6.21 of TS 23.288 [x]. The subscribe/request message includes the 3GPP UAV ID (e.g. GPSI) received in step 4, and the Area of Interest based on the 3GPP identifiers determined in step 5. And the other parameters included in the request are specified in clause 6.21.1 of TS 23.288 [x].

9.   [Optional] The NEF subscribes/requests QoS Sustainability Analytics provided by NWDAF as specified in clause 6.9 of TS 23.288 [x]. The subscribe/request message includes the 3GPP UAV ID (e.g. GPSI) and QoS requirements received in step 4, and location information (e.g. the flight path defined as 3D location waypoints, or list of TAIs or Cell IDs). And the other parameters included in request are specified in clause 6.9.1 of TS 23.288 [x].

10.	[Optional] The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as specified in clause 6.20.3 of TS 23.273 [x].

11.	The NEF generates assistance information for pre-mission flight planning based on the information and analytics from steps 8-10. The assistance information may include the best matched route among the ones provided from the USS/UTM, or the QoS can’t be satisfied at specific location (e.g. waypoints/geographical area) and the time interval that cannot be used for a flight path.

12.	The NEF responses to the USS/UTM with the assistance information.

13.	The USS/UTM determines flight planning by considering the assistance information.

NOTE 2:	How USS/UTM determines flight planning by considering the assistance information is out of scope of the 3GPP.

14.	The USS/UTM sends response to the UAV with the planned flight path.

5.x.2.2	Procedure for in-mission Flight Monitoring with assistance of 5GC

Figure 5.x.2.2-1: In-mission Flight Monitoring with assistance of 5GC

1.	The USS/UTM sends In-mission flight monitoring assistance request to NEF. The request message includes the 3GPP UAV ID (e.g. GPSI) and information of the flight path (e.g. the 3D location waypoints at specific time interval), monitoring mode (e.g. UAV location monitoring, flight path monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring), report requirements (e.g. event triggered or periodically), QoS monitoring requirement.

2.	The NEF maps the geographical area/waypoints into an area of interest that is represented by 3GPP identifiers (e.g. a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location), and NWDAF analytics service (e.g. QoS Sustainability Analytics).

3.	[Optional] If the NEF cannot provide the assistance information as requested in step 1, the NEF sends In-mission flight monitoring assistance response to reject the request from the USS/UTM.

4.	[Optional] For UAV location monitoring, the NEF performs steps 2-5 in clause 5.3.2 to obtain the location of the target UAV.

5.	[Optional] For flight path monitoring, the NEF performs steps 2-4 in clause 5.3.3. The NEF may compare the monitored flight path with the planned flight path (if provided in step 1) and generate monitoring result on whether the UAV is flight in right path.

6.	[Optional] For flight environment monitoring, the NEF executes steps 2-5 in clause 5.3.4 to obtain the information of aerial UEs in the same geographic area of the target UAV.

7.	[Optional] For Ranging/Sidelink positioning location monitoring, the NEF requests GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [x], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation the monitoring, steps 21-31 for monitoring periodic). The list of UAVs in the areas of interest output from step 6 can be used as inputs to step 7 for NEF to request GLMC service for Ranging/Sidelink Positioning location, if the UAVs is unknown in the areas of interest.

8.	[Optional] For QoS Sustainability Analytics, the NEF subscribes/requests QoS Sustainability Analytics provided by NWDAF as specified in clause 6.9 of TS 23.288 [x]. The subscribe/request message includes the 3GPP UAV ID (e.g. GPSI), location information (e.g. the flight path defined as 3D location waypoints), and QoS requirements received in step 1. And the other parameters included in request are in clause 6.9.1 of TS 23.288 [x].

9.	The NEF notifies the USS/UTM the monitoring results (e.g. the location of the target UAV, indication on whether the UAV is flight in right path, the target UAV presence in areas of interest, QoS monitoring result at specific location).

>>>> End of Changes <<<<