3GPP TSG-WG2 Meeting #164	S2-2407821

# Maastricht, Netherlands, 19 August – 23 August, 2024                   (revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0237 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EAS and L-UPF (re)selection considering N6 delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_ph3 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR intends to support the enhancement of EAS and local PSA UPF (re)selection based on the conclusion of clause 8.2 in TR 23.700-49, and address the three NOTEs in the conclusion. - For the following NOTE1: “NOTE 1: For the enabling of SMF to measure N6 delay aspect, whether one mechanism between AF influence and EDI is enough will be determined in normative phase.” It is concluded that AF can provide an indication of considering N6 delay, a list of DNAI(s), measurement endpoint, the measurement protocol(s) and optionally the protocol-specific configuration parameters. - For “Indication of considering N6 delay”, it is per application and more suitable to be provided via AF influence procedure. - For “measurement endpoint”, AF may provide different measument endpoints for different applications, instead of providing all the possible measurement endpoints deployed in the Local part of the DN. So this parameter is also per application. For “measurement protocol(s) and protocol-specific configuration parameters”, it is corresponding to specific measurement endpoint. So they are more suitable to be provided via AF influence procedure. - For “a list of DNAI(s)”, the existing parameter Potential Locations of Applications could be reused. Proposal: The AF influence mechanism is more suitable and enough to provide the parameters for enabling SMF to measure N6 delay. - For the following NOTE2: “NOTE 2: For AF-provided delay requirement, it will be determined in normative phase whether a new information element (e.g., end-to-end delay requirement) is needed or the existing User Plane latency requirement can be reused.” The existing User Plane latency is used to indicate the latency between UE and potential PSA-UPF, while the new parameter end-to-end delay requirement is used for AF to request the delay requirement between UE and endpoint at application side (e.g. EAS IP address). So the User Plane latency can’t be reused to represent end-to-end delay requirement. Proposal: Introduce a new information element “End-to-End delay requirement” to indicate the delay between UE and endpoint at application side (e.g. EAS IP address(es)/Designated IP (range)) should be lower than specific threshold. - For the following NOTE3: “NOTE 3: Whether SMF considers N6 delay for EAS selection will be determined in normative phase.” In current approach, since the AF should provide all the measurement endpoint(s) corresponding to the application(s) to 5GC. The L-PSA UPF needs to measure the N6 delay between all pairs of measurement endpoint (EAS) and L-PSA UPF, which leads to large overhead and waste of resource. If the SMF determines the target measurement endpoint (EAS) based on the EAS information received in DNS response, the L-PSA UPF could reduce many measurements, which avoid the issues above. Since the N6 delay measurement is between the pair of L-PSA UPF and EAS, to guarantee the delay requirement is met, the SMF should consider N6 delay for EAS selection based on the measured N6 delay. Proposal: SMF may determine the measurement endpoint (e.g. EAS IP address(es)) based on the EAS information provided in DNS response, and considers N6 delay for EAS selection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add general description for EAS discovery with EASDF and L-UPF selection considering N6 delay. - Add new procedure for EAS discovery with EASDF and L-UPF selection considering N6 delay. - Update EAS rediscovery procedure for considering N6 delay based EAS rediscovery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new functionality of enhancement of EAS and local PSA UPF (re)selection can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.1, 6.2.3.2.x(new), 6.2.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR S2-2407822 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.502 CR S2-2407823 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> First change <<<<

6.2.3.2.1	General

For PDU Session with Session Breakout connectivity model, based on UE subscription (e.g. DNN) and/or the operator's configuration, the DNS Query sent by UE may be handled by an EASDF (see clause 6.2.3.2.2), or by a local or central DNS resolver/server (see clause 6.2.3.2.3).

NOTE 1:	For the scenario where the TE and MT are separated, information provided by the SMF in the NAS message during the PDU Session Establishment or Modification may not be provided to the TE. Annex C documents mitigations for this scenario.

NOTE 2:	The DNS Query sent by UE may or may not carry EDNS Client Subnet option in the DNS message.

The common EAS for a set of UEs can be provided by AF or determined by 5GC, if not provided by AF, with EAS discovery procedure via EASDF (see clause 6.2.3.2.5).

The common DNAI for a set of UEs can be provided by AF or determined by 5GC, if not provided by AF, with EAS discovery procedure via EASDF (see clause 6.2.3.2.6) or Local DNS resolver/server (see clause 6.2.3.2.4).

For the different procedures, when 5GC determines the common EAS/DNAI, the NEF determines the common EAS/DNAI with input from SMFs (see clause 6.2.3.2.7).

The EAS discovery with EASDF and local PSA UPF selection can be enhanced by considering N6 delay (see clasue 6.2.3.2.x).

>>>> Second change <<<<

6.2.3.2.x 	EAS discovery with EASDF and L-UPF selection considering N6 delay

The N6 delay between the local PSA UPF and EAS is a component of the End-to-End delay (i.e. delay between the UE and EAS), which is also an important factor for the delay sensitive application, and it should be taken into account for the local PSA UPF and EAS selection.

The following is the procedure for EAS discovery with EASDF and local PAS UPF selection considering N6 delay.

Figure 6.x.2-1: EAS discovery with EASDF and L-UPF selection considering N6 delay

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request EAS discovery based on N6 delay.

The following information may be included in AF request as defined in clause 5.6.7.1 of TS 23.501 [2]:

-	Indication of considering N6 delay: indicates N6 delay between the local PSA UPF and endpoint at application side (e.g. EAS IP address(es)/Designated IP (range)) should be taken into account for EAS and local PSA UPF (re)selection for the delay sensitive application.

-	End-to-End delay requirement: indicates the delay between UE and endpoint at application side (e.g. EAS IP address(es)/Designated IP (range)) should be lower than specific threshold.

-	Measurement endpoint: indicates the target EAS IP address(es)/Designated IP (range) per DNAI as the candidate enpoint(s) to be measured.

-	Measurement protocol: indicates the protocol(s) can be applied to the measurement endpoint for N6 delay measurement, and optionally with protocol-specific configuration parameters.

-	Potential Locations of Applications: indicates potential locations of applications, represented by a list of candidate DNAI(s).

In step 5 of figure 4.3.6.2-1 of TS 23.502 [3], the PCF creates PCC rule with Indication of considering N6 delay, End-to-End delay requirement, Measurement endpoint, Measurement protocol and DNAI(s), and then sends it to SMF.

2.	Based on the indication of considering N6 delay received in PCC rule, the SMF determines the N6 delay should be taken into consideration for EAS and local PSA UPF selection.

Based on the Measurement protocol received in PCC rule or pre-configuration in SMF, and the protocol supported by UPF, the SMF determines the measurement protocol.

3a. [Optional] If candidate DNAI(s) is provided in AF request, the SMF determines the candidate UPF(s) according to the candidate DNAI(s).

If the Measurement endpoint is provided in AF request, the SMF indicates the candidate UPF(s) to perform N6 delay measurement by configuring the N6 delay Reporting Rule, which includes the Measurement endpoint and determined measurement protocol.

3b. [Optional] Local PSA UPF(s) performs N6 delay measurements and reports the N6 delay measurement result to SMF as described in clause 5.8.2.x of TS 23.501 [2].

NOTE:	The N6 delay between local PSA UPF and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) is implementation specific. It can be measured based on existing mechanisms (e.g., ICMP, TWAMP, OWAMP or other protocols defined by IETF).

4.	The UE triggers PDU session establishment procedure and sends DNS query to EASDF, and EASDF retrieves the DNS response (including EAS information, e.g. one or more EAS IP addresses) from DNS server and notifies the DNS message reporting to SMF as shown in step 1-15 of figure 6.2.3.2.2-1.

5.	[Optional] If the steps 3a and 3b are not performed, based on EAS information received from the EASDF in Neasdf_DNSContext_Notify of step 3 and other UPF selection criteria, the SMF determines one or more candidate L-PSA UPF(s). The SMF also determines one or more candidate EAS(s) as included in EAS information. Then the steps 5a and 5b are performed.

5a. The SMF indicates the candidate UPF(s) to perform N6 delay measurement by configuring the N6 delay Reporting Rule, which includes the Measurement endpoint (i.e. candidate EAS(s) received in EAS information) and determined measurement protocol.

5b. Local PSA UPF(s) performs N6 delay measurements and reports the N6 delay measurement result to SMF as described in clause 5.8.2.x of TS 23.501 [2].

6.	The SMF selects a pair of Local PSA UPF and EAS based on N6 delay determined in step 3 or 5, optionally based on the end-to end delay requirement (if received in step 1).

7. The SMF may insert the selected Local PSA UPF.

8a.	The SMF sends the updated DNS message handling rule to EASDF via Neasdf_DNSContext_Update Request. The DNS message handling rule includes the selected EAS IP address.

8b.	The EASDF responds with Neasdf_DNSContext_Update Response.

9.	The EASDF sends the DNS Response with selected EAS IP address to the UE as indicated in the DNS message handling rule.

>>>> Third change <<<<

6.2.3.3	EAS Re-discovery Procedure at Edge Relocation

The support for EAS rediscovery indication procedure enables the UE to refresh stale EAS information stored locally so that the UE can trigger EAS discovery procedure to discover new EAS information.

For PDU Session with Session Breakout connectivity, the UE may indicate its support for refreshing stale EAS information to the SMF during the PDU Session Establishment procedure or, when the UE moves from EPS to 5GS for the first time, by using the PDU Session Modification procedure. If the UE indicates such support, the SMF may send to the UE the EAS rediscovery indication, with an optional impact field, so that the UE may trigger to re-discover the EAS (see the step 2 of Figure 6.2.3.3-1) after the insertion/change/removal of an L-PSA based on AF influence or its local configuration using the PDU Session Modification procedure, or based on the AF triggered EAS relocation.

This procedure is used by the SMF to trigger the EAS rediscovery procedure when a new connection to EAS need to be established. It applies to both Session Breakout using ULCL and Session Breakout using BP.

Figure 6.2.3.3-1: EAS re-discovery procedure at Edge relocation

During a previous EAS Discovery procedure on this PDU Session the UE may have EAS information (i.e. EAS IP address corresponding to an EAS FQDN) locally stored, e.g. acquired during the previous connection with the EAS (for more information see Annex C UE considerations for EAS (re)discovery).

1a.	Due to the UE mobility the SMF triggers L-PSA insertion, change or removal for the PDU Session. The insertion, change or removal of L-PSA triggers EAS rediscovery.

The L-PSA insertion, change or removal for the PDU Session may be triggered due to update of a common DNAI.

1b.	The AF triggers EAS relocation e.g. due to EAS load balance or maintenance, etc. and informs the SMF the related information indicating the EAS relocation, as described in clause 4.3.6 AF influence on traffic routing procedure in TS 23.502 [3].

AF may request to add/remove a UE to/from a set of UEs via AF influence on traffic routing procedure in TS 23.502 [3]. The PCF updates PCC rule to SMF (i.e. adding/removal of EAS Correlation indication/indication of traffic correlation and Traffic Correlation ID to/from the PCC rule). With the update of the PCC rule, the SMF detects that the UE is belonging to a set of UE(s) for a common EAS/DNAI, or the UE is not belonging to a set of UEs for common EAS/DNAI any longer and the common EAS/DNAI is not optimized for the UE, it may trigger EAS rediscovery.

If UE is belonging to a set of UE(s) for a common EAS/DNAI as instructed by the PCC rule, the SMF interacts with NEF for common EAS/DNAI selection as described in clauses 6.2.3.2.5 and 6.2.3.2.6.

1c.	Based on the N6 delay measurement reporting from local PSA UPF (e.g. SMF subscribes UPF to report the N6 delay measurement event per event triggered or periodically) or updated End-to-End delay requirement received in PCC rule, and SMF determines the N6 delay exceeds the specific threshold (e.g. determines based on the End-to-End delay requirement). The SMF determines to reselect the L-PSA and trigger EAS rediscovery.

2.	This step may be performed as part of step 1a/1b/1c. The SMF performs the network requested PDU Session Modification procedure from the step 3b-11b as defined in clause 4.3.3.2 TS 23.502 [3].

If the UE has indicated that it supports to refresh EAS information stored locally corresponding to the impact field per the EAS rediscovery indication from network, the SMF may send the impact field with the EAS rediscovery indication. SMF determines the impacted EAS(s) which need be rediscovered as the following:

-	If an L-PSA is inserted/relocated/removed, the SMF determines the impact field, which is associated with the L-DN to be inserted, relocated or removed and identified by FQDN(s) or IP address range(s) of the old EAS, based on the association between FQDN(s)/IP address range(s) and DNAI provided by AF or SMF local configuration on the L-DN.

-	For AF triggered EAS rediscovery, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by Application Identifier(s), to the SMF via the AF influence on traffic routing procedure.

The SMF sends PDU Session Modification Command (EAS rediscovery indication, [impact field]) to UE. The EAS rediscovery indication indicates to refresh the cached EAS information. The impact field is used to identify which EAS(s) information need to be refreshed. The impact field includes the L-DN information corresponding to the impacted EAS(s), which are identified by FQDN(s) or IP address range(s) of the old EAS(s). If the impact field is not included, it means all EAS(s) information associated with this PDU Session need to be refreshed.

The SMF may choose new DNS settings for the PDU Session and if so, it provides them to the UE as new DNS server (see Option C in clause 6.2.3.2.3). Otherwise the UE uses the existing DNS server for EAS rediscovery.

For the following connection with the EAS(s) for which the EAS rediscovery needs to be executed per the received EAS rediscovery indication and impact field, the UE has been instructed not to use the old EAS information stored locally. Instead it should trigger EAS discovery procedure to get new EAS information as defined in clause 6.2.3.2.

For the Split-UE, it is not possible to provide the NAS level EAS rediscovery indication and the impact field to the TE. Annex C documents mitigations for this scenario.

NOTE 1:	In case of EAS IP Replacement (see 6.3.3.1) the support for EAS rediscovery indication procedure is not required.

NOTE 2:	Depending on the UE implementation, the EAS rediscovery indication triggers an EAS Rediscovery procedure. If the EAS rediscovery indication is not sent to the UE Application Layer or to the UE OS, then the DNS Query to discover a new EAS is triggered only if the IP flows are terminated or via application/OS implementation means, e.g. based on application redirection, other application server information or DNS cache time-to-live. If DNS cache has not expired in the Application Layer or the OS, the triggered re-discovery can lead to the old EAS. For more information see Annex C.

NOTE 3:	The active connection(s) between the UE and the EAS(s) are not impacted.

>>>> End of Changes <<<<