3GPP SA WG2 Meeting #164	 S2-2407835

# Maastricht, NL, 19-23 August 2024											                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5464 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User identifiers and subscription aspects in TS23.501 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the conclusion of KI#1 in TR 23.700-32, the format of user identifier and the information about user identity profile are agreed. In addition, in order to provide differentiated service for different user identifiers, the differentiated QoS parameters, e.g., DNN, S-NSSAI, 5QI, should be needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It’s proposed to define the format of the user identifiers and introduce the enhancement about subscription aspects. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The differentiated user identifier service can not be achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y.(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

5.X.Y	User identifiers and subscription aspects

The User Identifier takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in clause 28.7 of TS 23.003 [19]. The realm part of the NAI may include the PLMN ID.

The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

The UDM stores the subscribed User Identifiers information in the User Identity Profile, which may contain:

-	SUPI of linked subscribed user;

-	one or more User Identifiers;

-	optionally, differentiated QoS parameters of specific User Identifier, which may contain DNN, S-NSSAI, 5QI.

# >>>>END OF CHANGES<<<<