3GPP TSG-SA2 Meeting #164	S2-2407859

# Maastricht, Netherlands, 19 - 23 August 2024	       		          (revision of S2-240)

Title:	[draft] LS on XRM metadata exchange between 3GPP Core and an Application server

Response to:

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	Nokia, Will be SA2

To:	IETF Multiplexed Application Substrate over QUIC Encryption WG chairs: Dennis Jackson, Eric Kinnear.

IETF Media Over QUIC WG chairs: Alan Frindell, Martin Duke

Cc:	liaison-coordination@iab.org, 3gpp-liaison@ietf.org.

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2407857 (discussion document)

- Overall Description:

Background information

3GPP SA2 is working to provide dedicated Quality of Service (QoS) for XRM (eXtended Reality) traffic flows over 3GPP networks (5G); for this purpose, a 3GPP network needs inband (along with the packets) certain metadata about the structure and characteristics of the application media content in the traffic flows. The following metadata has been defined in 3GPP Release 18:

PDU Set: One or more PDUs (for IP based traffic a PDU is an IP packet) carrying the payload of one unit of information generated at the application level (e.g. frame(s) or video slice(s) etc. for eXtended Reality (XR) Services).

End PDU of the PDU Set

PDU Set Importance [PSI]

PDU Set Sequence Number [PSSN]

PDU Sequence Number within a PDU Set [PSN]

PDU Set Size [PSSize]

End of Data Burst [EDB] ( A set of multiple PDUs generated and sent by the application in a short period of time)

It is also Important to note that the set of XRM metadata information to be made available to the 3GPP network (UPF) will be extended in the future (as part of 3GPP R19 and beyond e.g. to deal with media multiplexing in an UDP/IP flow).

For RTP based traffic PDU Set and End of Data Burst information has been defined by 3GPP to be carried by a new RTP header extension. The same can be followed for any extended metadata. When XRM traffic is carried encrypted end to end (between UE and XRM application server) in protocols which do not allow unencrypted header extensions, additional mechanisms need to be defined for 3GPP networks (UPF) to be able to access to this XRM metadata information while ensuring that actual XRM traffic remains protected end to end and that entities beyond the 3GPP network (UPF) cannot read or modify the XRM metadata information either.

Questions related with MASQUE

To support 5G Core UPF access to XRM metadata, 3GPP (TR 23.700-70) is considering to use Proxy-UDP-in-HTTP/3 per RFC 9298 + QUIC-Aware proxying per draft-ietf-masque-quic-proxy.

A possible deployment of the feature would be where the Client as defined by IETF in draft-ietf-masque-quic-proxy would be split up between UE including the application/QUIC client and UPF including the HTTP client using the CONNECT-UDP method towards the Proxy (in this deployment the UE would NOT initiate Proxy-UDP-in-HTTP/3 per RFC 9298 + QUIC-Aware proxying). UE and UPF would be connected by IP networking as specified by 3GPP with UE acting as an IP host and UPF as an IP Router. Metadata about the end-to-end (UE to Target) QUIC connection would be carried between UPF and Proxy. The arrangement between Proxy and Target (the QUIC server acting as the Application Server for the XRM traffic) is not in scope of 3GPP.

UPF

UE

For this kind of deployment following question have been asked that require detailed expertise on draft-ietf-masque-quic-proxy:

QUIC aware proxying relies on the Client and Proxy regenerating actual CID based on Virtual CID exchanged between the client (here UPF) and the Proxy.

QUIC allows new CID values to be negotiated between the UE (QUIC client) and the Target (QUIC Server) anytime during the lifetime of the end-to-end QUIC connection. When this happens, would the UPF be able to learn these values and to provide them to the Proxy over the HTTP Capsule protocol on behalf of the UE?

If not does it mean that forwarding mode is no more possible for the corresponding traffic when QUIC CID has been changed by one of the peers (UE/target =AS)?

Do you see other potential issues with the 3GPP usage of this kind of deployment?

Questions related with MoQ

To support 5G Core UPF access to XRM metadata, 3GPP (TR 23.700-70) is considering to use Media over QUIC Transport as defined in draft-ietf-moq-transport-03. The idea is to use the metadata part of the MoQ object to carry such XRM metadata information.

3GPP would thus like to know whether and when MoQ WG could fulfil the needs of 3GPP (for its Release 19 i.e. for end of 2024) such as described above and if the MoQ object metadata can be extended to carry such XRM metadata details beyond what is now defined in MoQT draft at IETF.

- Actions:

To IETF Multiplexed Application Substrate over QUIC Encryption WG and IETF Media Over QUIC WG:

ACTION: 	SA2 kindly asks IETF Multiplexed Application Substrate over QUIC Encryption WG and IETF Media Over QUIC WG to answer to the questions above.

- Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#165 | October 14th – 18th, 2024 | Hyderabad | India |
| SA2#166 | November 18th –22nd, 2024 | Orlando | USA |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/