3GPP TSG-SA WG2#164	S2-2407865

Maastricht, NL, 19-23 August 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Updating Solution 3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes updates to solution 3.

# 1.	Discussion

# 2.	Text proposal

This paper proposes the following changes to TR 23.700-13.

# >>>>BEGINNING OF CHANGES<<<<

## 6.3	Solution #3: Lightweight Ambient IoT system

### 6.3.1	Description

#### 6.3.1.1	Introduction

This solution proposes a lightweight Ambient IoT system.

#### 6.3.1.2	Definitions

-	Command: Refers to an instruction sent by an AF to an AIoT Device. The following instructions may be supported:

-	Read: Reading data from an AIoT Device;

-	Write: Writing data to an AIoT Device;

-	Disable: Disable an AIoT Device temporarily or permanently.

-	Command Response: Refers to the message sent by an AIoT Device in response to a Command. This may include an acknowledgement and optionally data (e.g. in case of the Read operation).

-	Enrichment Data: Additional information that a Reader may include when providing Inventory information or a Command Response to the AIoT Controller. Enrichment Data may include information about the signal strength for each detected Device ID and the location of the Reader (if known).

-	Filter Criteria: Criteria to limit an Inventory or Command to AIoT Devices that match certain criteria. Filter Criteria may consist of the following:

-	 an EPC as defined by GS1 in the EPC Tag Data Standard [10] and optionally a bitmask;

-	a Closest indication.

NOTE 1:	The Closest indication only applies to the Inventory procedure.

NOTE 2:	The term Filter Criteria can be revisited to avoid clashes with existing terminology.

-	Inventory: Refers to determining the identity of all or a subset of AIoT Devices in range of a reader.

#### 6.3.1.3	Assumptions

This solution makes the following assumptions:

-	Commands and Command Responses are end-to-end protected between AF and AIoT Device.

NOTE 1:	Details of the end-to-end protection of Commands and Command Results are assumed to be addressed by SA WG3.

-	Commands and Command Responses are transparent to the AIoT Controller and the Reader, i.e. the AIoT Controller and Reader are not aware of the contents of Commands and Command Responses.

-	AIoT Devices are assumed to be pre-provisioned with a Device ID and the security material for the end-to-end protection of Commands and Command Results. The EPC as defined by GS1 in the EPC Tag Data Standard [10] is used as the Device ID.

NOTE 2:	Whether also dynamic (re-)provisioning of Ambient IoT Devices can be supported is up to SA WG3.

-	The radio configuration (frequency bands, etc.) of the Reader is assumed to be configured through an OAM system, which is beyond the scope of SA WG2.

#### 6.3.1.4	Reference architecture

This solution proposes the reference architecture depicted in Figure 6.3.1.4-1 and Figure 6.3.1.4-2.

Figure 6.3.1.4-1: Ambient IoT system architecture for Topology 1

Figure 6.3.1.4-2: Ambient IoT system architecture for Topology 2

The Ambient IoT system can support different deployment options. For example, the Reader may be co-located with a 3GPP UE so that the communication between Reader and AIoT Controller uses a PDU Session to enable Topology 2 (enabling Topology 2 as depicted in Figure 6.3.1.4-2). Alternatively, Readers may be deployed independent of 3GPP UEs, i.e. as stand-alone base stations to enable Topology 1 (Figure 6.3.1.4-1).

The AIoT controller is assumed to be a 3GPP core network entity that enables Ambient IoT scenarios in the context of 5G. The 5GC network functions shown in Figure 6.3.1.4-2 (AMF, SMF, UPF) are assumed to be in the PLMN domain.

NOTE:	For Topology 2, the UE is assumed to be configured with the DNN/S-NSSAI for the PDU Session to be used for Reader to AIoT Controller communication and also with an FQDN to discover the AIoT Controller.

Editor's note:	Whether there is a need to validate the AIoT Device ID by a serving network and if so, how to achieve this is FFS.

#### 6.3.1.5	Network function description

##### 6.3.1.5.1	Reader

The Reader supports the following functionality:

-	Supports the A-Uu air interface towards Ambient IoT Devices.

-	Registers with an AIoT Controller.

-	Supports the following functionality based on requests from an AIoT Controller:

-	Perform one-time or periodic Inventory, deliver Inventory result to AIoT Controller.

-	Delivers Commands from an AIoT controller to an AIoT Device.

-	Delivers Command Responses received from an AIoT Device to an AIoT Controller.

##### 6.3.1.5.2	AIoT Controller

The AIoT Controller supports the following functionality:

-	Register Readers.

-	Authenticate and authorize AFs.

-	Based on requests from an AF:

-	Verify whether an AF is entitled to issue a specific Inventory Request.

-	Select Readers to fulfil Inventory or Command request by AFs.

-	Forward Inventory request to Readers and deliver Inventory result to AF.

-	Forward Command to Readers and Command Responses to AF.

-	Optionally collect usage data per AF, e.g. for charging purposes.

-	Store last known Reader information for AIoT Devices.

This solution does not use the NEF. Instead, to authenticate and authorize AFs and to protect the AIoT Controller - AF communication, the security mechanisms defined for the Network Exposure Function in TS 33.501 [15] (TLS for mutual authentication, integrity protection, replay protection and confidentiality protection, OAuth-based authorization) are assumed to also be used for the AIoT Controller. Details can be specified by SA WG3.

NOTE:	The details of usage data collection per AF are in the scope of SA WG5. It is up to SA5 to decide whether CHF functionality can be reused to achieve that.

The solution assumes that the CAPIF framework can also be used to register and discover APIs of the AIoT Controller.

##### 6.3.1.5.3	AF

The AF is assumed to support the following functionality:

-	Authenticate towards the AIoT Controller.

-	Send Inventory and Command Requests.

-	Receive Inventory Responses and Command Responses.

NOTE:	The AF is assumed to be preconfigured with the IP address or FQDN of the AIoT Controller.

#### 6.3.1.6	Protocol Stacks

##### 6.3.1.6.1	Protocol Stack between Reader and AIoT Controller

Legend:

-	Reader Application Protocol (R-AP): Application Layer Protocol between the Reader and the AIoT Controller.

-	Service-based Interface (SBI) Protocol Stack: The protocol stack for service-based interfaces as defined in TS 29.500 [9].

Figure 6.3.1.6.1-1: Control Plane between Reader and AIoT Controller for Topology 1

NOTE 1:	Whether an SBI protocol stack (consisting of IP/TCP/HTTP2/JSON) as defined in TS 29.500 [9] or an SCTP-based stack will be used will be decided in coordination with RAN wg3.

Figure 6.3.1.6.1-2: Control Plane between Reader and AIoT Controller for Topology 2 using a PDU Session between UE and UPF

NOTE 2:	Whether an SBI protocol stack (consisting of IP/TCP/HTTP2/JSON) as defined in TS 29.500 [9] or an SCTP-based stack will be used will be decided in coordination with CT WG1.

NOTE 3:	The R-AP protocol is assumed to be defined by RAN WG3 in coordination with SA WG2.

NOTE 4:	The motivation to propose the R-AP protocol instead of reusing NGAP is that (a) NGAP terminates on the AMF, while AMF is not assumed to be used by this solution and (b) that most of the underlying concepts of NGAP (existence of UE contexts at RAN nodes, PDU Sessions, support of UE mobility, etc.) do not apply to AIoT in this solution.

NOTE 5:	Whether instead of a new R-AP protocol a simplified version of NGAP can be defined to support AIoT scenarios will be decided in coordination with RAN WG3.

##### 6.3.1.6.2	Protocol Stack between AIoT Device and Application Function

Legend:

-	Reader Application Protocol (R-AP): Application Layer Protocol between the Reader and the AIoT Controller.

-	Service-based Interface (SBI) Protocol Stack: The protocol for service-based interfaces as defined in TS 29.500 [9].

-	AIoT API: The API between AIoT Controller and Application Function to support Inventory and Command Procedures.

-	Command Protocol: Application Layer Protocol between AIoT Device and Application Function to support Commands and Command Responses.

Figure 6.3.1.6.2-1: Protocol stack between AIoT Device and Application Function for Topology 1

Figure 6.3.1.6.2-2: Protocol stack between AIoT Device and Application Function for Topology 2

NOTE 1:	The AIoT API is assumed to be defined by SA WG2 (Stage 2 aspects) and CT WG3 (Stage 3 aspects).

NOTE 2:	The Command Protocol is assumed to be defined by SA WG2 (Stage 2 aspects) and CT WG1 (Stage 3 aspects).

### 6.3.2	Procedures

#### 6.3.2.1	Inventory procedure

Figure 6.3.2.1: Inventory procedure

1.	The AF sends an Inventory Request to the AIoT controller. The AF may optionally include the following information:

-	Filter Criteria to limit the inventory to AIoT Devices matching those criteria;

NOTE 1:	If the AF does not provide Filter Criteria, then a full inventory of all AIoT devices reachable by the Reader(s) will be performed.

-	a list of reader IDs to limit the inventory to specific Readers;

-	a periodicity value to request the inventory to be performed periodically.

As part of the Filter Criteria an AF may provide an EPC and optionally a bitmask. This enables the AF to trigger a targeted inventory:

-	For example, an AF may want to inventory complete shipments only (identified by AIoT devices attached to the pallets that carry the shipment), without having to unnecessarily inventory also all AIoT devices which may be located on the pallets. This can be achieved by limiting the inventory to AIoT devices that carry an EPC based on the Serial Shipping Container Code EPC scheme. To do so, the AF may provide a bitmask that only matches on the Header value of the EPC and an EPC value that contains the standardized Header value for the Serial Shipping Container Code EPC scheme. (See Annex X for further details on EPC schemes and EPC Header values.).

-	Another example is an AF that may only want to inventory individual items from a specific company. This can be achieved by limiting the inventory to AIoT devices that carry an EPC that is based on the Serialised Global Trade Item Number (SGTIN) EPC scheme and where the GS1 Company Prefix contained in the SGTIN matches the company prefix of the specific company that the AF is interested in. To do so, the AF may provide a bitmask matching on the Header value of the EPC and the bits of the GS1 Company Prefix contained in the Serialised Global Trade Item Number (SGTIN) EPC scheme and may provide an EPC that contains the standardized Header value for the Serialised Global Trade Item Number (SGTIN) and the Company Prefix value of the company that the AF is interested in.

As part of the Filter Criteria an AF may provide a Closest indication. This enables the AF to limit the Inventory Response to the AIoT device closest to the Reader:

-	For example, an AF may want to determine the Device ID of the AIoT device that a handheld Reader is close to, e.g., when a handheld Reader is held close to a shipping box with an AIoT Device attached.

2.	The AIoT controller verifies whether the AF is entitled to make the received Inventory Request, e.g. the AIoT controller verifies whether the AF is allowed to issue an Inventory Request with the specified Filter Criteria or without any Filter Criteria, without providing reader IDs, etc.

3.	The AIoT controller sends the Inventory Request to the Readers identified by the Reader IDs or to all readers, includes the Filter Criteria and periodicity information, if provided by the AF.

4.	Each Reader that received the Inventory Request from the AIoT Controller performs the Inventory procedure according to the Filter Criteria, if provided. If the Filter Criteria contain an EPC and optionally a bitmask, then the Reader sends the EPC and the bitmask as part of the inventory request. The Readers either perform a one-time inventory or perform the inventory periodically according to the received periodicity.

5.	If the Inventory Request contains an EPC, then the AIoT Device compares the received EPC with its Device ID, i.e. the locally stored EPC. If the Inventory Request also contains a bitmask, then the AIoT Device only compares the relevant bits according to the bitmask. If the Inventory Request does not contain an EPC, then the AIoT Device skips this step.

6.	If the Inventory Request contains an EPC and optionally a bitmask, and the received EPC matches the locally stored EPC (or parts of the EPC in case a bitmask was provided) as described in the previous step, then the AIoT Device responds to the Reader.

If the Inventory Request does not contain an EPC, then the AIoT Device responds to the Reader.

NOTE 2:	The AIoT Device responds to Inventory Requests that do not contain an EPC to enable the case that an AF wants to perform a full inventory.

The AIoT Device includes its Device ID in the Inventory Response.

7.	The Readers collects the received Device IDs and provides the Device IDs to the AIoT Controller. The Readers may optionally include Enrichment Data. If the Inventory Request from the AIoT Controller contain Filter Criteria including a Closest Indication, then the Reader only provides the Device ID of the AIoT Device closest to the Reader to the AIoT Controller.

NOTE X:	How the Reader determines which of the AIoT Devices is closest to the Reader is up to Reader implementation, for example signal strength can be used to achieve this.

8.	For each reported Device ID, the AIoT Controller stores the reader ID that reported the Device ID together with a timestamp. The timestamp enables the AIoT controller to purge outdated last known Reader information; the details of this are up to AIoT Controller implementation.

9.	The AIoT Controller provides the Device IDs and the Reader ID that reported the Device ID, respectively, and optionally the Enrichment Data to the AF.

NOTE 3:	The AIoT Controller provides the Device IDs as received to the AF, i.e. the AIoT Controller is not assumed to verify Device IDs.

#### 6.3.2.2	Command procedure

Figure 6.3.2.2: Command procedure

1.	The AF issues a Send Command request, which includes the Command to be sent and either:

-	a list of Device IDs that the Command is destined to; or

-	Filter Criteria that identify the AIoT Devices that are supposed to act upon the Command.

In addition, if the AF provides Filter Criteria, the AF may additionally include a list of reader IDs to use for sending the Command.

NOTE 1:	Including the list of reader IDs enables the AF to limit sending of the Command to a specific area.

2.	The AIoT Controller determines the list of Readers to use for sending the Command taking the Reader IDs (if provided by the AF) and any stored information about the last known Reader for specific Device IDs into account. The details of determining candidate Readers are up to AIoT Controller implementation.

3.	The AIoT Controller provides the Command and the Device IDs or Filter Criteria (whichever has been provided by the AF) to the Readers.

NOTE 2:	Whether the Reader needs to perform an Inventory before sending the command to the AIoT devices is a RAN WG2 decision.

4.	Each Reader that receives the Send Command request from the AIoT Controller, sends the Command to AIoT Devices as follows:

-	If the Send Command request contains one or more Device IDs, then the Reader sends the Command message to the AIoT Devices identified by the Device IDs.

-	If the Send Command request contains Filter Criteria consisting of an EPC and a bitmask, then the Reader sends the EPC and the bitmask as part of the Command message to AIoT Devices.

5.	If the Command Message contains an EPC, then the AIoT Device compares the received EPC with its Device ID, i.e. the locally stored EPC. The AIoT Device only compares the relevant bits according to the bitmask. If the Command Message contains not contain a Device ID, then the AIoT Device skips compares the received Device ID with its local Device ID.

6.	If there was a match in the previous step, then the AIoT Device executes the Command and provides the Command Response to the Reader.

7.	The Readers send the received Command Response(s) to the AIoT Controller. The Readers may optionally include Enrichment Data. If a Reader has not received any Command Response, then the Reader informs the AIoT Controller accordingly.

8.	The AIoT Controller sends the received Command Response(s) and optionally Enrichment Data to the AF. If all Readers that the AIoT Controller sent the Command to in step 3 indicate to the AIoT Controller that no Command Responses have been received, then the AIoT Controller may perform any of the following actions:

-	Attempt to deliver the Command using additional Readers, i.e. continue the procedure from step 2 and select additional Readers;

-	Buffer the Command and attempt sending the Command again later (details of this are up to AIoT Controller implementation);

-	Inform the AF that the Command could not be delivered.

NOTE 3:	Support of sending the same Command to multiple AIoT Devices requires support of group security for the Command (e.g. group keys for protecting the Command), which depends on SA WG3.

### 6.3.3	Impacts on services, entities and interfaces

New network entities and interfaces are proposed.

# >>>>NEXT CHANGE<<<<