3GPP TSG-WG2 Meeting #164	S2-2407875

19th – 23rd August 2024, Maastricht, Netherlands

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0129 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Network-assisted DAA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding support for Network-assisted DAA based on the conclusions agreed in TR 23.700-59 clause 8.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added high-level functional description of network-assisted DAA. - Added procedure for UAV/UAV-C Triggered Network-assisted DAA. - Added procedure for for AAM Triggered Network-assisted DAA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed Rel-19 feature is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7, 5.7.1, 5.7.2, 5.7.X(new), 5.7.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

## 5.7	Ground-based DAA for an Area

### 5.7.1	Functional Description

This clause provides a network-assisted (ground based) DAA (NWDAA) mechanism. It is applicable for a specific area, such as a stadium or arena where UAVs are used e.g. for filming an event. It is based on a ground-based entity, Area Airspace Manager (AAM), that is able to detect UAVs in the specific area and provide local steering policies to the individual UASs in order to avoid the UAVs crashing into each other or different structures etc. The policy may e.g. include allowed flying zones and positions allowed for the specific UAV. The policy may also apply to a specific outdoor area, e.g. an event, where specific measures to avoid collision between drones are established locally.

NOTE 1:	The policies provided by the AAM is application specific steering/DAA policies that are out of scope of 3GPP. The same or different local policies can be provided to the UAVs in the arena. UAVs can, for example, be given different fly zones in the arena just like people have different seating in an arena.

The high-level principles of the network-assisted (ground based) DAA are:

-	The arena/area has a ground-based entity Area Airspace Manager (AAM). The AAM includes one or more UEs enabled for use of PC5. The AAM may also have a direct connection to the Data Network.

-	For the applicable airspace of the area/arena the AAM defines individually adapted local collision avoidance rules for correspondingly located UAVs.

NOTE 2:	How the AAM can determine the local steering policy for collision avoidance is out of scope of 3GPP. It can e.g. be based on maps of the area/arena and detailed information about the facilities together with an awareness of the current usage need for the airspace.

-	Provisioning of AAM local collision avoidance rules to a UAV/UE must comply with the policies for PC5 operations received from the 5GC or being preconfigured in the UE.

-	Detected UAVs are identified by their coordinates and by the Remote ID as retrieved by Broadcast Remote ID (BRID) or Network Remote ID (NRID) mechanisms dependent on the method used by the UAV.

NOTE 3:	How the AAM can scan the UAVs in specific area is out of scope of 3GPP.

NOTE 4:	How the AAM retrieves the Remote ID for detected UAVs is out of scope of 3GPP. For example, in case of BRID different UAVs can use different methods, such as WLAN, Bluetooth or PC5 direct communication, for broadcasting the Remote ID and in case of NRID the AAM will, based on the geolocation of the UAV, retrieve the Remote ID from the USS at application layer. For the NRID case the AAM will typically have to be authorized by the UTM to be allowed to retrieve the Remote ID.

-	Based on the retrieved Remote ID, the AAM activates PC5 communication with each detected UAV by triggering establishment of an A2X PC5 unicast link based on procedures described in clause 6 using the UAV's Remote-ID as Application-Layer-ID (i.e. Target User Info) in the Direct Communication Request message.

For Direct Communication over PC5 the UAV and AAM shall comply with the authorization and provisioning principles described in clause 4.2.1.2.2 including the following considerations:

-	The default destination Layer-2 ID to be used for initial signalling to establish a unicast connection for the A2X service.

-	Parameters for Groupcast are not applicable.

NOTE 5:	Use of LTE PC5 for the unicast is not supported in this Release.

-	The AAM uses PC5 unicast link to provide each UAV present in the arena/area with local DAA policies as user traffic.

-	A UAV that receives local collision avoidance related policies from the AAM over PC5 may forward the policies to its UAV-C so that the UAV-C can steer the UAV accordingly by use of C2 communication (e.g. over Uu, PC5, or other means) in order to enforce the local policies and avoid collisions.

NOTE 6:	Whether and how to forward the policies from UAV to its UAV-C and how UAV-C uses the policies is out of scope of 3GPP.

NOTE 7:	A UAV-C that receives local collision avoidance policies from an AAM, can inform the AAM about its network address (e.g. IP address or URL) in order to enable direct AAM to UAV-C communication via Data Network. How a UAV-C communicates with the AAM is out of scope of 3GPP.

Figure 5.7.1-1: Logical architecture for ground based DAA

To enable NWDAA, the following information can be utilised:

-	Absolute location of a UAV provided by GMLC;

-	Relative Location of UAVs provided by GMLC;

-	Output of Relative Proximity Analytics provided by NWDAF.

Absolute location of a UAV is generated/derived in 5G network by using the existing LCS feature defined in TS 23.273 [8]. USS may request to GMLC to obtain one or multiple UAV location as defined in clause 6.2.1, 6.3.1 or 6.20.5 in TS 23.273 [8]. As an example, USS may use the absolute location of the UAV(s) provided by the network to e.g. calculate relative distance of the pair of UAVs, and based on that to deal with the UAV tactical deconfliction, collision avoidance etc.

Relative location of a UAVs is provided in 5G network by using the existing LCS Ranging feature defined in TS 23.273 [8]. USS may send a Ranging request to GMLC to obtain relative location between one and multiple UAVs as defined in clause 6.20.3 or 6.20.4 in TS 23.273 [8]. USS may use the relative location of the UAV(s) to avoid collision.

USS (acting as an AF) may request Relative Proximity Analytics provided by NWDAF for the purpose of DAA.

NOTE 8:	How USS uses the relative distance of UAVs or absolute location of a UAV is out of 3GPP scope.

>>>> Next Change <<<<

### 5.7.2	High-level Procedure for Ground-based DAA in an Area

Figure 5.7.2-1: High-level procedure for Ground-based DAA for an Area

Ground-based DAA for an area leverages the procedures and mechanisms defined for A2X. Any references to TS 23.287 [11] made in this clause shall be interpreted in accordance with corresponding definitions and descriptions for A2X in clause 6.

Prerequisites:	The AAM/UE and the UAVs/UEs are configured to use an A2X service for Ground-based DAA for an area.

1.	The UAVs/UEs listens for signals on the correspondingly destination Layer-2 ID configured for the used service type in accordance with clause 6.3.3.1 of TS 23.287 [11] (Fig 6.3.3.1-1 step 1).

2.	The AAM can scan the airspace over the area/arena for UAVs, e.g. by making use of upward pointing radars and cameras etc. For each detected UAV it determines the coordinates.

NOTE 1:	How step 2 and step 3 are done is out of 3GPP scope.

3.	The AAM retrieves for each detected UAV the corresponding Remote-ID using the method applicable for the specific UAV. Methods which may be used includes broadcast of Remote-ID via PC5, WiFi and Bluetooth and also Network Remote ID.

How this is done is out of the 3GPP scope.

4.	For each detected UAV/UE the AAM establishes a PC5 direct communication link with the discovered UAV for AAM to UAS interaction by performing the Layer-2 link establishment procedure as described when Target User Info is included in clause 6.3.3.1 of TS 23.287 [11] (Fig 6.3.3.1-1 step 2, 3 4a, 5a and 6). The AAM application layer provides a service type indicating the A2X service "Ground-based DAA for an area" and the retrieved Remote-ID as target UE's Application Layer ID. As a result of this procedure a PC5 unicast direct communication link enabling bidirectional data exchange is set up between the application layer in the AAM and the UAV application layer of the UAV/UE having the specified Remote-ID.

5.	Optionally the UAV/UE may, when the PC5 unicast direct communication link between the AAM and the UAV/UE has been set up for NR, activate a corresponding bidirectional communication connection extending the link from the UAV to the UAV-C using the specific communication technology used for C2 communication (may e.g. be LR Wi-Fi or PC5). This enables packets received on the link (i.e. from the AAM) to be forwarded to the UAV-C and packets received from the UAV-C to be forwarded on the link (i.e. towards the AAM). Implementation aspects for this step are out of scope for 3GPP.

6.	Using the PC5 unicast direct communication link the AAM and the UAV establishes a bidirectional communication channel for exchange of messages. Optionally this channel can be extended to involve the UAV-C using the bi-directional tunnel. The protocol and implementation aspects for this step are out of scope for 3GPP.

NOTE 2:	The UAV-C and/or the UAV can use the communication channel to provide application specific information to the AAM, how and what information is out of 3GPP scope. Based on awareness of the current UAV traffic situation, local info and plans for the area/arena, the AAM determines locations and flight paths appropriate for the specific UAV such that collisions can be avoided and creates a corresponding application specific steering policy being out of scope of 3GPP.

7.	The AAM provides the determined steering policy to the specific UAV using the activated communication channel. Optionally it can be forwarded to the UAV-C which can return corresponding C2 commands. Protocols, semantics and syntax for handling this are application specific and out of 3GPP scope.

8.	The UAV is steered to avoid collisions in accordance with received policy and using mechanisms that are out of scope for 3GPP.

NOTE 3:	A UAV-C that optionally receives local policies for DAA from an AAM, can inform the AAM of its address (e.g. IP address or URL) to enable direct AAM to UAV-C communication via Data Network.

>>>> Next Change <<<<

### 5.7.X	Procedure for UAV Triggered Network-assisted DAA

Figure 5.7.X-1: Procedure for UAV/UAV-C Triggered Network-assisted DAA

1.	The UAV establishes a PDU Session for communication with the USS as described in clause 5.2.3.

2.	The DAA service may be triggered by UAV(s). The UAV(s) (via its paired UAV-C) requests DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides to subscribe/request to 5GC for GMLC service on Ranging/Sidelink Positioning location and/or Relative Proximity predictions on collision from NWDAF.

NOTE 1:	The other content of DAA service information derived at USS is out of scope.

3a.	The USS may request GLMC service via NEF for Ranging/Sidelink Positioning location results, i.e. the relative positioning between UAV and UAV-C as described in clause 6.20.3 of TS 23.273 [8] for one notification, or clause 6.20.4 of TS 23.273 [8] for notifications (steps 1-20 for initiation monitoring, steps 21-31 for monitoring periodic).

3b.	USS as AF/LCS client may send the absolute positioning request to GMLC, including UAV UE1 ID (UE1 GPSI) and UAV UE2 ID (UE2 GPSI) as described in clause 6.2.1 and 6.3.1. GMLC replies to USS of the absolute location of UAV 1 and UAV 2. USS calculates the relative positioning result based on the UAV UE1 absolute location and UAV UE2 absolute location.

NOTE 2:	USS can request same or different GMLC for the UAV1 and UAV 2 location, e.g. depending on which PLMN the UAV registers.

4.	The USS may subscribe or request notification on Relative Proximity predictions provided by NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as defined in clause 6.1.1.2 of TS 23.288 [Y] or Nnef_AnalyticsExposure_Fetch service operation as defined in clause 6.1.2.2 of TS 23.288 [Y]. The subscribe/request message include identifier of the UAV(s) (e.g. GPSI(s)). The other parameters included in the request are described in clause 6.5.3.3.

5.	After NEF receive the request from the USS, the NEF interacts with the NWDAF as described in the procedure in clause 6.19.4 of TS 23.288 [Y]. The NEF maps the parameters in the request from the USS to information used by the 3GPP system.

6.	If the NEF receives the response from the NWDAF, the NEF notifies the USS with the Relative Proximity predictions by invoking Nnef_AnalyticsExposure_Notify service operation for a Subscribe-Notify model as defined in clause 6.1.1.2 of TS 23.288 [Y] or Nnef_AnalyticsExposure_Fetch service operation for a Request-Response model as defined in clause 6.1.2.2 of TS 23.288 [Y].

7.	If receiving the location of UAV1 and UAV2, USS calculates the relative positioning result for UAV UE1 location and UAV UE2 location from step 1 and step 2. The USS estimates the potential collision based on the information received in step 3 and/or the predictions from step 6. The USS informs the UAV-C(s) the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

USS may be replaced by TPAE.

8.	The UAV-C(s) informs its paired UAV(s) the potential collision, and information received from the USS, include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision, and deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

9.	UAVs performs operations to avoid collision.

9a.	If both the two UAVs which may collision (e.g. UAV 1 and UAV 2) been informed by their paired UAV-C(s), the UAVs can be steered to avoid collision in accordance with the received information (e.g. trajectory correction information to avoid collision) and using mechanisms that are out of scope for 3GPP.

9b.	If only one of the two UAVs which may collision (e.g. UAV 1) been informed by its paired UAV-C, the UAV 1 triggers conflict resolution procedure with UAV 2 as described in steps 4-7 in clause 5.6.1.

>>>> Next Change <<<<

### 5.7.Y	Procedure for AAM Triggered Network-assisted DAA

Figure 5.7.Y-1: Procedure for AAM Triggered Network-assisted DAA

1.	As described in clause 5.7.2 steps 1-6, the UAV(s) listens for signals on the correspondingly destination Layer-2 ID configured for the used service type. The AAM scans the airspace over the area/arena for UAV(s), retrieves for each detected UAV the corresponding Remote-ID, and establishes a PC5 direct communication link with the discovered UAV. Using the PC5 unicast direct communication link the AAM and the UAV establishes a bidirectional communication channel for exchange of messages.

2.	The AAM establishes a PDU Session for communication with the USS as described in clause 5.2.3.

3.	The AAM may request network assist DAA service from USS. The request message includes identifier of the UAV(s) (e.g. GPSI(s), CAA-Level UAV ID(s)). USS derives information on DAA service and decides, e.g. to request GMLC service for Ranging/Sidelink Positioning location, to subscribe/request to NWDAF for Relative Proximity predictions on collision.

NOTE:	The other content of DAA service information derived at USS is out of scope.

4.	The USS requests GMLC service for Ranging/Sidelink Positioning location, and/or subscribes/requests notification on Relative Proximity predictions provided by NWDAF via NEF, as described in clause 5.7.X steps 3-6.

5.	The USS estimates the potential collision based on the received information/analytics in step 4. The USS informs the AAM the potential collision. The message may include collision alert, predicted time of collision, CAA-level UAV IDs of the paired UAVs which may collision (e.g. UAV 1 and UAV 2), deconflicting specific parameters (e.g. trajectory correction information to avoid collision).

6.	The AAM provides the determined steering policy to the specific UAVs (e.g. UAV 1 and UAV 2) according to the information received in step 5, and the UAVs are steered to avoid collisions in accordance with received policy and using mechanisms that are out of scope for 3GPP, as described in clause 5.7.2 steps 7-8.

>>>> End of Changes <<<<