3GPP SA WG2 Meeting #164	S2-2407883

Maastricht, NL, 19-23 August 2024

# Title:	[Draft] LS on input data for analytics of NWDAF assisted Network Abnormal Behaviour Mitigation and Prevention

# Response to:	-

# Release:	Release 19

# Work Item:	AIML_CN

Source:	SA2

To:	SA5

Cc:	-

Contact Person:

Name:	Juan Zhang

E-mail Address:	juanzhan@qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

KI#4 in Rel-19 SID FS_AIML_CN deals with NWDAF-assisted Network Abnormal Behaviour Mitigation and Prevention. The conclusion of KI#4 is approved in clause 8.4 in TR 23.700-84 and the corresponding objectives have already been approved in the AIML_CN Rel-19 WID. Based on the conclusions, SA2 aims to determine a new Analytics ID to support signalling storm mitigation and prevention caused by NFs' signalling in normative work. SA2 will further determine in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support signalling storm mitigation and prevention caused by massive signalling of UEs.

SA2 also approved to collect several input data from NFs or OAM to support analytics by NWDAF as described in clause 6.35.2.2 and table 6.39.1-1 in TR 23.700-84. However, there is still an Editor’s note remaining for all the proposed input data tables in clause 6.35.2.2:

Editor's note:	It is FFS which parameters in tables above are per UE, and which ones are per NF, and whether and how the ones per NF are retrieved from OAM.

There is also a NOTE 2 for the input data from 5GC related to Control Plane Signalling Abnormality in table 6.39.1-1:

NOTE 2:	The already supported counters/KPIs by OAM defined per NF in TS 28.552 [11] are collected from OAM; other remaining counters/KPIs are collected from the corresponding NFs.

Some of the parameters referred in clause 6.35.2.2 and table 6.39.1-1 in TR 23.700-84 are possibly already supported by OAM, and NWDAF can collect these parameters from OAM. Therefore, it may not be necessary to collect these parameters from specific NFs. SA2 kindly asks SA5 to check the parameters mentioned in clause 6.35.2.2 and table 6.39.1-1 in TR 23.700-84 and provide feedback which parameters are already supported by OAM.

SA2 will finally determine the source of the parameters in normative work.

2. Actions:

To SA5:

ACTION: 	SA2 kindly asks SA5 to indicate which parameters mentioned in clause 6.35.2.2 and table 6.39.1-1 in TR 23.700-84 are already supported by OAM.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #165		14-18 October 2024	Hyderabad, India

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, FL, USA