3GPP TSG-WG SA2 Meeting #164	S2-2407894

# Maastricht, NL, 19th Aug – 23rd Aug, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1143 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction NWDAF discovery and selection. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in 6.1B of TS 23.288, the procedures for analytics subscription and analytics context transfer allow one NWDAF instance to transfer its ongoing analytics subscriptions to another NWDAF instance. An analytics subscription transfer to the target NWDAF may be initiated by the source NWDAF (6.1B.2.2). The contents of analytics context contain the ML model information and only the AnLF which vendor information is in the ML interoperability indicator can support interoperate the Model from the MTLF. In order to enhance the performance of NF discovery and selection, the source AnLF prefer to discovery the Target AnLF with the ability to using the model. In the FL related procedure, the globle model may be send to the FL client, the FL server shall make sure the Vendor ID of the FL client is within its Interoperability Indicator. The NWDAF with FL capability shall register its NF information (vendor ID) in the NRF and the FL server may get the Vendor ID from the NF profile during the NF discovery. Besides, in order to align with TS33.501 clause X.10, in case of NF consumer request ML models on behalf of a ML Model consumer to NWDAF containing MTLF, the request shall contain the NF Instance ID of Model consumer and Vendor ID of Model consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add ML model interoperability information in the NWDAF procedure. Adding ML Model consumer information in the Model procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Analytics Context Transfer cannot be fully supported. MTLF-based ML Model Accuracy Monitoring cannot be fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2C.2.1, 6.2C.2.2, 6.2A.2, 6.1B.2.2, 6.2E.2, 7.5.2, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.501 CR 5481, 5482 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-    If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer may query NRF providing the ML Model Interoperability indicator of the specific NWDAF containing MTLF in the discovery request to get the NWDAF(s) which NF consumer information is in the ML Model Interoperability indicator.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and NF consumer information. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include NF consumer information, FL capability type (i.e. FL server) and may include Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include NF consumer information, FL capability type (i.e. FL client) and may include Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 6:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 7:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

# * * * * Next changes * * * *

6.2C	Federated Learning among Multiple NWDAFs

6.2C.1	General

This clause specifies how NWDAF containing MTLF can leverage Federated Learning technique to train an ML Model.

6.2C.2	Procedures

6.2C.2.1	Registration and Discovery procedure for Federated Learning

Figure 6.2C.2.1-1: Registration and Discovery procedure for Federated Learning

Steps 1 to 3 are the NWDAF registration procedure.

1-3.	NWDAF containing MTLF as FL Server NWDAF or FL Client NWDAF registers to NRF with its NF profile, which includes NF consumer information, NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, FL capability type information (i.e. FL server and/or FL client) and Time interval supporting FL as described in clause 5.2.

Steps 4 to 6 are the NWDAF Discovery procedure.

4-6.	NWDAF containing MTLF determines ML Model requires FL based on operator policy (e.g. pre-configured list of ML Models), Analytic ID, Service Area/DNAI or data can not be obtained directly from data producer NF (e.g. due to privacy reasons).

If the NWDAF containing MTLF can not perform as FL Server NWDAF, the MTLF first discovers and selects FL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, Model filter information as defined in clause 6.2A.2, FL capability Type (i.e. FL server), Time Period of Interest, Service Area.

Once the FL Server NWDAF (the requested or the selected one) is determined, the FL Server NWDAF discovers and selects other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, FL capability Type (i.e. FL client), Service Area, NF consumer information of FL Client NWDAF, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local ML Model training, Time Period of Interest, ML Model Interoperability Indicator.

7.	FL Server NWDAF sends Federated Learning preparation request to the FL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML Model training requirement (e.g. Analytics ID, ML Model Interoperability information, Data Availability requirement, FL Availability time requirement (time span needed for the FL process), etc.). Data Availability requirement includes a list of Event IDs of the local data for training, and may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples.

NOTE:	Federated Learning preparation procedure (i.e. steps 7-9) can be skipped if the FL Server NWDAF can decide that the FL Client NWDAF(s) supports the FL procedure to be performed, e.g. based on information acquired from previous FL procedures or from the NRF, or based on local configuration.

8.	FL Client NWDAF(s) checks if it can meet the ML Model training requirement and/or can successfully download the model if the model information is provided in the request and decides whether to join the Federated Learning process based on operator policy (e.g. pre-configured list of ML Models) and/or implementation. Example criteria used by FL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and ML Model Interoperability information.

9.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the FL Server NWDAF whether it will join the FL procedure and may include the reason in the response message if it cannot join the FL process.

10.	FL Server NWDAF determines the FL Client NWDAFs to be involved in the FL procedures based on the information received in step 6 and other information received in step 9 (if available).

6.2C.2.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.2C.2.2-1: General procedure for Federated Learning among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF or NWDAF containing MTLF) sends a subscription request to FL server NWDAF to retrieve an ML Model, using Nnwdaf_MLModelProvision including Analytics ID, ML Model Monitoring information as defined in clause 7.5.2, desired ML Model metric (e.g. ML Model Accuracy).

NOTE 1:	The ML Model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process and the FL server NWDAF may stop the training process when the ML Model Accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF or NWDAF containing MTLF) provides the Time when the ML Model is needed, the FL Server NWDAF can take this information into account to decide the maximum response time for its FL Client NWDAF(s).

1.	FL Server NWDAF selects NWDAF(s) containing MTLF (FL Client NWDAF(s)) as described in clause 6.2C.2.1.

2.	FL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) containing MTLF (FL Client NWDAF(s)), which participate in the Federated learning to perform the local model training and determine the interim local ML Model information based on the input parameter in the request from FL Server NWDAF. The request includes the desired ML Model metric and initial ML Model and also includes the maximum response time, the FL Client NWDAF has to report the interim local ML Model information to the FL Server NWDAF before the maximum response time elapses.

3.	[Optional] Each FL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the Client NWDAF has not local data available already.

4.	During Federated Learning training procedure, each FL Client NWDAF further trains the ML Model provided by the FL Server NWDAF based on its local data and reports the interim local ML Model information to the FL Server NWDAF in Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response may also include the Status report of FL training that includes local ML Model metric value (and optionally the used metric) computed by the FL Client NWDAF and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.) in the FL Client NWDAF. The Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Response also includes the global ML Model metric (value (and optionally the used metric) when the ML Model Accuracy Check Flag was included in the Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request (as described in step 7), the global ML Model metric value is calculated by the FL Client NWDAF using the local training data as the testing dataset.

NOTE 2:	The parameters in characteristics of local training dataset are up to the implementation.

The local ML Model, which is sent from the FL Client NWDAF(s) to the FL Server NWDAF during the FL training process, is the information needed by the FL Server NWDAF to build the aggregated model.

If the FL Client NWDAF is not able to complete the training of the interim local ML Model within the maximum response time provided by the FL Server NWDAF, the FL Client NWDAF shall send the Delay Event Notification that include the delay event indication, an optional cause code (e.g. local ML Model training failure, more time necessary for local ML Model training) and the expected time to complete the training if available to the FL Server NWDAF before the maximum response time elapses.

4a.	[Optional]If FL Server NWDAF receives notification/response that the FL Client NWDAF is not able to complete the training within the maximum response time, the FL Server NWDAF may send to the FL Client NWDAF a new maximum response time in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request, before which the FL Client NWDAF has to report the interim local ML Model information to the FL Server NWDAF. Otherwise, the FL Server NWDAF may indicate FL Client NWDAF to skip reporting for this iteration. FL Server NWDAF includes the current iteration round ID in the message to indicate that the request is to modify the training parameters of the current iteration round.

Alternatively, the FL Server NWDAF may inform the FL Client NWDAF to cease the ML Model training by sending termination request and to report back the current local ML Model updates.

5.	The FL Server NWDAF aggregates all the local ML Model information retrieved at step 4, to update the global ML Model. The FL Server NWDAF may also compute the global ML Model metric value, e.g. based on the local ML Model metric value(s) provided by the FL Client NWDAF(s) or by applying the global model on the validation dataset (if available). The FL Server NWDAF may update the global ML Model each time a FL Client NWDAF provides updated local ML Model information, or the FL Server NWDAF may decide to wait for local ML Model information from all FL Client NWDAFs before updating the global ML Model.

If the FL Server NWDAF provides the maximum response time for the FL Client NWDAF(s) to provide the interim local ML Model information in step 2, or the new maximum response time in step 4a, the FL Server NWDAF decides either to wait for the FL Client NWDAF(s) which have not yet provided their interim local ML Model within the new maximum response time or to aggregate only the retrieved local ML Model information instances to update global ML Model. The FL Server NWDAF makes this decision, considering the notification/response from the FL Client NWDAF or, if the notification is not received, based on local configuration.

6a.	[Optional] Based on the consumer request in step 0, the FL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message to update the global ML Model metric value to the consumer periodically (e.g. a certain number of training rounds or every 10 min) or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

6b.	[Optional] The consumer decides whether the current model can fulfil the requirement, e.g. global ML Model metric value is satisfactory for the consumer and determines to stop or continue the training process. The consumer re-invokes Nnwdaf_MLModelProvision_Subscribe service operation as used in step 0 to continue the training process or invokes Nnwdaf_MLModelProvision_Unsubscribe service operation to stop the training process.

6c.	[Optional] Based on the subscription request sent from the consumer in step 6b, the FL Server NWDAF updates or terminates the current FL training process.

If the FL Server NWDAF received a request in step 6b to stop the Federated Training process, steps 7 and 8 are skipped.

7.	If the FL procedure continues, FL Server NWDAF may determine FL Client NWDAF as described in clause 6.2C.2.3 and sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the aggregated ML Model information to selected FL Client NWDAF(s) for next round of Federated Training. The request may also include the ML Model Accuracy Check Flag, that indicates the FL Client NWDAF(s) to use the local training data as the testing dataset to calculate the Model Accuracy of the global ML Model provided by the FL Server NWDAF.

8.	Each FL Client NWDAF updates its own ML Model based on the aggregated ML Model information distributed by the FL Server NWDAF at step 7.

NOTE 3:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the Federated Training procedure is complete, the FL Server NWDAF requests the FL client NWDAF(s) to terminate the FL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the FL process has finished and optionally with the final aggregated ML Model information. Then the FL client NWDAF(s) terminate the local model training and if the final aggregated ML Model information is received from the FL server NWDAF, the FL client NWDAF(s) can store it for further use.

9.	After the training process is complete, the FL Server NWDAF shall verify the NF consumer information of FL Client NWDAF(s) is in the Vendor list indicated by its ML Model Interoperability indicator of the Analytic ID and send Nnwdaf_MLModelProvision_Notify that includes the globally optimal ML Model information to the consumer.

# * * * * Next changes * * * *

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML Model provisioning services as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	A list of Analytics IDs: identifies the analytics for which the ML Model is used.

-	[OPTIONAL] NF consumer information: identifies the vendor of NWDAF containing AnLF or NWDAF containing MTLF.

-    [OPTIONAL] Vendor ID of the ML model consumer: identifies the vendor of NWDAF which is the Model consumer (in the case of the NF consumer request ML models on behalf of a ML Model consumer to NWDAF containing MTLF (e.g., FL server NWDAF) as described in clause 5.3).

-    [OPTIONAL] NF Instance ID of ML Model consumer  identifies the NWDAF which is the Model consumer (in the case of the NF consumer request ML models on behalf of the ML Model consumer to NWDAF containing MTLF (e.g., FL server NWDAF)  as described in clause 5.3).

NOTE 1:	NF consumer information such as Vendor ID is defined in Stage 3.

For each Analytics ID, the following parameters may be provided:

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant t ML Model.

NOTE 2:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML Model, when several ML Models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: enables the NWDAF containing MTLF to select which ML Model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML Model is requested, e.g. specific UEs i.e. a list of SUPIs, a group of UEs i.e. a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UE(s) in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs i.e. a list of Internal-Group-Ids.

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML Model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by AnLF during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 3:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used, indicated as a list of NF instance (or NF set) identifiers.

-	[OPTIONAL] Number of ML model(s), indicating the maximum number of ML models that the NWDAF containing MTLF could provide to the NWDAF containing AnLF for the Analytics ID.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	[OPTIONAL] Indication of supporting multiple ML Models.

-	[OPTIONAL] Accuracy level(s) of Interest.

NOTE 4:	Multiple ML Models Filter Information are composed by Accuracy level(s) of Interest and Number of ML Model(s).

-	[OPTIONAL] Time when model is needed: indicates the latest time when the consumer expects to receive the ML Model(s).

-	[OPTIONAL] ML Model Monitoring Information:

-	desired ML Model metric.

NOTE 5:	In this Release, only "ML Model Accuracy" is defined as ML Model metric.

-	[OPTIONAL] (only for Nnwdaf_MLModelProvision_Subscribe service operation) ML Model monitoring reporting mode: such as Accuracy reporting interval or pre-determined status. Depending on the reporting mode, the NWDAF containing MTLF reports the ML Model accuracy to NWDAF containing AnLF either periodically or when the ML Model accuracy is crossing an ML Model Accuracy threshold, i.e. the accuracy either becomes higher or lower than the ML Model Accuracy threshold.

-	[OPTIONAL] ML Model Accuracy Threshold: indicating the accuracy threshold of the ML Model requested by the consumer (as a kind of pre-determined status). It also can be used as an indication that the MTLF is triggered to execute the accuracy monitoring operations for the ML Model provisioned to AnLF.

-	[OPTIONAL] DataSetTag and ADRF ID if available: indicates the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) stored in ADRF which can be used by MTLF to retrain or reprovision of the ML Model.

-	[OPTIONAL] ML Model identifier: indicates the Model that the data corresponding to the DataSetTag is related to (in the case of subscription modification).

The NWDAF containing MTLF provides to the consumer of the ML Model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the following information.

-	ML Model Information, which includes:

-	the ML Model file address; or

-	ADRF (Set) ID.

When ADRF (Set) ID is provisioned and the MTLF authorizes the NF Service Consumer to retrieve all ML Models corresponding to a Storage Transaction ID, a Storage Transaction ID may also be provisioned.

-	[OPTIONAL] ML Model Filter Information and/or Target of ML Model Reporting may be also provided. They are provided when the ML Model provisioning request include same Analytics IDs but with different Targets of ML Model Reporting and/or ML Model Filter Information.

-	[OPTIONAL] ML Model provide indicator: indicates that the ML Model corresponding to the ML Model identifier is updated (e.g. re-trained ML Model).

-	[OPTIONAL] ML Model degradation indicator: indicates whether the provided ML Model is degraded.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 6:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

-	[OPTIONAL] ML Model representative ratio: indicating the percentage of UE(s) in the group whose data is used in the ML Model training when the Target of ML Model Reporting is a group of UE(s).

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the data characteristics and granularity with which this data has been used (i.e. a sampling ratio, the maximum number of input values and/or a maximum time interval between the samples of this input data, data range including maximum and minimum values, mean and standard deviation and data distribution when applicable) and the time, i.e. timestamp and duration, when this data was obtained.

-	the data sources related to the "Input Data" that were used for ML Model training, which have been identified by a list of NF instance (or NF set) identifiers.

NOTE 7:	This can be a subset of the possible Input Data specified for a certain analytics type.

NOTE 8:	Data source information enables ML Model selection when different models are available for an Analytics ID, or it enables a consumer to avoid selecting a ML Model that used data from a specific data source at a particular time or used data characterized by specific data characteristics.

-	[OPTIONAL] ML Model Accuracy Information: indicates the accuracy of the ML Model if related ML Model Monitoring Information was provided, which includes:

-	the metric value of the ML Model.

-	[OPTIONAL] used ML Model metric.

# * * * * Next changes * * * *

6.1B.2.2	Analytics Subscription Transfer initiated by source NWDAF

The procedure in Figure 6.1B.2.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF should not perform the procedure. In such a case, the source NWDAF may send a message to indicate to the analytics consumer that it will not serve this subscription anymore.

NOTE 1:	To discover the possible change of analytics consumer, if the Analytics ID is UE related, the source NWDAF takes actions responding to external trigger (such as UE mobility), for example, checking if the Target of Analytics Reporting is still within the serving area of the analytics consumer, if the serving area information is available.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2.2-1 is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 6.1B.2.2-1: Analytics subscription transfer initiated by source NWDAF

0.	The analytics consumer subscribes to analytics from source NWDAF. The analytics consumer may send its NF ID or serving area, enabling NWDAF to determine whether the following analytics subscription transfer procedure is applicable. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in clause 6.1B.2.3.

2.	Source NWDAF determines, e.g. based on the UE location information received and the analytics consumer's serving area either directly received in step 0 or indirectly received via NRF, to perform an analytics subscription transfer to target NWDAF(s). Therefore, the source NWDAF determines the analytics subscription(s) to be transferred to a target NWDAF.

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF as specified in clause 5.2. The procedure for NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer. In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer. If the analytics subscription to be relocated to a target NWDAF also includes the Analytics Accuracy Request information, the source NWDAF selects, if possible, a target NWDAF also with accuracy checking capability. If the source NWDAF does not discover a target NWDAF with accuracy checking capability, the source NWDAF notifies the analytics consumer with Accuracy Information Termination. Therefore, the consumer based on local policy may decide to unsubscribe the analytics ID or to keep using the analytics ID even without receiving the accuracy information.

4.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request may also contain active data source ID(s) and ML Model related information, which are related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The ML Model related information contains the ID(s) of NWDAF(s) containing MTLF that provided the trained models and may contain the file address(es) of the trained ML Model(s), where the file address(es) of the trained ML Model(s) is included only when the source NWDAF itself provides the trained ML Model(s) for the analytics subscription(s) being transferred. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation and if applicable, the Analytics Accuracy Information generation, based on the information received from the source NWDAF.

The target NWDAF may use analytics accuracy request information included in the analytics subscription transfer received in step 4 to start the process of checking and generating Analytics Accuracy Information for the consumer of the transfer analytics subscription.

Target NWDAF may use the ML Model related information, if provided in the Nnwdaf_AnalyticsSubscription_Transfer request. If the ID(s) of NWDAF(s) containing MTLF is provided in the Nnwdaf_AnalyticsSubscription_Transfer request and the NWDAF(s) containing MTLF is part of the locally configured set of NWDAFs containing MTLF, target NWDAF may request or subscribe to the ML Model(s) from the indicated NWDAF(s) containing MTLF as specified in clause 6.2A and use the ML Model(s) for the transferred analytics subscription. If the file address(es) of the trained ML Model(s) is provided and if the NWDAF containing MTLF is part of the locally configured set of NWDAFs containing MTLF, the target NWDAF may retrieve the ML Model using the file address of the trained ML Model. If the provided ID(s) of NWDAF(s) containing MTLF are not part of the locally configured set of ID(s) of NWDAFs containing MTLF, the target NWDAF discovers the NWDAF(s) supporting MTLF that can provide trained ML Model(s) for the Analytics ID(s) as described in clause 5.2.

NOTE 3:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new Subscription Correlation ID to the received analytics subscriptions.

NOTE 4:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message. A new Subscription Correlation ID, which was assigned by the target NWDAF, is provided in the Subscription Correlation ID and the old Subscription Correlation Id, which was allocated by the source NWDAF, is provided in the Subscription Change Notification Correlation ID parameter of this message as specified in clause 7.2.4.

NOTE 5:	Notification correlation information in the Nnwdaf_AnalyticsSubscription_Notify message allows the analytics consumer to correlate the notifications (of analytics output and if applicable of Analytics Accuracy Information) to the initial subscription request made with the source NWDAF in step 0.

NOTE 6:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

NOTE 7:	If this subscription is used as input for analytics aggregation by the analytics consumer, the analytics consumer might inform the other NWDAFs instance participating in this analytics aggregation that the Set of NWDAF identifiers of NWDAF instances used by the NWDAF service consumer for this analytics aggregation (see clause 6.1.3) has changed using the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the target NWDAF requests the "analytics context". The analytics context transfer procedure is specified in clause 6.1B.3.

If the transfer request received by the target NWDAF also includes the Analytics Accuracy Request information, the target NWDAF will include in the Nnwdaf_AnalyticsInfo_ContextTransfer request the field Requested Analytics Context Type with value set to Analytics accuracy related information (as defined in clause 6.1B.4) in order to retrieve the necessary information for generating the Analytics Accuracy Information.

The target NWDAF may also retrieve from source NWDAF containing AnLF the ML Model Accuracy Information for the ML Model when ML Model accuracy related information context type is included in the "analytics context identifier(s)" in the transfer request. Based on the retrieved ML Model accuracy related information, the target NWDAF containing AnLF registers as provider of ML Model Accuracy Information for the ML Model as defined in clause 6.2E.3.2.

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions. In addition, Source NWDAF unsubscribes with the NWDAF(s) containing MTLF, if exist, which are no longer needed for the remaining analytics subscriptions.

NOTE 8:	At this point, the analytics subscription transfer is deemed completed, i.e. the source NWDAF can delete all information related to the successfully transferred analytics subscription.

11-12.	Target NWDAF at some point derives new output analytics and Analytics Accuracy Information (if applicable) based on new input data and notifies the analytics consumer about the new analytics and new Analytics Accuracy Information (if applicable) using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

# * * * * Next changes * * * *

6.2E	MTLF-based ML Model Accuracy Monitoring

6.2E.1	General

MTLF-based ML Model Accuracy Monitoring procedure is where an NWDAF containing MTLF determines ML Model degradation based on newly collected test data and retrain or reprovisioning the existing ML Model.

6.2E.2	Procedure for MTLF-based ML Model Accuracy Monitoring

Figure 6.2E.2-1 illustrates the procedure for monitoring the accuracy of the provisioned ML Model using newly collected data. NWDAF containing AnLF may provide inference data to NWDAF containing MTLF for model accuracy monitoring and the NWDAF containing MTLF determines retraining or re-provisioning of the ML Model.

Figure 6.2E.2-1: Procedure for MTLF-based ML Model Accuracy Monitoring

1.	An analytics consumer initiates a subscription for analytics exposure services towards an NWDAF containing AnLF.

2.	The NWDAF containing AnLF requests an ML Model from the appropriate NWDAF containing MTLF, using the Nnwdaf_MLModelProvision_Subscribe service operation. The NWDAF containing AnLF may include an ML Model accuracy threshold which is used as an indicator to execute the accuracy monitoring operations as defined in clause 6.2A.2. NWDAF containing AnLF may include a DataSetTag (see clause 6.2B.1) and/or ADRF ID, which is used to store and fetch the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) from ADRF which are relevant for the accuracy monitoring and re-training/re-provisioning of ML Model.

If the NWDAF containing AnLF receives ML Model(s), the NWDAF containing AnLF sends set of tuples (unique ML Model identifier and a DataSetTag and/or ADRF ID) to the NWDAF containing MTLF by invoking Nnwdaf_MLModelProvision_Subscribe service operation for subscription modification.

3.	The NWDAF containing MTLF provides trained ML Model(s) to the NWDAF containing AnLF as specified in clause 6.2A and 6.2B. The NWDAF containing MTLF may include an accuracy information which is used to indicate the accuracy of ML Model during the training.

When the step 2 is for a subscription modification (i.e. including Subscription Correlation ID) and contains the set of tuples (unique ML Model identifier and a DataSetTag and/or ADRF ID), the NWDAF containing MTLF determines the relationship between the ML Model and the DataSetTag.

4.	The NWDAF containing AnLF registers the use of the ML Model with the NWDAF containing MTLF to indicate its capability of sending Analytics Feedback Information and/or ML Model Accuracy Information from the analytics consumers for the ML Model.

5.	Due to the registration in the previous step, the NWDAF containing MTLF may subscribe to the NWDAF containing AnLF to get Analytics Feedback Information and/or ML Model Accuracy Information from the analytics consumers for the provisioned ML Model by invoking Nnwdaf_MLModelMonitor_Subscribe service operation, if the service operation is supported by the NWDAF containing AnLF.

6.	The Analytics consumer may send Analytics Feedback Information in an Nnwdaf_AnalyticsSubscription_Subscribe message as described in clause 6.1.1.

7.	The NWDAF containing AnLF may send the Analytics Feedback Information and/or ML Model Accuracy Information received from the analytics consumer for the provisioned ML Model by invoking Nnwdaf_MLModelMonitor_Notify service operation as requested in step 5. When the NWDAF containing MTLF receives Analytics Feedback Information and/or ML Model Accuracy Information, the NWDAF containing MTLF may trigger step from 8 to 13 to enhance the ML Model accuracy.

8a-8f.	The NWDAF containing MTLF, based on the request(s) from one or more NWDAF containing AnLF or its local policy, determines whether to perform ML Model Accuracy Monitoring and re-training/re-provisioning of ML Model by collecting new data from various data sources:

-	The NWDAF containing MTLF may collect new data for ML Model Accuracy monitoring, re-training and re-provisioning from the data source NFs and DCCF by invoking Nnf_EventExposure_Subscribe and Ndccf_DataManagement_Susbscribe service operation, respectively.

-	When ADRF ID and/or DataSetTag is given by step 2, the NWDAF containing MTLF may retrieve historical data from the ADRF indicated by the NWDAF containing AnLF at step 2. by invoking Nadrf_DataManagementRetrievalRequest or Nadrf_DataManagementRetrieval_Subscribe service operation. Otherwise, the NWDAF containing MTLF may retrieves the historical data from the DCCF or the NWDAF containing AnLF by invoking Ndccf_DataManagement_Subscribe or Nnwdaf_DataManagement_Subscribe service operation, respectively.

-	If the NWDAF containing AnLF does not include a DataSetTag with ADRF ID at step 2, the NWDAF containing MLTF may request ADRF to subscribe for the collection of the analytics and data that correspond to the analytics generated by the ML Model provisioned in step 3, using the procedures defined in clause 6.2B.3.

-	The NWDAF containing MTLF may subscribe to UDM to get notification on change in the subscription data for Target of ML Model Reporting by invoking Nudm_SDM_Subscribe service operation and the UDM subscribes to the UDR to get notifications of the modification on UE subscription data by invoking Nudr_DM_Subscribe service operation.

-	The NWDAF containing MTLF may consider the data quality into the accuracy monitoring by collecting fault prediction analytics data from MDAS to determine the status of Data Source NFs, using MDA Request.

If the NWDAF containing MTLF has already collected new test data and performed ML Model Accuracy Monitoring and retraining which is triggered by other NWDAF containing AnLF(s) (for ML Model Accuracy Monitoring and retraining), the NWDAF containing MTLF, based on its internal logic, determines whether to use the data for the subscription or not.

9a-9f.	The NWDAF containing MTLF receives the requested data from various sources as requested in steps 8a-8f.

10.	Based on the collected analytics and data from steps 9a-9f, the NWDAF containing MTLF computes the accuracy using the methods described in clause 5C.1. The NWDAF containing MTLF may discard data from data sources if it detects the data quality of that source is not good. The NWDAF containing MTLF may generate prediction with the collected input data to calculate the accuracy if only input data and ground truth data are available.

NOTE:	How the NWDAF containing MTLF determines whether the data from the data source is of good quality or needs to be discarded is up to the NWDAF implementation and configuration.

11.	An accuracy report is sent to the NWDAF containing AnLF, e.g. when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

12.	Based on the computed accuracy, the NWDAF containing MTLF may decide to re-train/re-provision the ML Model.

13.	When the newly generated or re-trained ML Model is ready, the NWDAF containing MTLF sends new or re-trained ML Model to the NWDAF containing AnLF by invoking Nnwdaf_MLModelProvision_Notify service operation. The NWDAF containing MTLF may send the accuracy report of the new or re-trained ML Model to the NWDAF containing AnLF.

# * * * * Next changes * * * *

7.5	Nnwdaf_MLModelProvision services

7.5.1	General

Service Description: This service enables the consumer to receive a notification when an ML Model matching the subscription parameters becomes available.

When the subscription is accepted by the NWDAF containing MTLF, the consumer NF, receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of ML Model subscription can be enforced by NWDAF based on operator policy and configuration.

7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe

Description: Subscribes to NWDAF ML Model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML Model subscription), ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2, NF consumer information, NF Instance ID of ML Model consumer, Vendor ID of ML Model consumer, Requested representative ratio, ML Model Reporting Information (including e.g. ML Model Target Period), Expiry time, Use case context, Inference Input Data information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information, Time when model is needed, ML Model Monitoring Information (including e.g. desired ML Model metric, ML Model monitoring reporting mode, ML Model Accuracy Threshold, DataSetTag and ADRF ID, ML Model identifier).

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

# * * * * Next changes * * * *

7.6	Nnwdaf_MLModelInfo service

7.6.1	General

Service description: this service enables the consumer to request and get from NWDAF containing MTLF ML Model Information.

7.6.2	Nnwdaf_MLModelInfo_Request service operation

Service operation name: Nnwdaf_MLModelInfo_Request

Description: The consumer requests NWDAF ML Model Information.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2.

Inputs, Optional:

-	For each Analytics ID the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data Information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-	NF consumer information.

-    NF Instance ID of ML Model consumer.

-    Vendor ID of ML Model consumer.

-	ML Model Accuracy Monitoring Information.

Outputs, Required: Set of:

-	the tuple (Analytics ID, one or more tuples of unique ML Model identifier and the information as defined in clause 6.2A.2.

Outputs, Optional: ML Model Accuracy Information.

# * * * * End of changes * * * *