3GPP TSG-WG SA2 Meeting #164	S2-2407915

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1338 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the misalignment on Policy control based on URSP awareness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eUEPO |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.288, the Analytic ID related to URSP awareness (i.e. PDU Session traffic analytics) will notify PCF about the unmatched traffic of the existing PDU Session(s), not the unexpected traffic. The PCF for the UE can know whether there is an existing PDU Session by existing means (e.g. via BSF or AMF/BSF event). And only part of the Traffic descriptors of the URSP rule can be used in the request (i.e. IP Descriptors, the Domain Descriptors or the Application Identifiers corresponding to the Application Descriptors). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align with TS 23.288 on the terms and add the same example in Policy control decisions based on awareness of URSP rule enforcement without UE assistance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignement on Policy control based on URSP awareness with TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.1.5.3	Policy control decisions based on awareness of URSP rule enforcement without UE assistance

The PCF for the UE may subscribe to Analytics ID "PDU Session traffic analytics" described in TS 23.288 [24] for traffic monitoring of known traffic in order to verify the the traffic running in the existing PDU Session(s) with respect to the provisioned URSP rule(s). If the PCF for the UE is notified by the NWDAF about traffic which is not expected according to any of the URSP rules, the PCF for the UE may adjust the URSP rule(s) related to unexpected application.

In detail, the PCF for the UE generate the input as described in clause 6.20.2 of TS 23.288 [24] and the NWDAF notifies the unmatched traffic as described in clause 6.30.3 of TS 23.288 [24].

During the request of Analytics ID "PDU Session traffic analytics", the PCF for the UE includes all the Traffic Descriptor of the URSP rules in the request of the analytic for the currently existing PDU Session(s) of the UE (only the IP Descriptors, the Domain Descriptors or the Application Identifiers corresponding to the Application Descriptors are applicable). The PCF sets Analytics Filter Information per currently existing PDU Session as following:

-	Area of Interest as the location of the UE where URSP rule enforcement is monitored;

-	S-NSSAI of the PDU Session;

-	DNN of the PDU Session.

# * * * * End of changes * * * *