3GPP TSG SA WG2 Meeting #164	S2-2407956

# 19 - 23 August, 2024, Maastricht, Netherlands				    		(Revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1417 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminating local BDC establishment without BDC media component in SDP of incoming INVITE request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2024-07-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 sent a LS (C1-243693) on local BDC setup on terminating side in case INVITE does not contain DC description. The scenario mentioned in the LS is when the terminating network receives a initial INVITE request without data channel media component in SDP, if the terminating UE has IMS data channel subscription, whether the terminating network can initiate the establishment of local bootstrap DC for terminating UE. In our opinion, the terminating network should initiate the establishment of local bootstrap DC by adding bootstrap DC media component towards terminating UE into the INVITE request, based on operator policy for the following reasons: - Since the terminating UE has IMS data channel subscription, the terminating network may need to provide applications for terminating UE during the session, e.g. translation applications, for which a local bootstrap DC targeting the terminating UE is needed. - If the network does not initiate the establishment of the bootstrap DC during the session establishment, the originating and terminating UEs have to further update the session with re-INVITE request to establish bootstrap DC, which probably happen simultaneously and cause unnecessary signaling and possible race condition between bootstrap DC establishment requests from originating UE and terminating UE. Based on the reason above, it is proposed to update the bootstrap DC establishment procedure in 23.228 to clarify the corresponding handling in terminating network, which is missing in current specification, and also allow the establishment of IMS session without DC media to be notified to DCSF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarifications to AC.4; Update bootstrap DC establishment procedure to clarify the handling in terminating network; Remove the restriction of session event notification and allows the session without DC media to be notified to DCSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.4, AA.2.4.2.2, AC.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# AC.4	IMS DC Channel Setup

The following principles apply when an IMS Data Channel is established: -

-	UE can establish an IMS Data Channel simultaneously while establishing an IMS audio/video session or upgrade an ongoing IMS audio/video session through a re-INVITE to an IMS Data Channel session.

NOTE:	An IMS Data Channel established simultaneously with an IMS audio/video session can have impact to the IMS session setup time.

-	The UE may be configured by the HPLMN either via Device Management or in the UICC whether and when to initiate a DC establishment request. If the UE is not configured by the HPLMN, it is left for UE implementation whether and when to initiate the DC establishment request. The UE shall not initiate a DC establishment request if the network has not indicated support for IMS DC service during IMS registration or if the UE is configured to not establish an IMS Data Channel.

-	If a UE that has not subscribed to IMS Data Channel establishes an IMS Data Channel simultaneously with an IMS audio/video IMS session, the IMS AS shall discard the Data Channel request and proceed with the audio/video IMS session establishment.

-	The DCSF may add an IMS Data Channel, e.g. bootstrap Data Channel, to a IMS session when the UE it serves has subscribed to IMS Data Channel and the DCSF decides to provide DC application to the UE, regardless of whether the original SDP containing Data Channel media or not.

-	The DCSF provides stream ID and a corresponding URL to the MF via the IMS AS during data channel media resource reservation for the bootstrap data channel that enables downloading a subscriber specific graphical user interface to the UE via the bootstrap data channel. The stream ID is optional. Once the MF, acting as HTTP proxy, receive a HTTP GET Request containing the root ("/") URL through the bootstrap data channel, the MF replaces the root ("/") URL in the HTTP GET Request with the subscriber specific URL corresponding to the stream ID (if available) of the bootstrap data channel and forward the request to the DCSF for downloading the subscriber specific graphical user interface to the UE. The MF retrieves the stream ID of the bootstrap data channel from the transport layer of the data channel connection. The UE shall maintain the HTTP session state when interacting with the MF.

**** Second Change ****

AC.7.1	Bootstrap Data Channel Setup Signalling procedure

Figure AC.7.1-1depicts a signalling flow diagram for establishing a bootstrap data channel in a person-to-person use case. The MF anchors the bootstrap data channel, and the originating network is offering a bootstrap data channel to the remote peer as well for application download.

In the call flow the two UEs firstly establish an IMS audio session, and the originating UE further updates the IMS audio/video session with IMS data channel media components.

NOTE 1:	Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

Figure AC.7.1-1: Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

- UE#1 initiates an IMS session with audio/video media components to UE#2.

- The INVITE request is sent to the terminating network and to the IMS AS#2.

- The IMS AS#2 decides to notify the event of session establishment to the DCSF based on the IMS DC subscription of the UE#2 and discovers a DCSF.

- The IMS AS#2 notifies the DC call event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, Session Case, Event initiator, Media InfoList) request to the DCSF#2.

- The DCSF#2 determines whether the local bootstrap data channel for terminating UE is needed based on operator policy and/or service policy. If the local bootstrap data channel for terminating UE is needed, step 6-11 apply.

- The DCSF#2, based on its policies, generates the media information of terminating side MDC1 for remote bootstrap media (targeting terminating UE), which are used to receive UE#2 request for application downloading from MF.

- The DCSF#2 invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up local bootstrap data channel for terminating UE with MF. The MediaInstructionSet provided by the DSCF#2, includes its MDC1 media endpoint addresses created in step 6, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

- The IMS AS#2 selects a MF based on local configuration or discovers and selects a MF instance via NRF.

- The IMS AS#2 invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources. IMS AS request creation of the Media Termination for the local bootstrap media to be offered to UE#2. Each Media Termination includes information required to allocate resources in both Mb and the MDC1 interfaces. The MF responds with the reserved data channel media resource information to IMS AS.

- The IMS AS#2 responds to the MediaInstruction request received in step 9. The response may include the atomic success result of operation and also includes reserved data channel media resource information for MDC1.

- The DCSF#2 stores the media resource information and responds to the Notify Request received in step 4.

12-13.	The IMS AS#2 sends the INVITE request to UE#2. If step 6-11 apply, the IMS AS may add the SDP offer of local bootstrap DC for terminating UE#2 into INVITE request.

14.	UE#2 and terminating network returns an 18X response with the SDP answer to originating network.

15.	If the local bootstrap data channel for terminating UE is established, the UE#2 downloads the DC application list and applications.

16.	UE#1 sends the SIP re-INVITE request with the updated SDP to the IMS AS#1, through P-CSCF#1 and S-CSCF#1 in the originating network. The updated SDP contains offers for the bootstrap data channel establishment request with bootstrap DC stream ID. In this example procedure, the SDP contains both bootstrap data channel offers for originating side and terminating side.

NOTE 2:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup. The SIP re-INVITE can be initiated either from UE#1 or UE#2.

17.	IMS AS#1 validates user subscription data to determine whether the data channel call request should be notified to DCSF.

If the IMS AS#1 determined, based on the user profile, that data channel call request needs to be notified to DCSF, the IMS AS#1 selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS#1 determined, based on the user profile, that the data channel call request need not to be notified to DCSF, or DCSF decides that DC request is not allowed, the IMS AS#1 proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel bootstrapping, by deleting DC related media information and sending the updated SIP INVITE to the originating S-CSCF.

18.	IMS AS#1 notifies the DCSF#1 of the DC call event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, Session Case, Event initiator, Media InfoList, DC Stream ID) request to the DCSF#1.

19.	After receiving the DC control request, the DCSF#1 determines the policy about how to process the bootstrap data channel establishment request based on the related parameters in the Data Channel control request (e.g. CallingID, CalledID, DC Stream ID) and/or DCSF service specific policy.

20.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF#1, based on its policies reserves originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE), which are used to receive UE request for application downloading from MF.

21.	DCSF#1 invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS#1 how to set up bootstrap data channel with MF#1 both for originating and terminating side. The MediaInstructionSet provided by the DSCF#1, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

In this scenario, the DCSF#1 instructs the IMS AS#1 to terminate bootstrap data channel establishment request on originating MF, and initiate remote bootstrap data channel establishment request(targeting remote UE) as well as forwarding remote bootstrap data channel establishment request of served user (targeting remote DCSF) towards terminating network.

22.	The IMS AS#1 selects a MF based on local configuration or discovers and selects a MF instance via NRF.

23.	IMS AS#1 invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF#1 to allocate required data channel media resources. IMS AS#1 request creation of two different Media Terminations, one representing the local bootstrap media to be terminated and the other representing the remote bootstrap media to be offered to remote UE. Each Media Termination includes information required to allocate resources in both Mb and the MDC1 interfaces. The MF#1 responds with the negotiated data channel media resource information to IMS AS.

NOTE 3:	The MF media resource allocation in step 23 could be done with one or multiple service invocations.

24.	IMS AS#1 responds to the MediaInstruction request received in step 21. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1.

25.	The DCSF#1 stores the media resource information and responds to the Notify Request received in step 18.

26-27.	IMS AS#1 sends the re-INVITE which includes the updated SDP offer adding media information of MF via the originating S-CSCF to remote network side and UE#2. In this scenario, the SDP offer for bootstrap data channel to UE#2 is included.

28.	The terminating network reserves the added DC media resource as described in step 4-11.

29-30.	the IMS AS#2 sends re-INVITE request to UE#2 via S-CSCF#2 and P-CSCF#2.

31:	UE#2 and terminating network returns a 200 OK response.

32.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

33.	The DCSF responds to the Nimsas notification request.

34.	200 OK forwarded to UE#1 which indicates the bootstrap data channel has been established.

35.	The originating network P-CSCF executes QoS procedure for bootstrap data channel as specified in TS 23.203 [54] and TS 23.503 [95].

36-39:	The bootstrap data channels have been established between originating MF and UE#1/UE#2. The UEs send application request messages to MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 and UE#2 based on their data channel capabilities and their choices through MF.

The bootstrap data channels have also been established between terminating MF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF

NOTE 4:	IMS-AGW needs to allow the establishment of bootstrap data channels based on the information in the 200 OK to PRACK and UPDATE messages.

40.	Subsequent procedures continue.

NOTE 5:	In the DC application list, the DCSF may provide DC applications supported by both UEs, or only supported by the UE who sends the application request message. The details of how to provide the application list to the UE and how to use it by the UE are not defined in this specification.

**** Third Change ****

AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |

**** End of Changes ****