3GPP TSG-SA2 Meeting #164	S2-2407957

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5491 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support UE-SAT-UE communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 5GS supporting UE-sat-UE has been concluded in TR 23.700-29, by which general description is needed to be added to TS 23.501 based on the conclusions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add general description of UE-SAT-UE communication feature based on the conclusions in TR 23.700-29. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE-SAT-UE communication features is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

5.4.x	Support of UE-Satellite-UE communication

5.4.x.1	General

The present clause 5.4.x provides the specific aspects to support UE-Satellite-UE communication.

The UE-SAT-UE communication in a 5G system with satellite access refers to the direct communication between UEs that are under the coverage of one or more serving satellites. In this type of communication, the user traffic is routed exclusively through the satellite network, bypassing the terrestrial ground segment. A connection to the ground is anticipated to be consistently available, whether it is through an ISL to another satellite that has a ground gateway connection, through multiple satellites using ISLs to reach a satellite with a ground gateway connection, or directly from the serving satellite to a ground gateway.

To support UE-SAT-UE communication, in terms of architecture, the gNB and UPF are depolyed onboard the satellite, it is optional whether the IMS-AGW is deployed on the satellite. The ground UPF is selected acts as an PSA UPF during the IMS PDU session, and the onboard UPF is inserted as UL-CL (local PSA) for UE-satellite-UE communication procedure during the call setup procedures. When there is no IMS-AGW onboard the satellite, an IP over IP tunnel over the N6 interface can be used between the onboard local UPF on the MO UE side and the onboard local UPF on the MT UE side. The decision to use the AGW on the ground or the onboard UPF for an IMS call, when AGWs are not placed on the satellite, is left to the operator's policy.

5.4.x.2	UPF and IMS-AGW onboard the satellites

In case both UPF and IMS-AGW onboard the satellites, P-CSCF shall select the IMS-AGW onboard the satellite based on the satellite ID of the serving satellite connected by UE and access network information.

When the serving satellite changes, an IMS-AGW relocation procedure is necessary to route the UE-satellite-UE traffic to the new IMS-AGW. The simultaneous uplink/downlink change caused by the satellite switch can be synchronized with the AGW relocation to prevent packet loss.

* * * End of Change* * *