SA WG2 Meeting #164	S2-2407970

August 19 – 23, 2024, Maastricht, The Netherlands

Source:	Ericsson

Title:	On the handling of Coarse Location Information provided by the UE

Document for:	Approval

Agenda Item:	8.27

Work Item / Release:	IoT_SAT_ARCH_EPS / Rel-18

Abstract of the contribution: This contribution discusses topics related to the handling of Coarse Location Information provided by the UE.

# Introduction

At SA2#161 (Athens), it was agreed that MME can request the UE to provide Coarse Location Information (CLI) via NAS, and the UE then provides the CLI to the MME. An LS was sent to RAN2 and RAN3 asking whether it would be beneficial to provide the CLI to RAN.

RAN2 provided a reply in R2-2403770/S2-2405879 indicating that "from RAN2 perspective, even if not essential it may be useful for the eNB if the MME further signals the coarse location information received from the UE in NAS back to eNB".

RAN3 has discussed the topic but not yet provided a reply.

Observation 1: It is not essential for RAN2 to get the coarse UE location information. RAN3 has not yet replied. Since this is for Rel-18, only essential corrections should be made. Therefore, SA2 should postpone discussions until RAN3 has provided a reply.

At SA2#163 (Jeju), even though there was no reply from RAN3, there were company proposals to enhance 23.401 to provide Coarse Location information to RAN in the Location Reporting procedure and to reply to RAN2/RAN3. The discussion was postponed. It is not clear whether SA2 will receive a reply from RAN3 at the SA2#164 (Maastricht), but since there were input papers to SA2#163 (Jeju) we provide this DP with our view from SA2 perspective.

# Discussion

#### Use cases and requirements.

To discuss new functionality related to Coarse Location Information, it should be clear what the use case and requirements are.

A purpose that has been discussed in SA2 is that eNB, if it knows the CLI, could provide an updated Mapped Cell ID and possibly an updated UE location derived TAC based on the CLI. This could e.g. be a more accurate Mapped Cell ID. Below we analyze what issues an updated CLI may solve, if any:

- 	Location verification should not be an issue since that was addressed based on the solution already defined. I.e. there is nothing more to be done.

- 	Emergency should not be an issue since it is not supported for NB-IoT.

- 	LI may need access to a detailed UE location information, but this is mainly for discussion in SA3-LI.

- 	Charging: Location based charging may be a use case, but this does not seem to be CAT F.

- 	Policy control: Location based QoS control may be a use case, but this does not seem to be CAT F.

- 	Paging assistance: A more accurate Mapped Cell ID could potentially be used for paging assistance information. However, this would be an optimization, does not seem to be CAT F and it should anyway be driven from RAN WGs.

- 	Location services in general should not be an issue. This NOTE was agreed in Athens (23.401, clause 4.13.4):

"NOTE 1:	Provisioning of coarse UE location information in the SMC procedure as described above is intended to enable basic support for UE location verification. If additional accuracy or other positioning features are needed it is assumed that the UE and the network support LPP and the procedures defined in TS 23.271 [57]."

Observation 2: There does not seem to be any CAT F use case from SA2 point of view that justifies further SA2 work for providing CLI to RAN.

#### How would an updated ULI be handled by MME?

Even if there was a use case from SA2 perspective, one should analyse how a solution could work e2e. None of the papers provided in Jeju explain this. The proposals submitted to Jeju were simply that MME sends the CLI in a Location Reporting Control message, allowing the RAN to use it for reasons not discussed in SA2 and/or to provide an updated (ideally more accurate) ECGI + TAI in the Location Report message. But how this is then handled by MME in a wider perspective were not discussed or described.

With these proposals, the MME will have to handle different "kinds" of TAI and CGI. When RAN does not (yet) know the CLI, e.g. in the Initial UE or first set of UL NAS Transport messages, the MME will receive initial TAI and ECGI in S1AP messages. For simplicity, this initial CGI+TAI is hereafter referred to as Initial ULI (I-ULI). Then the MME could trigger NAS SMC procedure to request Coarse UE location from the UE, and after that provide the CLI in the Location Reporting Control message to eNB and get a new updated ULI in the Location Report message (hereafter called U-ULI).

Issue 1: MME does not know when to ask the UE for CLI.

In case the CLI can be provided to RAN via RRC (AS layer), e.g. in NR NTN, RAN can, based on implementation, have logic to determine when to ask the UE for the CLI again. This could e.g. be based on measurements, cell changes etc. The CLI may also be reported by the UE together with measurement results, i.e. using no extra RRC messages for providing the CLI. However, in case the CLI is provided via NAS, the MME has no reasonable way to determine when to again ask the UE for the CLI and it will also result in extra signalling.

The MME may e.g. request the CLI from the UE only during a subset of the interactions. In the latter case, the MME may e.g. based on implementation decide to ask the UE for CLI e.g. in every Attach and at certain TAU. At other times (e.g. SR or MO/MT CP data) the MME does not trigger SMC to get coarse UE location. In this case, the ULI in MME may be a I-ULI at some times and a U-ULI at other times. This is thus not a complete solution to the problem.

Alternatively, the MME will at every new interaction with the UE trigger the NAS SMC procedure to get the coarse UE location and then provide it to RAN to get a more accurate cell ID. However, this causes a lot of extra signalling load (N1 and N2) which may also cause problems for power saving in NB-IoT use cases.

Issue 2: Interaction with NAS procedures

Timing is also an issue. If the MME sends a Location Reporting Control S1AP message with CLI during a TAU or Attach procedure, the MME needs to pause that procedure until RAN has sent a Location Report message in order to be able to provide the U-ULI instead of I-ULI to e.g. PGW/PCRF. The S1AP Location reporting messages are Class 2 S1AP messages and even if RAN should promptly send a Location Report to MME there is no timing constraints, and the MME will need a guard timer to avoid that the NAS procedure times out while waiting for the U-ULI.

It is also not clear that it is feasible to retrieve the CLI and provide it to RAN in all NAS procedures. E.g. the EDT MO data procedures for CP CIoT cannot easily be extended with an SMC procedure. In those cases, the MME would need to page the UE and then execute SMC, causing additional overhead. However, even in that case, at the next MO CP data, RAN would again provide a I-ULI unless RAN somehow caches the UE location for IDLE mode UEs.

If the MME will not be able to execute SMC at each interaction with the UE, after a NAS procedure, the MME will thus sometimes have a I-ULI and sometimes a U-ULI as the last know ULI. This raises several questions:

- 	What ULI is provided in location reports to PGW/PCRF or in monitoring event reports to SCEF? Would MME always use the last known U-ULI, even if there is a more recent I-ULI received from RAN?

- 	What ULI (ECGI and TAI) is kept by MME in the Last known ECGI and Last known TAI in the UE context? Will the MME keep the last known I-ULI only, or will it keep the last ULI provided by RAN irrespective of whether eNB has the CLI?

As a summary, the proposal seems to result in that the MME needs to differentiate two types of CGIs: “U-CGIs” that refers to CGIs that RAN generated based on coarse location info received from MME and “I-CGIs” that RAN generated without coarse location info. The solution will never be able to get rid of the latter as it will be arriving e.g. in Initial UE message and many UL NAS Transport messages. MME will also not be certain which one is which, e.g. if RAN sends a Location Report with CGI1, and 5 minutes later sends a UL NAS message with another CGI2 it is not clear whether this CGI2 should be used instead if CGI1. There is also a risk that the approach significantly increases the N1 and N2 signalling load.

An option may be that MME is configured to (depending on RAT type, TAI etc) always ignore the location info received in Initial UE and UL NAS messages and only use the location info from Location Report after having sent a CLI in a Location Reporting Control. However, such approach requires further evaluation, study and discussion.

Observation 3: This is an incomplete solution that causes ambiguities, complexities and new functionality in MME (and RAN). It is also not CAT F.

#### Alternative ways forward

If the purpose is to provide the LI systems with more accurate UE location info, it may be better to enhance LI interfaces to include CLI. That would be more explicit and clear and also more signalling effective than deriving a new Mapped Cell ID. This is an enhancement that could be specified by SA3-LI.

Another option that was proposed in one paper in Jeju is that the E-SMLC maps the CLI to a new Mapped Cell ID and provides it to MME. That may be possible, but this new Cell ID should be clearly separated from the RAN-provided Cell ID, i.e. MME should not replace the RAN-provided Cell ID with an E-SMLC derived Cell ID. The RAN provided Cell ID should remain RAN provided, e.g. as its derivation can take also other (radio related) info into account while E-SMCL derived Cell ID would be explicitly based on UE-provided information and thus be of different nature. In this option, the MME should instead provide the E-SMCL-derived Cell ID as an additional Cell ID (already possible with some LCS procedures today, where E-SMCL provides the last know Cell ID it has). However, this has impacts to several NFs and interfaces.

# Proposal

From SA2 perspective, there is no further FASMO work needed in rel-18 for handling of Coarse Location Information. Potential extension of the LI interfaces to carry Last know Coarse Location Information can be discussed in SA3-LI. If RAN3 decides to add the CLI to S1AP due to RAN related use cases, SA2 can align with RAN3.