3GPP TSG-SA2 Meeting #164	S2-2407992

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | 0493 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of MPS for Messaging enabling via SCEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS4msg |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | An SCS/AS needs to be able to set (enable)/clear (disable) the MPS for Messaging indication in the HSS via the T8 interface and the SCEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First Change: Adds MPS for Messaging to list of SCEF capabilities. Second Change: Adds "No MPS subscription" event monitor for MPS for Messaging to list of event monitors. Third Change: Adds general description clause for MPS for Messaging via the SCEF. Fourth Change: Adds a monitor clause for "No MPS subscription", so that the SCS/AS can inform the user if the MPS subscription is removed from the system. Fifth Change: Adds "No MPS subscription" event reporting clause. Sixth Change: Adds "No MPS subscription" event reporting for roaming clause. Seventh Change: Adds procedure for an SCS/AS to set (enable)/clear (disable) the MPS for Messaging indicator in the HSS. The HSS must check whether the UE has an MPS subscription in the HSS before allowing the MPS for Messaging indicator to be set (enabled). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPS for Messaging feature will not be able to be set (enabled) or cleared (disabled) via a SCEF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8, 4.5.6.1, 4.5.W (new), 5.6.1.X (new), 5.6.3.V (new), 5.6.8.Y (new), 5.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ***** First change *****

### 4.4.8	Service Capability Exposure Function

The Service Capability Exposure Function (SCEF) provides a means to securely expose the services and capabilities provided by 3GPP network interfaces. The SCEF provides a means for the discovery of the exposed services and capabilities. The SCEF provides access to network capabilities through homogenous network application programming interfaces (e.g. Network APIs) defined over T8 interface. The SCEF abstracts the services from the underlying 3GPP network interfaces and protocols.

Individual instances of SCEF may vary depending on what service capabilities are exposed and what API features are supported.

The SCEF is always within the trust domain. An application can belong to the trust domain or may lie outside the trust domain.

The SCEF may support CAPIF. When CAPIF is supported, the SCEF supports the CAPIF API provider domain functions. The CAPIF and associated API provider domain functions are specified in TS 23.222 [43].

The functionality of the SCEF may include the following:

-	Authentication and Authorization:

-	Identification of the API consumer,

-	Profile management,

-	ACL (access control list) management.

NOTE 1:	The details of security aspects of T8 interface are outside the scope of this specification.

-	Ability for the external entities to discover the exposed service capabilities

-	Policy enforcement:

-	Infrastructural Policy: policies to protect platforms and network. An example of which maybe ensuring that a service node such as SMS-SC is not overloaded.

-	Business Policy: policies related to the specific functionalities exposed. An example may be number portability, service routing, subscriber consent etc.

-	Application Layer Policy: policies that are primarily focused on message payload or throughput provided by an application. An example may be throttling.

-	Assurance:

-	Integration with O&M systems,

-	Assurance process related to usage of APIs.

-	Accounting for inter operator settlements.

NOTE 2:	The details of accounting aspects of T8 interface are outside the scope of this specification.

-	Access: issues related to external interconnection and point of contact

-	Abstraction: hides the underlying 3GPP network interfaces and protocols to allow full network integration. The following functions are among those that may be supported:

-	Underlying protocol connectivity, routing and traffic control,

-	Mapping specific APIs onto appropriate network interfaces,

-	Protocol translation.

NOTE 3:	Abstraction is applied only in cases where required functionality is not natively provided by 3GPP network

The services and capabilities offered by SCEF to SCS/AS include:

-	Group Message Delivery (see clause 4.5.5),

-	Monitoring events (see clause 4.5.6),

-	High latency communication (see clause 4.5.7),

-	Informing about potential network issues (see clause 4.5.8),

-	Resource management of background data transfer (see clause 4.5.9),

-	E-UTRAN network resource optimizations based on communication patterns provided to the MME (see clause 4.5.10),

-	Support of setting up an AS session with required QoS (see clause 4.5.11),

-	Change the chargeable party at session set-up or during the session (see clause 4.5.12),

-	Non-IP Data Delivery (see clause 4.5.14),

-	Packet Flow Description management (see clause 4.5.15),

-	Enhanced Coverage restriction control (see clause 4.5.17),

-	Network Parameter Configuration (see clause 4.5.21),

-	MPS for Messaging (see clause 4.5.W),

-	Accessing MTC-IWF Functionality via T8 (see clause 5.17).

The SCEF shall protect the other PLMN entities (e.g. HSS, MME) from requests exceeding the permission arranged in the SLA with the third-party service provider.

When needed, the SCEF supports mapping between information exchanged with SCS/AS (e.g. geographical identifiers) and information exchanged with internal PLMN functions (e.g. cell-Id / ENB-Id / TAI / MBMS SAI, etc.). This mapping is assumed to be provided by the SCEF based on local configuration data.

# ***** Second change *****

#### 4.5.6.1	General

The Monitoring Events feature is intended for monitoring of specific events in 3GPP system and making such monitoring events information available via the SCEF. It is comprised of means that allow the identification of the 3GPP network element suitable for configuring the specific events, the event detection, and the event reporting to the authorised users, e.g. for use by applications or logging, etc. If such an event is detected, the network might be configured to perform special actions, e.g. limit the UE access. Configuration and reporting of the following monitoring events may be supported:

-	Monitoring the association of the UE and UICC and/or new IMSI-IMEI-SV association;

-	UE reachability;

-	Location of the UE, and change in location of the UE;

NOTE 1:	Location granularity for event request, or event report, or both could be at cell level (CGI/ECGI), TA/RA level or other formats e.g. shapes (e.g. polygons, circles, etc.) or civic addresses (e.g. streets, districts, etc.).

-	Loss of connectivity;

-	Communication failure;

-	Roaming status (i.e. Roaming or No Roaming) of the UE, and change in roaming status of the UE; and

NOTE 2:	Roaming status means whether the UE is in HPLMN or VPLMN based on the most recently received registration state in the HSS.

-	Number of UEs present in a geographical area;

-	Availability after DDN failure;

-	PDN Connectivity Status; and

-	No MPS subscription.

To support monitoring features in roaming scenarios, a roaming agreement needs to be made between the HPLMN and the VPLMN. The set of capabilities required for monitoring may be accessible via different 3GPP interfaces/nodes. Selection of 3GPP interface(s) to configure/report the event is dependent on the type of the event, operator configuration, required frequency of event reporting, application provided parameters in monitoring event request, etc.

Support for Monitoring Events can be offered either via HSS, MME/SGSN (as described in clause 4.5.6.2) or via PCRF (as described in clause 4.5.6.3). Based on operator policies, it shall be possible to configure Monitoring Events such that some Monitoring Event follows procedures in clause 4.5.6.2 while another Monitoring Event follows procedures in clause 4.5.6.3. SCEF shall not enable a given Monitoring Event for the same UE via both HSS/MME/SGSN, and PCRF. For the case of group based Monitoring Events, the SCS/AS (either the same SCS/AS or different SCSs/ASs) may configure a Monitor Event with different External Group Identifiers. If, in such a case, more than one External Group Identifier point to the same UE and no Group Reporting Guard Time was provided with any of the monitoring event configurations, the MME, HSS, and SCEF should not send duplicate reports of the same event for the same UE to the same destination.

NOTE 3:	If the configuration of Monitoring Events uses signalling which was specified as part of another feature than the Monitoring feature, then the requirements on the HSS, MME/SGSN and PCRF as specified by that feature apply e.g. not to generate accounting information, not to verify SLA etc.

# ***** Third change *****

### 4.5.W	Support for MPS for Messaging via SCEF

MPS for Messaging is specified in TS 23.401 [7], clause 4.3.18.X.

As an optional feature, an SCS/AS may interface with the SCEF via T8 to enable the setting (enabling)/clearing (disabling) of MPS for Messaging for a UE. The SCEF relays the setting (enabling)/clearing (disabling) of MPS for Messaging with the HSS via S6t. The HSS stores the value (set [enable]/clear [disable]) of MPS for Messaging.

The SCS/AS is under the control of the mobile network operator. Authentication for using the SCS/AS for MPS for Messaging is out of scope of 3GPP. The SCEF may support functionality to control the frequency of set (enable)/clear (disable) requests via the SCS/AS.

The HSS and SCEF may support reporting to the SCS/AS. If the HSS has an MPS subscription for the UE, the HSS may return a success message to the SCEF and the SCEF may return a success message to the SCS/AS. If the HSS does not have an MPS EPS priority subscription for the specific UE upon a request to set (enable) MPS for Messaging, the HSS may return a message to the SCEF and then the SCEF may return a message to the SCS/AS that there is no MPS subscription and that MPS for Messaging is not allowed.  When the MPS EPS priority subscription is removed from the HSS, while MPS for Messaging is set (enabled), the HSS changes the MPS for Messaging to clear (disable) and may report this event to the SCEF, which may report this to the SCS/AS.

When MPS for Messaging is set (enabled), consuming network functions (e.g., MME, IP-SM-GW, SMS-MSC) can use this indication to treat messages as per TS 23.401 [7] with MPS priority. When MPS for Messaging is cleared (disabled), consuming network functions may use this indication to treat messages without MPS priority.

# ***** Fourth change *****

### 5.6.1.X	Specific Parameters for Monitoring Event: No MPS subscription

The 3GPP network can detect that the UE does not have/no longer has an MPS EPS priority subscription. This indication may be shared with an SCS/AS.

The operator may have controls for setting (enabling) this event that are outside the scope of 3GPP. For example: the operator might want to control the frequency with which MPS for Messaging is set/cleared.

1.	The SCS/AS sets Monitoring Type to "No MPS subscription" in the Monitoring Request to the SCEF as in step 1 of clause 5.6.1.1.

2.	The SCEF executes step 2 of clause 5.6.1.1.

3.	The SCEF executes step 3 of clause 5.6.1.1.

4.	The HSS executes step 4 of clause 5.6.1.1, immediately, or when the event occurs while MPS for Messaging is set (enabled). The HSS uses the non-presence/removal of MPS EPS priority subscription in the HSS as the basis for this event report.

# ***** Fifth change *****

#### 5.6.3.V	Reporting Event: No MPS Subscription Reporting

1b.	This monitoring event is detected as of step 1b of clause 5.6.3.1, which is when the MPS EPS priority subscription in the HSS for the UE changes.

2.	Step 2b of clause 5.6.3.1 is executed. The Monitoring Event Report indicates that the UE has no MPS subscription.

3.	Step 3 of clause 5.6.3.1 is executed.

# ***** Sixth change *****

#### 5.6.8.Y	Reporting Event: No MPS Subscription Reporting

1b.	This monitoring event is detected as of step 1b of clause 5.6.8.1, which is when the MPS EPS priority subscription in the HSS for the UE changes.

2.	Step 2b of clause 5.6.8.1 is executed. The Monitoring Event Report indicates that the UE has no MPS subscription.

3.	Step 3 of clause 5.6.8.1 is executed.

# ***** Seventh change *****

### 5.Z	MPS for Messaging procedure

Figure 5.Z-1: Procedure for MPS for Messaging via SCEF

1.	The SCS/AS sends a T8 MPS for Messaging Request (External Identifier or MSISDN, SCS/AS Identifier, set (enable)/clear (disable) Indication, the MTC Provider Information) message to the SCEF.

2.	Based on operator policies, if either the SCS/AS is not authorized to perform this request, or the request is malformed, the SCEF skips steps 3-5 and provides a Cause value appropriately indicating the error.

3.	The SCEF sends an MPS for Messaging Request (External Identifier or MSISDN, SCEF ID, set (enabled)/clear (disabled)) to the HSS. The HSS checks whether the UE has a subscription for MPS (i.e. according to its MPS profile) as specified in TS 23.401 [7].

4.	The HSS examines the MPS for Messaging Request message, e.g., with regard to the existence of External Identifier or MSISDN, the set (enabled)/clear (disabled) indication, and completeness of the request. The HSS checks whether the UE has a subscription for MPS (i.e. according to its MPS profile) as specified in TS 23.401 [7]. The UE shall have a subscription for MPS in order to be authorized to set (enable) MPS for Messaging. If these checks fail, the HSS provides a Cause value indicating the reason for the failure condition to the SCEF.

5.	The HSS sends MPS for Messaging Response (SCEF Reference ID, SCEF Reference ID for Deletion, Cancelled SCEF Reference ID, Cause) message to the SCEF to acknowledge acceptance or indicate the rejection of the MPS for Messaging Request.  If the UE has no MPS subscription, the Cause reflects that.

6.	The SCEF sends a T8 MPS for Messaging Response (Cause) message to the SCS/AS to acknowledge acceptance or indicate the rejection of the T8 MPS for Messaging Request. If the UE has no MPS subscription, the Cause reflects that.

# ***** End of changes *****