3GPP TSG-SA2 Meeting #164	S2-2408001

# Maastricht, Aug 19-Aug 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1145 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sample and feature alignment procedure for VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Futurewei, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-01-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR aims at specifying procedures for sample and/or feature alignment as concluded in TR 23.700-84 clause 8.2. In the conclusion, it is captured in P#2.3.2 that in case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g., internal versus external information) , then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training, and in P#2.3.4: Feature description information may be registered to the NRF or locally configured in the NWDAF or the AF, negotiated between VFL server and VFL client when performing feature alignment. Feature alignment is optional in a VFL process. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2x.2 is added with new procedures for sample and feature alignment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In VFL, it is required that VFL server and VFL clients have same sample and different features. Existing procedures are not enough to ensure same samples and different features are used in VFL. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * * (All new text)

### 6.2X.2	Procedures

### 6.2X.2.X	Sample and feature alignment procedure for Vertical Federated

### Learning between VFL server NWDAF and VFL client NWDAFs or AF

6.2X.2.X-1: Sample and feature alignment procedure for Vertical Federated Learning between VFL server NWDAF and VFL client NWDAFs or AF

1a.   VFL server NWDAF may determine a list of target samples and required features for VFL training. The criteria used at this step may include pre-configured sample and feature information, Analytics ID of the requested ML model, data features for which VFL server owns data labels, corresponding NF type(s) or instances of the requested analytics ID, recent data collection operations and/or ML Model Interoperability information.

1b-c.  [conditional] If pre-configured information and criteria in step 1a are not sufficient to determine the list of target samples and required features, VFL server NWDAF may collect input data and determine the list of target samples and required features based on collected data.

2.  VFL server NWDAF sends a sample and feature alignment request with the list of required features and target samples to VFL clients NWDAF and/or AFs. Existing Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operations may be enhanced to include feature and sample information for interactions between VFL server NWDAF and VFL client NWDAFs.

NOTE 1: Sending the list of required features is optional.

NOTE 2: NEF will be involved if the VFL client AF is an untrusted AF.

- Each VFL client NWDAF or AF may determine a list of supported features (a subset of  the list of required features shared by VFL server NWDAF) and a list of supported samples (a subset of target samples shared by VFL server NWDAF) based on the list of required features and target samples received from the VFL server NWDAF, pre-configured information, its available data, corresponding NF type(s) or instances of the requested analytics ID, recent data collection operations, and ML Model Interoperability information.

3b-c.  [conditional] If pre-configured information and criteria in step 3a are not sufficient to determine the list of supported samples and features, VFL client NWDAF or AF may collect input data and determine the list of supported samples and features based on collected data.

- Each VFL client NWDAF or AF may send the list of their supported features and samples to VFL server NWDAF.

NOTE 3: Sending the list of supported features is optional.

- VFL server NWDAF may perform sample alignment by identifying overlap/intersection of supported samples of all VFL client NWDAFs or AFs. A VFL client NWDAF or AF may be excluded from VFL training if the list of its supported samples has zero or very little overlap with supported samples of other VFL client NWDAFs or AFs.

- [optional] Based on supported feature of each NWDAF VFL client NWDAF or AF (feature dimension of each NWDAF VFL client NWDAF or AF), NWDAF VFL server NWDAF may decide how to partition data features between NWDAF VFL clients NWDAFs or AFs and assigns a subset of features to each NWDAF VFL client NWDAF or AF. A VFL client NWDAF may be excluded from VFL training if the list of its supported feature has zero overlap with the list of required features.

* * * * Second Change * * * * (All new text)

### 6.2X.2.X	Sample and Feature alignment procedure for Vertical Federated Learning with AF as the VFL server

Figure 6.2X.2.X-1: Sample and Feature alignment procedure for Vertical Federated Learning with AF as the VFL server

0. VFL Server AF selects NWDAF(s) containing MTLF (VFL Client NWDAF(s)) as described in clause X.

1.	VFL server AF sends the VFL sample and feature alignmemnt request to the NEF using the Nnef_VFLSampleFeatureAlignment_request including the selected VFL client NWDAF ID(s), Analytics ID, the target samples (i.e. external UE IDs), and optionally, the feature profile (including features per each sample) indicates what features the AF can provide (i.e., a list of VFL server AF supported features) and what features are needed for performing the VFL model training (i.e., a list of required features for VFL tarining).

NOTE 1:	NEF is needed only when the AF is untrusted AF.

Editor's note:	Whether the feature profile is a part of the ML Model Interoperability Information is FFS.

2.	NEF maps the external UE IDs to the internal UE IDs (e.g. GPSI to SUPI).

3.	NEF sends the VFL sample and feature alignment request to the VFL client NWDAF(s) using Nnwdaf_MLModelProvision_Subscribe including the analytics ID, the internal UE IDs (e.g. SUPIs), and feature profile. If multiple NWDAFs involved, the NEF sends the VFL sample and feature alignment request to each VFL client NWDAF, respectively.

Editor's note:	Whether the existing or new service operation will be used to support the sample and feature alignment between NEF and NWDAF is FFS.

4.	[Optional] Based on the feature profile and requested analytics ID, each VFL client NWDAF may collect input data by using the current mechanism in clause 6.2 if local data is not currently available at the VFL Client NWDAF.

5.	Based on the available local data, each VFL Client NWDAF may determine whether it can support target samples and required features provided by the VFL server AF. Each VFL client NWDAF may down-select UEs based on its served UEs to ensure the samples are same among VFL client NWDAF(s) and VFL server AF as requested by the VFL server AF. The supported features of each VFL client NWDAF may also a subset of required features but different with supported features of the VFL server AF as requested by the VFL server AF. Each VFL Client NWDAF sends the sample and feature alignment response using Nnwdaf_MLModelProvision_Notify to the NEF including the samples and optionaly the features that each VFL Client NWDAF can support.

6.	If multiple VFL Client NWDAFs involved, the NEF collects the sample and feature alignment response from each VFL Client NWDAFs. NEF may generate the intersection of samples and maps the intersection of samples related UE IDs to the external UE IDs.

7.	NEF sends the VFL sample and feature alignment response to the VFL server AF using the Nnef_VFLSampleFeatureAlignment_response including the intersection of samples, the feature supported by each VFL client NWDAF and the analytics ID.

8.	The VFL server AF determine the final VFL client NWDAFs with the selected samples and features. If the VFL server AF determines that there is an insufficient quantity of samples or features, it can stop the sample and feature alignment procedure. Subsequently, it has the capability to initiate a new sample and feature alignment procedure by adjusting the sample and feature requirements.

9.	[Optional] When the selected VFL client NWDAF need update, the VFL server AF evaluates whether sample and feature realignment is needed for the ongoing VFL procedure. If necessary, the VFL server AF initiates the new sample and feature alignment procedure.

* * * * End Change * * * *