SA WG2 Meeting SA2#164	S2-2408002

19-23, August 2024, Maastricht, NL

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5501 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture for UE Satellite UE Communicatiions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been agreed in TR 23.700-29 that to support UE-Satellite-UE Communications - Only IMS voice/video services are considered in this release. - Both UEs are served by the same HPLMN. - A ground UPF is selected and acts as an PSA UPF during the IMS PDU session, and the onboard UPF is inserted as UL-CL (local PSA) for UE-satellite-UE communication procedure during the call setup procedures. - During call setup, P-CSCF will determine the possibility of UE-Sat-UE call based on, offeror's satellite ID and access network information. - P-CSCF shall provision the AGW based on the above information received from PCF and UE. - When no AGW is onboard, e.g. an IP over IP tunnel over N6 between the onboard local UPF on MO UE side and the onboard local UPF in the MT UE side can be supported. This is up to operator's preference to use the AGW on the ground or select the UPF onboard satellite for an IMS call, when AGW are not placed on-board satellite. - An AGW relocation procedure is needed for change of satellite to route the UE-satellite-UE traffic to the new AGW. The simultaneous UL/CL change due to satellite change can be coordinated with AGW relocation to avoid packet loss. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clauses 5.4.x.1, 5.4.x.2, 5.4.x.2.1, 5.4.x.3, and 5.4.x.3.1 are proposed to describe general asepcts on UE-Satellite-UE communications, UPF and IMS-AGW onboard satellite, UPF on board satellite and IMS-AGW on the ground. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not be able to support UE-Satellite-UE Communicaitons. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.1, 5.4.x.2, 5.4.x.2.1, 5.4.x.3, and 5.4.x.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502, TS 23.228 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

### 5.4.x	Support of UE-Satellite-UE Communications

#### 5.4.x.1	General

UE-Satellite-UE communication refers to the communication between UEs under the coverage of one or more serving satellites without the user plane traffic going through the ground network.

NOTE: In this release, the UE-Satellite-UE Communications only support IMS voice/video services. And both UEs shall be served by the same HPLMN.

***** Next Change *****

#### 5.4.x.2	UPF and IMS-AGW onboard the satellites

When UPF and IMS-AGW onboard the satellites, the P-CSCF determines whether the conditions for UE-Satellite-UE Communications are met for IMS services based on access network information provided by the 5GC.

NOTE: SA3-LI specifications do not currently support AGW reallocation during the course of a call. Necessary enhancements to SA3-LI specifications may be required to meet regulatory requirements.

##### 5.4.x.2.1	Activation of UE-Satellite-UE Communication

Both parties of UE-Satellite-UE Communication complete IMS registration using the PDU sessions anchored on UPFs on the ground. When one UE initiates a call, the SIP INVITE message containing satellite-specific information is processed by the P-CSCF, which uses this information to determine if the conditions of UE-Satellite-UE Communication can be met and to activate it.

By activating UE-Satellite-UE Communication, the P-CSCF requests routing decisions from the SMF. If the conditions are met, the SMF inserts the onboard UPF as UL CL and local PSA for local traffic handling. If both UEs are served by the same or interconnected satellites, the call is routed entirely within the network onboard the satellite. Once the call setup is confirmed, the media path is established between the UEs through the onboard UPF and IMS-AGW, bypassing ground-based networks. For scenarios requiring transcoding or additional processing, the IMS-AGW onboard handles these tasks, maintain the traffic within the satellite network as described in TS 23.228 [15].

When serving satellite changes, SMF may perform procedures for simultaneous change of UL CL and additional PSA as described in clause 4.3.5.7 of TS 23.502 [3].

***** Next Change *****

#### 5.4.x.3	UPF onboard the satellite and IMS-AGW on the ground

When UPF onboard the satellite and IMS-AGW on the ground, to establish UE-Satellite-UE Communication, both parties must have a pre-configured transcoding option dedicated to UE-Satellite-UE Communications. This allows the IMS AGW to be bypassed during the course of the call.

NOTE: Necessary enhancements to SA3-LI specifications may be required to meet regulatory requirements.

##### 5.4.x.3.1	Activation of UE-Satellite-UE Communication

Both parties of UE-Satellite-UE Communication complete IMS registration using the PDU sessions anchored on UPFs on the ground. When one UE initiates a call, the SIP INVITE message containing satellite-specific information is processed by the P-CSCF, which uses this information to determine if the conditions of UE-Satellite-UE Communication can be met and to activate it. P-CSCF also determines, based on the received SIP message, that the IMS-AGW shall not be invoked for this session as described in TS 23.228 [15].

By activating UE-Satellite-UE Communication, the P-CSCF based on information requests routing decisions from the SMF. If the conditions are met, the SMF inserts the onboard UPF as UL CL and local PSA for local traffic handling. If both UEs are served by the same or interconnected satellites, the call is routed entirely within the network onboard the satellite. Once the call setup is confirmed, the media path is established between the UEs through the onboard UPF, bypassing ground-based networks.

When serving satellite changes, SMF may perform procedures for simultaneous change of UL CL and additional PSA as described in clause 4.3.5.7 of TS 23.502 [3].

NOTE: Necessary enhancements to SA3-LI specifications may be required to meet regulatory requirements.

***** End of Changes *****