3GPP TSG-WG2 Meeting #164	S2-2408009

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5503 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying devices behind UE/5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Conclusions for work item FS_UIA_ARC Rel 19 has been made in 3GPP meeting#163 and been added to TR 23.700-32 and this CR is part of normative work to include agreed features in the specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - Adding a new section 5.9.x to specify device identifier - Adding a new section 5.x to describe general aspects of the feature |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications not aligned with study conclusions |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9.x (new), 5.x (new), 5.x.1 (new), 5.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.503 CR#1344 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The referenced clause 6.1.3.x of TS 23.503 being added by 23.503 CR1344 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.9.x	Device Identifier

A Device Identifier (Device ID) is a permanent generic string assigned to a non-3GPP device behind a UE or 5G-RG to enable identification of the non-3GPP device in 5GC as described in clause 5.x.

NOTE:	An example of the Device ID is the device’s MAC address, however, the allocation of the Device ID is out of the scope of 3GPP.

The Device ID is unique at least within the scope of the UE’s or 5G-RG’s SUPI.

# * * * * Second change * * * *

## 5.x	Support for identifying non-3GPP devices behind a UE or a 5G-RG

### 5.x.1	General

Non-3GPP devices may get connectivity to the DN via 5GS through a UE or 5G-RG. To obtain differentiated QoS and/or charging per device, 5GS supports identification of non-3GPP device traffic and association of policies for each device.

The Non-3GPP Device QoS Information is stored in UE/5G-RG’s policy data in UDR by the OAM. The Non-3GPP Device QoS Information can be accessed via SUPI as a key as described in Table 5.2.12.2.1-1 of TS 23.502 [3] and it includes a list of Device IDs with the corresponding QoS parameters for each non-3GPP device and it may include the Maximum Number of Connected Devices (see clause 6.1.3.x of TS 23.503[45]).

Application guidance for URSP determination as described in clause 4.15.6.10 of TS 23.502 [3] can be performed to provide the UE/5G-RG with URSP rules that contain Device Descriptor.

### 5.x.2	Session management enhancement for supporting non-3GPP device identification

For one or more non-3GPP devices requiring traffic differentiation, the UE/5G-RG may send the Non-3GPP Device Information to the SMF in a NAS-SM message as described in UE-requested PDU Session Establishment in clause 4.3.2.2 of TS 23.502 [3] and PDU Session Modification in clause 4.3.3 of TS 23.502 [3]. The Non-3GPP Device Information contains a list of Device IDs and User Plane address for each of the non-3GPP devices. The SMF forwards the Non-3GPP Device Information to the PCF. The PCF may use the Device ID to obtain the QoS parameters from the UE/5G-RG’s policy data and take it into account for policy and charging decisions as described in clause 6.1.3.x of TS 23.503 [45].

The non-3GPP device address indicates the address that is used for sending and receiving non-3GPP device's traffic. The device address for IPv4 traffic is the UDP/TCP port number range, which may be assigned by NAT, and for IPv6 traffic is the IPv6 address/prefix and it may also contain the UDP/TCP port number range. For Ethernet PDU Session Type, the device address is the MAC address and/or the VLAN tag ID.

The Device Descriptor, whose value is identical to the Device ID, may be used by the UE/5G-RG as traffic descriptor to create PDU Session Establishment/Modification request for the non-3GPP devices connected to it.

Support for identifying devices behind RGs using DHCPv6 relaying option as specified in clause 4.10x of TS 23.316 [84].

# * * * * End of changes * * * *