SA WG2 Meeting #164	S2-2408013

19 - 23, August 2024, Maastricht, ND

Source:	Ericsson

Title:	FS_UIA_ARC KI#1-3 Solution Evaluation

Document for:	Approval

Agenda Item: 	19.8.1

Work Item / Release:	FS_UIA_ARC /Rel-19

Abstract of the contribution: This contribution provides solution evaluation for KI#1-3 of the FS_UIA_ARC.

# Introduction

This contribution provides evaluation for solutions related to KI#1, KI#2 and KI#3 in TR 23.700-32.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-32.

* * * * Start of Changes * * * *

# 7	Overall Evaluation

## 7.X	Key Issue #1, #2 and #3 Solution Evaluation

## 7.X.1	Introduction

Twenty-nine (29) solutions have been documented in TR 23.700-32 to address various aspects of Key Issues #1, #2 and #3, including,

- The information to be included in User Identity Profile and its storage location.

- The methods for linking, unlinking User Identifiers with subscriptions in the 5GC.

- The methods for authenticating User Identifiers and authorizing the activation of User Identifiers with subscriptions in the 5GC.

- The identification of User Identifiers associated with a UE’s traffic and the consideration of user-specific policies by 5GS for service differentiation.

- The support for access to specific services with a User Identifier.

- The exposure of User Identity Profile Information.

Additionally, it is worth to evaluate the solutions regarding other aspects such as,

- Support in roaming scenarios.

- Support interworking with EPS.

- Overall system impact.

## 7.X.2	User Identity Profile Information and Storage

Most of the solutions are aligned when it comes to the general content of the User Identity Profile.

- User Identifier and relation of 5GC subscriptions the User Identifier is linked with (or vice versa).

- Per User Identifier:

  - User Identifier credentials for authentication of the user identifier.

  - Service differentiation parameters associated to the user identifier.

Most of the solutions clearly refer to QoS and other type of Policy Control parameters as the main User Identity Profile information used for service differentiation. Some solutions mention that the User Identity Profile may include S-NSSAI related information, but it is unclear how this slice information will be used by the 5GC. According to one of the architectural principles, the user profile should not be used to override information within the 5GC subscription so it would not be acceptable to e.g. provide to AMF a subset of the subscribed S-NSSAIs for a SUPI when a User Identifier is activated in the SUPI.

Regarding storage options for the User Identity Profile, two main options are proposed:

- User Identity Profile stored at the 5GC UDR.

Some of the solutions that propose that the User Identity Profile is stored in UDR indicate the need for one or multiple new additional data sets to be managed in the UDR. However, many of these solutions are not specific to where exactly the user profile information would fit within UDR as it is then mentioned that e.g. UDM can include related User Identity Profile parameters within Access and Mobility and/or Session Management procedures or even influence the 5GC subscription data provided to AMF or SMF, which again would be against the principle that the user profile should not be used to override information within the 5GC subscription. Similarly, it is mentioned that PCF should consider information within the User Identity Profile to apply user specific policies.

It is assumed that when the User Identity Profile is stored in UDR, UDR consumers in the 5GC (i.e. NEF, UDM, PCF) uses the SUPI as data key with optionally the User Identifier as secondary key.

Finally, these proposals consider two main options for managing the User Identity Profile at the 5GC UDR, by OAM or by an AF using External Parameter Provisioning via NEF.

NOTE: Provisioning of User Identifier Credentials in the 5GC UDR via Parameter Provisioning needs to be evaluated by SA3 WG.

- User Identity Profile stored in a User Identity Profile Server outside of the 5GC at the Application Layer.

According to these solutions, the User Identity Profile can be managed by the operator via OAM or by the user via Portal or User Plane procedures. The specification of the management procedures of the User Identity Profile at the User Identity Profile Server would be out of scope of SA2 WG (maybe in scope of SA6 WG).

## 7.X.3	Linking/Unlinking User Identifiers with 5GC Subscriptions

As described in 7.X.2, the relation of a User Identifier and relation of 5GC SUPI(s) the User Identifier is linked with is an integral part of the User Identity Profile content. Therefore, the procedures for linking/unlinking User Identifiers with 5GC subscriptions are the same ones as the ones used for managing the User Identity Profile information described in clause 7.X.2.

- When the User Identity Profile is stored at the 5GC UDR, User Identifiers are linked/unlinked with 5GC subscriptions either via operator OAM or by an AF using External Parameter Provisioning via NEF.

- When the User Identity Profile is stored in a User Identity Profile Server outside of the 5GC at the Application Layer, User Identifiers are linked/unlinked with 5GC subscriptions either by the operator via OAM or by the user via Portal or User Plane procedures.

## 7.X.4	Authenticating User Identifiers, Authorizing and Activating/Deactivating User Identifiers with 5GC Subscriptions

A user is considered active if the associated User Identifier has been authenticated and authorized to use a linked subscription to access the 5GS. Therefore, the activation of a User Identifier with a 5GC subscription shall be preceded by the authentication of the User Identifier and the authorization that the User Identifier can be used with the 5GC subscription.

Regarding authentication of a User Identifier, solutions can be grouped as follows:

- NAS-MM solutions, where a User Identifier is authenticated during UE registration of the 5G subscription in the 5GC including a User Identifier in NAS-MM signalling. At reception of a UE registration including a User Identifier, the AMF executes the primary authentication and registration procedures as defined in the baseline and additionally, the AMF triggers a new authentication procedure for the User Identifier.

- NAS-SM solutions, where a User Identifier is authenticated during session management procedures. At reception of a PDU Session establishment request that includes a User Identifier the SMF triggers a new authentication procedure for the User Identifier following principles similar to the ones used in secondary authentication.

- Application Layer solutions, where a User Identifier is authenticated by the User Identity Profile Server during User Identity Profile management procedures via Portal or User Plane. There is no support required from the 5GC to execute the authentication procedures of the User Identifier at the User Identity Profile Server and therefore its specification would be out of scope of SA2 WG.

The following shall be observed for the authentication solutions based on either NAS-MM or NAS-SM:

- Study of the details of the authentication procedure are in the remit of SA3 WG but in relation to the content of the User Identity Profile where User Identifier Credentials are considered to be required, it is worth mentioning that the option to use an external AAA as authentication server for the User Identifier is not compatible with the proposal to store the corresponding credentials to authenticate the User Identifier at the 5GC UDR, as the external AAA shall not have access to the 5GC UDR.

- Also, in the case where an external AAA server is used as authentication server, the 5GC is basically used as a transport channel to convey authentication messages between the UE and the external AAA. The “modem part” of the UE or the 5GC have no means to determine whether the user identifier credentials or biometrics actually correspond to the appropriate human user.

- Finally, AMF or SMF may require additional information within the subscription data received from UDM to decide whether and how to trigger the User Identifier authentication procedure (e.g. address of the external AAA server authenticating the User Identifier).

Regarding authorization for activating a User Identifier with a 5G subscription,

- When the User Identity Profile is stored at the 5GC UDR, the AMF or SMF are supposed to be able to receive from UDM the list of User Identifiers associated to a given 5G subscription (SUPI). Then after successful authentication of the User Identifier via NAS-MM or NAS-SM, AMF or SMF can authorize that the User Identifier can be activated with the 5GC subscription.

- When the User Identity Profile is stored in a User Identity Profile Server outside of the 5GC at the Application Layer, the User Identity Profile Server can authorize that the User Identifier can be activated with the 5GC subscription during User Identity Profile management procedures via Portal or User Plane or after NAS-MM or NAS-SM authentication when the User Identity Profile Server is used as external AAA.

Before activation of the User Identity with the 5GC subscription, it may be needed to additionally check if another User Identifier was already active at that 5GC subscription in order to either reject the activation of the new User Identifier or deactivate the old User Identifier with the 5GC Subscription. This means that the User Profile Information shall also include the information about the active User Identity used with a given 5GC subscription (SUPI):

- When the authorization is executed by AMF or SMF based on User Identity Profile information received from the UDM/UDR, this means that extra procedures are required so that the UDM/UDR is updated with the information of the active User Identity per SUPI after successful User Identity authentication/authorization.

- When the authorization is executed by the User Identity Profile Server, it is the User Identity Profile Server the one that has all the information and control, so no extra procedure is required.

## 7.X.5	Service Differentiation

As mentioned in 7.X.1, QoS and other type of Policy Control parameters are the main User Identity Profile information used for service differentiation. This dynamic QoS and Policy Control functionality is under control of the PCF within the 5GC and therefore the corresponding User Identity Profile information shall be made available to the PCF in the first place.

- When the User Identity Profile is stored at the 5GC UDR, the PCF shall have direct access to the corresponding User Identity Profile information stored in UDR (using either SUPI or User Identifier or both). This makes evident that the User Identity Profile stored in UDR would be scattered over different Data Sets stored in UDR, i.e. at least in Subscription data for the information required to be managed via UDM, Policy or Application Data for the information to be managed via PCF and maybe in additional new Data Sets.

In order for the PCF to be able to retrieve from UDR and apply user specific policies, the PCF is also required then to receive the User Identifier from the SMF. This is, the User Identifier needs to be inserted by the UE in NAS-MM or NAS-SM signalling and AMF/SMF shall include the User Identifier in all subsequent requests within the 5GC.

- When the User Identity Profile is stored in a User Identity Profile Server outside of the 5GC at the Application Layer, after successful authentication, authorization and activation of a User Identifier with a 5GC subscription, the User Identity Profile Server selects the corresponding service differentiation parameters for the active user and requests the 5GC to apply the User Identity Profile to the 5GC subscription the User Identity is active at using existing or enhanced exposure APIs via NEF. In this option, the PCF is not required to have access to the User Identity Profile. The PCF just executes on the input provided by the User Identity Profile server acting as an AF via NEF. In principle, the PCF would not require either to be aware of the User Identity, although it would be required for charging purposes (to be confirmed).

## 7.X.6	Access to Specific Services

During the study phase it has been debated whether when an User Identifier is active in a given 5GC subscription, SMS and/or Voice services using public identifiers of the 5GC subscription shall be suspended or not. An LS to SA1 was sent asking for clarification but no answer from SA1 has been received so far. No conclusions in this aspect can be reached until a formal answer from SA1 is received and discussed in SA2.

## 7.X.9	Exposure of User Identity Profile

KI#3 in TR 23.700-32 focuses on how User Identifier related functionality and information is exposed. As described in previous clauses, many of the solutions related to KI#1 and KI#2 involve exposure interactions but rather in the sense for an AF to provide User Identity Profile information to the 5GC via NEF.

KI#3 also mentions the exposure of authorization/authentication results of a given User Identifier to an external AF via NEF and some solutions address this case based on the UDM being aware of the activation status of a given User Identifier with a given 5GC subscription.

When the User Identity Profile is stored in a User Identity Profile Server outside of the 5GC at the Application Layer, the awareness of the authentication status of a given User Identity is already at the application layer and then there is no need for the 5GC to expose such information. Relevant APIs for exposing the authentication result of a given User Identifier outside of the 3GPP domain can be enabled and specified by SA6 WG.

## 7.X.7	Support for Roaming

One additional aspect to consider during the evaluation of solutions for KI#1, KI#2 and KI#3 is whether the solutions would enable the user specific service differentiation while roaming.

Solutions that are based on NAS-MM signalling will not enable the authentication/authorization/activation of User Identifiers while roaming unless the AMF in the VPLMN also supports the corresponding procedures.

Solutions that are based on NAS-SM signalling may enable the authentication/authorization/activation of User Identifiers while roaming in Home Routed case (i.e. when SMF is in HPLMN).

Solutions that are based on Application Layer signalling enable the authentication/authorization/activation of User Identifiers while roaming with no restriction or dependency with support in VPLMN.

## 7.X.8	Support for Interworking with EPS

Another additional aspect to consider during the evaluation of solutions for KI#1, KI#2 and KI#3 is whether the solutions would enable the user specific service differentiation while the User Identifier is activated with a 5GC subscription which is connected in EPS.

Solutions that are based on NAS-MM signalling will not enable the authentication/authorization/activation of User Identifiers when the 5GC subscription is connected in EPS. Similar support in MME would be required as the one required in AMF and this is not covered.

Solutions that are based on NAS-SM signalling enables the authentication/authorization/activation of User Identifiers when the 5GC subscription is connected in EPS, provided that the PDU sessions established in EPS are anchored in a combined SMF/PGW-C. If the User Identifier bigger than 256 octets is included in ePCO of PDN connectivity request, solutions need to ensure that the User Identifier is provided only in PDN connection served by the combined SMF/PGW-C supporting the User Identifier, to avoid incorrect decoding of ePCO with the User Identifier bigger than 256 octets by a combined SMF/PGW-C not supporting the User Identifier or by a P-GW. If the User Identifier is included in ePCO of PDN connectivity request for IP based PDN connection in WB-S1 mode not for UAS services, the solutions need to ensure that legacy services, if any, provided via the PDN connection are not disrupted when the UE performs initial attach to a network not supporting ePCO (e.g. network which does not support NB-S1 mode, non-IP and Ethernet PDN types, inter-system change with 5GS, UAS services, and enforcement of use of DNS over (D)TLS).

Solutions that are based on Application Layer signalling enable the authentication/authorization/activation of User Identifiers when the 5GC subscription is connected in EPS with no restriction or dependency with support in EPC or 5GC.

## 7.X.9	Overall System Impact

The overall system impact in the 5GC of the different solutions shall be also considered during the evaluation of solutions for KI#1, KI#2 and KI#3, especially when it comes to considering impacts on the NAS signalling support in the UE as in solutions based on NAS-MM or NAS-SM signalling for authenticating/authorizing/activating User Identifiers. These solutions have also impact on AMF, SMF, UDM/UDR (when combined with storage of User Identity Profile in UDR), and an External AAA.

Solutions based on storage and management of the User Identity Profile in the UDR impact obviously UDR, NEF and AF (when the User Identity Profile is managed by an AF), UDM, PCF, AMF and SMF for the use of the User Identity Profile information for specific users.

Also, these solutions imply impact on multiple messages over multiple APIs (AMF, SMF, PCF, UDM, UDR, NEF) to include the User Identity in core signalling of the 5GC.

Solutions that are based on storage and management of the User Identity Profile and authentication, authorization and activation of User Identities in a User Identity Profile Server outside of the 5GC at the Application Layer, minimize the impacts on the 5GC as the interactions are restricted between the UE and the User Identity Profile Server via User Plane signalling. High reuse of the existing exposure APIs defined for user specific QoS and Policy Control is expected to be possible, but some extensions can be considered if needed.

## 7.X.10	Evaluation Summary

The previous evaluation can be summarized in the following comparison table.

| (User Id Profile Storage) | 5GC UDR | User Id Profile Server (UIPS) |
| --- | --- | --- |
| (Auth/Activation) | NAS-MM / NAS-SM |  |
| User Identity Profile Content | - List of User Identifiers linked per 5GC subscription (SUPI) - Active User Identifier with each 5GC Subscription. - Per User Identifier, - User Identifier credentials for authentication of the user identifier. - Service differentiation parameters associated to the user identifier (QoS and Policy Control mainly). NOTE: Use of other service differentiation parameters, such as Slicing, may violate architectural principle that the user profile should not be used to override information within the 5GC subscription and therefore is not further considered in this evaluation. | - List of User Identifiers linked per 5GC subscription (GPSI) - Active User Identifier with each 5GC Subscription. - Per User Identifier, - User Identifier credentials for authentication of the user identifier. - Service differentiation parameters associated to the user identifier (QoS and Policy Control mainly). |
| User Identity Profile Management (incl. Linking/unlinking User Ids) | OAM Parameter Provisioning with UDM (linked/active User Identifiers) Service Parameter Provisioning with PCF (Service differentiation params) NOTE: User Id Profile info scattered in different UDR Data Sets. NOTE: Credentials Provisioning in UDR via Parameter Provisioning to be evaluated by SA3 WG. | OAM User Plane |
| Authentication | New procedure AMF or SMF triggered based on User Id in NAS signalling and additional info in Subs Data (UDM/UDR). NOTE: Management of User Id authentication credentials when Ext AAA used not described. NOTE: 5GC used as transport of authentication signalling but 5GC cannot guarantee whether credentials correspond to appropriate user. | Over User Plane with UIPS |
| Authorization/Activation | After User Id authentication, controlled by AMF or SMF based on User Identifier Profile info stored in UDM/UDR (i.e. list of User Identifiers authorized to be linked with SUPI). New procedure to store in UDM active User Identifier with SUPI. New active user deactivates any previous active user with the SUPI. | After User Id authentication, controlled by UIPS based on User Identifier Profile info locally available to the UIPS (i.e. list of User Identifiers authorized to be linked with SUPI). UIPS updates active user identifier with 5GC subscription in User Identifier Profile locally. |
| Service Differentiation | After activation, PCF receives User Id of active user with the SUPI via AMF/SMF signalling. PCF retrieves User Id Profile info applicable to the SUPI/User Id from the UDR. PCF applies user specific QoS/Policies. | After activation, UIPS selects user specific QoS/Policy info applicable to the active user and requests the 5GC to apply them for the SUPI in which the User Identity has been activated using existing or enhanced exposure APIs via NEF. |
| Roaming Support | NAS-MM: NO unless AMF in VPLMN supports corresponding procedures/data. NAS-SM: YES (Home Routed Only) | YES with no limitation or dependency with VPLMN |
| EPS Interworking Support | NAS-MM: NO. NAS-SM: YES with combined PGW-C/SMF supporting corresponding procedures/data. Dependencies with MME to convey User Id to be confirmed. | YES with no limitation or dependency with EPS/5GC |
| Overall Impact | UE, AMF, SMF, PCF, UDR, UDM, NEF, AF, ext AAA NAS Impact Multiple core baseline procedures/services (AMF/SMF/PCF/UDM/UDR APIs) impacted to support User Identifier. | UE, UIPS Optionally NEF, UDR, PCF in case exposure APIs between UIPS and 5GC need to be enhanced. |

* * * * End of Changes * * * *