3GPP TSG-SA2 Meeting #164	S2-2408016

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1146 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: model storage and retrieval |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to conclusions for key issue 1 in TR 23.700-84, the LMF may request models trained at the MTLF for location inference or train such models on its own. Procedures to enable the LMF to retrive models from MTLF are required. It is also desirable to allow an LMF that trains models to store those models and related input data at the ADRF. See consideraions in discussion paper S2-2408015. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A pseudu Analytics ID for LMF models is defined. This enables the reuse of the NWDAF Model Info and NWDAF Model Provision services with the LMF as consumer without the addition of new parameters, and also the reuse of ADRF services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LMF is not able to retrieve models from MTLF and not able to store models it trained and related input data at the ADRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.0, 6.2A.3, 6.2B.5, 6.2B.6, 6.2B.7, 7.1, 10.1, 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 6.2A.0	General

This clause presents the procedure for the ML Model provisioning.

An NWDAF containing AnLF or an LMF may be locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML Models or may use the NWDAF discovery procedure specified in clause 5.2 for discovering NWDAFs containing MTLF. An NWDAF containing MTLF may determine that further training for an existing ML Model is needed when it receives the ML Model subscription or the ML Model request.

A NWDAF containing MTLF may retrieve trained ML Models from other NWDAF containing MTLF as described in clause 5.3. The NWDAF containing MTLF determines to train a ML Model either based on the request from NWDAF containing AnLF or an LMF, or based on local configuration. The NWDAF containing MTLF further determines the FL procedure is required but it can not act as an FL server, therefore, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the ML Model provisioning from the FL server NWDAF. How to protect the trained ML Model file from being used without authorization e.g. being forwarded by an NWDAF containing MTLF that has retrieved a model from another NWDAF containing MTLF is defined in Annex X, clause X.10 of TS 33.501  [49].

2nd change

### 6.2A.3	ML Model request

The procedure in Figure 6.2A.3-1 is used by an NWDAF service consumer, i.e. an NWDAF containing AnLF or an LMF to request and get from another NWDAF, i.e. an NWDAF containing MTLF ML Model Information, or an NWDAF containing MTLF to request and get from another NWDAF containing MTLF worked as FL server ML Model Information, using Nnwdaf_MLModelInfo services as defined in clause 7.6. The ML Model is used by an NWDAF containing AnLF to derive analytics. An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.3-1: ML Model Request

1.	The NWDAF service consumer requests a (set of) ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking Nnwdaf_MLModelInfo_Request service operation. The parameters that can be provided by the NWDAF Service Consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML Models if available.

Whe an ML Model Information request is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the request; or

-	determine whether triggering further training for the existing trained ML Models is needed for the request.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from NFs, (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML Model.

2.	The NWDAF containing MTLF responds to the NWDAF service consumer by invoking Nnwdaf_MLModelInfo_Request response service operation including:

-	a set of pair(s) of unique ML Model identifier, the ML Model Information for each Analytics ID that the NWDAF service consumer requests.

The content of ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

3rd change

### 6.2B.5	ML Model Storage in ADRF

The procedure depicted in figure 6.2B.5-1 is used by NF consumer, i.e., NWDAF containing MTLF or LMF to store ML Model in an ADRF.

Figure 6.2B.5-1: ML Model Storage in ADRF

0.	NWDAF containing MTLF or LMF determine to store ML Model in ADRF based on MTLF policy.

1.	NWDAF containing MTLF or LMF request to store a (set of) ML Model(s) to the ADRF by invoking Nadrf_MLModelManagement_StorageRequest service, optionally including allowed NF instance list for the ML Model identifier as described in TS 33.501 [49].

2.	The ADRF locally maintains the association between the ML Model identifier and the NF instance ID of NWDAF containing MTLF or LMF and allowed NF instance list (if there is any in step 1).

[Optional] If instead of the ML Model(s), the ML Model address(es) is/are included in request, ADRF downloads the ML Model(s) based on the ML Model address(es) and locally stores the ML Model(s).

3.	The ADRF sends Nadrf_MLModelManagement_StorageRequest Response message to the consumer including the ML Model storage result indication.

4th change

### 6.2B.6	ML Model removal from ADRF

The procedure depicted in figure 6.2B.6-1 is used by NF consumer, i.e., NWDAF containing MTLF or LMF to delete ML Model from an ADRF.

Figure 6.2B.6-1: ML Model removal from ADRF

1.	NWDAF containing MTLF or LMF request to delete the ML Model(s) previously stored in the ADRF using Nadrf_MLModelManagement_Delete request service operation.

2.	The ADRF deletes both the stored ML Model(s) and related ML Model information.

3.	The ADRF indicates the result (i.e. ML Model deleted, ML Model not found, ML Model found but not deleted) using Nadrf_MLModelManagement_Delete response service operation.

5th change

### 6.2B.7	ML Model retrieval from ADRF

The procedure depicted in Figure 6.2B.7-1 is used by consumers (NWDAF containing MTLF, NWDAF containing AnLF, or LMF) to retrieve ML Models from an ADRF

Figure 6.2B.7-1: Procedure for ML Model(s) retrieval from ADRF

1.	The NWDAF service consumer (NWDAF containing AnLF, NWDAF containing MTLF, or LMF) subscribes/requests for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelInfo_Request service.

2.	The NWDAF containing MTLF determines whether the set of ML Model(s) associated with a/an (set of) Analytics ID(s) should be retrieved from the ADRF.

When NWDAF containing MTLF authorizes the NF consumer to retrieve the ML Model(s) stored in the ADRF directly, steps 3 and 4 is skipped.

If NWDAF containing MTLF determines that the set of ML Model(s) corresponding Analytics ID(s) requested in step 1 needs to be retrieved from ADRF and the NF consumer is agnostic to where the ML Model(s) is stored, then Steps 3 and 4 is performed.

NOTE:	How NWDAF containing MTLF and ADRF authorizes the NF Consumer is specified in clause X.10 of TS 33.501 [49].

3.	The ADRF service consumer (NWDAF containing MTLF) requests for the ML Model stored in ADRF by invoking the Nadrf_MLModelManagement_RetrievalRequest Request (Storage Transaction Identifier or one or more unique ML Model identifier(s)) service operation.

4.	The ADRF verifies the service consumer (NWDAF containing MTLF) as described in Annex X.10 of TS 33.501 [49]. If verification is successful, the ADRF sends Nadrf_MLModelManagement_RetrievalRequest Response (ML Model file address of Model file(s) stored in ADRF) service operation.

5.	The NWDAF containing MTLF notifies/ response to the NWDAF service consumer with the tuple Analytics ID, one or more tuples of unique ML Model identifier and ML Model Information. The ML Model information may contain the ML Model file address or ADRF (Set) ID. The ADRF(Set) ID is included only when the NWDAF containing MTLF authorizes the NF consumer to retrieve the ML Model(s) stored in the ADRF in step 2. When ADRF (Set) ID is provided and the NWDAF containing MTLF authorizes the NF Service Consumer to retrieve all ML Models corresponding to a Storage Transaction ID, the Storage Transaction ID may be provided. In other cases, the NWDAF containing MTLF only provides ML Model identifier(s).

6.	If in step 5, the NWDAF service consumer (NWDAF containing AnLF, NWDAF containing MTLF, or LMF) received ADRF (Set) ID (where the ML Model(s) requested in step 1 is stored) and/or the ML Model provide indicator, then the NWDAF service consumer may invoke the Nadrf_MLModelManagement_RetrievalRequest (Storage Transaction Identifier or one or more unique ML Model identifier(s)) service operation to get the ML Model stored in ADRF.

7.	The ADRF verifies the service consumer as described in Annex X.10 of TS 33.501 [49]. If verification is successful, the ADRF sends Nadrf_MLModelManagement_RetrievalRequest Response (ML Model identifier(s) and address(es) of Model file(s) stored in ADRF to the NWDAF service consumer.

6th change

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |

The MTLF may also train models to be used by other NFs for which no Analytics information will be exposed by the NWDAF, and such models can also be stored at the ADRF. The purpose of such a model is also indicated by an Analytics ID, but such an Analytics ID is only applicable ML model related NWDAF and ADRF services and MTLF and ADRF related NF profiles at the NRF. Table 7.1-3 shows Analytics IDs which are only defined to express the purpose of models.

Table 7.1-3: Analytics IDs only defined to express the purpose of models

| Analytics ID | Description |
| --- | --- |
| LMF AIML positioning | This analytics ID is used to identify models trained for the purpose of determining UE positions at the LMF |

7th change

## 10.1	General

Table 10.1-1 shows the ADRF services and ADRF service operations.

ADRF service operations may be used to store data or analytics in the ADRF, retrieve data or analytics from an ADRF, or delete data or analytics from an ADRF.

ADRF service operations may also be used to store ML Model(s) or ML Model address(es) in the ADRF, retrieve ML Model(s) from an ADRF or delete ML Model(s) from an ADRF.

Table 10.1-1: NF services provided by ADRF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nadrf_DataManagement | StorageRequest | Request / Response | DCCF, NWDAF, MFAF, LMF |
|  | StorageSubscriptionRequest | Request / Response | DCCF, NWDAF, LMF |
|  | StorageSubscriptionRemoval | Request / Response | DCCF, NWDAF, LMF |
|  | RetrievalRequest | Request / Response | DCCF, NWDAF, LMF |
|  | RetrievalSubscribe | Subscribe / Notify | DCCF, NWDAF, LMF |
|  | RetrievalUnsubscribe |  | DCCF, NWDAF, LMF |
|  | RetrievalNotify |  | DCCF, NWDAF, LMF |
|  | Delete | Request / Response | DCCF, NWDAF, LMF |
| Nadrf_MLModelManagement | StorageRequest | Request / Response | NWDAF, LMF |
|  | RetrievalRequest | Request / Response | NWDAF, LMF |
|  | Delete | Request / Response | NWDAF, LMF |

8th change

### 10.3.2	Nadrf_MLModelManagement_StorageRequest service operation

Service operation name: Nadrf_MLModelManagement_StorageRequest

Description: The consumer NF uses this service operation to request the ADRF to store or update ML Model(s). ML Model(s) or ML Model address(es) stored in NWDAF containing MTLF or LMF are provided to the ADRF in the request message.

Inputs, Required: NF instance ID of the NWDAF containing MTLF or LMF and set of:

-	ML Model(s); or

-	the tuple (unique ML Model identifier and ML Model file address and Storage size required for each of the ML Models).

NOTE 1:	The ADRF can download ML Model file using the ML Model addresses provided by the NWDAF containing MTLF or LMF. How the ADRF locally stores the ML Model is left for implementation.

Inputs, Optional: Allowed NF instance list for the ML Model identifier.

Outputs Required:

-	Result Indication;

-	[Conditional] one or more tuples of unique ML Model identifier and ML Model address of Model file stored in ADRF.

NOTE 2:	The definition of ML file address such as e.g. name, location and access method is up to stage 3.

Outputs, Optional: Storage Transaction Identifier.

End of changes