3GPP SA WG2 Meeting #162	S2-2408023

Maastricht, NL  August 19-23, 2024

Source:	Nokia

Title:	EN Resolution for KI#1

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This CR resolves the ENs the conclusion for KI#1

# Discussion

In the conclusion for KI#1 there is the EN:

1.	According to the Protocol Description, the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator, , the NG RAN may indicate the 5GC to perform the PDU Set Information marking.

Editor's note:	whether reuse the existing indicator or need a new indicator for DL PDU Set Information Marking is supported is FFS.

Currently the process for activating PDU Set Information Marking is described in TS23.501, clause 5.37.5.2:

If the NG-RAN has provided a PDU Set based handling support Indication indicating that PDU Set handling is supported and a Protocol Description together with 1) a PSIHI and/or 2) PSDB and PSER is included in the PCC rule, the SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the DL Protocol Description used by the service data flow.

and clause 5.37.5.3:

The SMF, by sending PDU Set QoS parameters to the NG-RAN as described in clause 5.7.7.1, requests the NG-RAN to activate PDU Set QoS handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF.

Observation 1: Currently the indication sent by the SMF to the NG-RAN is comprised of PDU Set QoS parameters from the PCC rule: 1) a  PSIHI and/or 2) both PSDB and PSER.

It is not clear how PSIHI alone is used by the NG-RAN for PDU Set QoS. For the SMF to clearly indicate support for PDU Set Identification and marking at the UPF, independent of a request for PDU Set QoS based on PSER, PSDB and PSIHI, a separate indicator should be sent by the SMF.

Proposal 1: The SMF may:

1 – Request the NG-RAN to activate PDU Set QoS by sending PSDB, PSER and PSIHI PDU Set QoS parameters to the NG-RAN, or

2 – Indicate to the NG-RAN support for PDU Set Identification and marking at the UPF, which the NG-RAN may use for implementation specific purposes.

Observation 2: Currently the RAN response to the SMF request is the PDU Set Based Handling Support Indication. This serves two purposes. It indicates to the SMF that its request for PDU Set QoS handling has been accepted, and it triggers activation of PDU Set Identification Marking at the UPF. If, as discussed in Proposal 1, the SMF may separately request PDU Set QoS (for which it provides PSER, PSDB and PSIHI) or indicate that PDU Set Identification & Marking is supported at the UPF, then the response from the NG-RAN should be able to separately indicate 1 – whether the request for PDU Set QoS is accepted, and 2 - a PDU Set handling support indication. If either of these are provided in the response, the SMF should activate PDU Set Identification and marking.

Proposal 2: The RAN may indicate to the SMF:

1 – That the RAN will activate PDU Set QoS, or

2 – A PDU Set handling support indication

which are used by the SMF to activate PDU Set Identification and marking

In addition, for KI#1 there are two additional ENs that are similar:

For QoS Notification without AQP there is:

Editor's note:	It is FFS whether and how the 5GC and the AF understands PDB/PER or UL and/or DL PDU Set QoS parameters of the QoS profile are fulfilled or not in the corresponding direction upon reception of the QoS notification and it is FFS whether the RAN indicates the DL/UL PDU Set handling support or not.

and for QoS Notification with AQP there is:

Editor's note:	It is FFS whether and how the 5GC and the AF understands PDB/PER or UL and/or DL PDU Set QoS parameters in the AQP are fulfilled or not in the corresponding direction upon reception of the reference of the AQP and it is FFS whether the RAN indicates the DL/UL PDU Set handling support or not to the 5GC.

Observation 3: In both cases, the NG-RAN decides if and how to use ordinary QoS parameters (PDB/PER) or PDU Set QoS parameters (PSER/PSDB). The NG-RAN also decides if and when to formulate a notification that “GFBR cannot be guaranteed or can be guaranteed again” and the contents of the notification. The SMF forwards received notifications to the PCF. The most straightforward way to ensure that the AF understands whether ordinary QoS or PDU Set QoS parameters are used in the NG-RAN is for the NG-RAN to include an appropriate flag/indication when it formulates the notifications.

Proposal 3: When sending a notification that GFBR cannot be guaranteed or can be guaranteed again, the NG-RAN includes an indication whether PDU Set QoS (PSER/PSDB) or ordinary QoS parameters (PER/PDB) are used for the QoS flow.

# Proposal

Update the Conclusion for KI#1 in TR23.700-70 as follows:

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.1	Interim agreements for Key Issue #1

### 8.1.1	Agreement for KI#1_other PDU Set handling

The following agreement is considered for #1 for other PDU Set handling:

1.	According to the Protocol Description, the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator, the NG RAN may indicate the 5GC to perform the PDU Set Information marking. The SMF may request the NG-RAN to activate PDU Set QoS by sending PSER, PSDB and PSIHI PDU Set QoS parameters to the NG-RAN. If PDU Set QoS is not requested for the DL, the SMF may indicate to the NG-RAN support for PDU Set Identification and marking at the UPF.

In its response, the RAN may indicate to the SMF:

-	that the RAN will activate PDU Set QoS, or

-a PDU Set handling support indication

which are used by the SMF to activate PDU Set Identification and marking.

### 8.1.2	Agreement for KI#1_QoS Notification without AQP

The following interim agreement is considered for #1_PDU Set handling support or not indications in UL and/or DL and QoS notification:

1.	In order to support QoS Notification Control "GFBR cannot be guaranteed or can be guaranteed again" (see clause 5.7.2.4 in 23.501 [2]), for a GBR QoS flow with the QoS profile containing UL and/or DL PSDB and UL and/or DL PSER, instead of PDB and PER, the fulfilment or not of UL and/or DL PSDB or UL and/or DL PSER in the QoS profile are used by the NG-RAN to determine QoS notification if the PDU Set QoS handling is applied in the corresponding direction. Otherwise, the fulfilment or not of the PDB and/or PER in the corresponding direction is still used for the QoS notification. The NG-RAN includes an indication in the notification of whether PDU Set QoS (PSER/PSDB) or ordinary QoS parameters (PER/PDB) are used when determining whether GFBR cannot be guaranteed or can be guaranteed again.

### 8.1.3	Agreement for KI#1_AQP

The following conclusion principles are agreed for key issue #1_AQP.

1.	The AF may provide Alternative Service requirements to 5GC which may contain the UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

2.	Based on the Alternative Service Requirements from AF, the PCF generates PCC rule with Alternative QoS parameters in a prioritized order, which can include UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER) and the corresponding reference to each Alternative QoS parameter Set.

3.	SMF provides one or more Alternative QoS Profile(s) with UL and/or DL PDU Set QoS parameters to NG-RAN. When the QoS profile includes UL and/or DL PDU Set QoS parameter, the Alternative QoS Profile shall include the corresponding PDU Set QoS parameters; otherwise, the Alternative QoS profile does not include the corresponding PDU Set QoS parameters(s) when the QoS profile doesn't include the UL and/or DL PDU Set QoS parameter.

4.	If QoS notification is determined as "GFBR cannot be guaranteed", the NG-RAN determines the AQP that can be used as a reference to the currently fulfilled QoS considering the priority order: If the PDU Set QoS handling is applied in the corresponding direction, the UL and/or DL PSDB and UL and/or DL PSER in the AQP are used by the NG RAN to determine the AQP that can be used as a reference to the currently fulfilled QoS. The NG-RAN includes an indication in the notification of whether PDU Set QoS (PSER/PSDB) or ordinary QoS parameters (PER/PDB) are used when determining the AQP that can be used as a reference.

5.	During PDU Session Establishment/Modification procedures, the SMF/PCF may update the QoS profile and/or Alternative QoS Profiles to include/exclude the PDU Set QoS parameters based on the PDU Set Based Handling Support Indication from the NG-RAN.

*** END of changes ***