3GPP TSG-SA2 Meeting #164	S2-2408040

# Maastricht, Netherlands, 19 August - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1149 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Preparation procedure including sample alignment for AF-initiated VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following conclusion with NOTEs in TR 23.700-84 clause 8.2 needs to be addressed: P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples. NOTE 3: The procedure to perform sample alignment is not agreed yet, i.e. it can be done in a standalone procedure or part of the preparation phase, which will be determined in normative phase. NOTE 4: How NEF assists sample alignment in P#2.3.3 will be determined in normative phase. NOTE 5: For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added to address the above conclusion, including the preparation procedure including sample alignment for AF-initiated VFL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-84 (AIML_CN) KI#2 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.2X	Vertical Federated Learning

6.2X.1	General

This clause specifies how the training and/or inference processes by the vertical federated learning (VFL) technique can be performed in the following scenarios:

-	AF-initiated VFL between AF and NWDAF(s) within a single PLMN.

-	NWDAF-initiated VFL among NWDAFs or AF and NWDAF(s) within a single PLMN.

6.2X.2 AF-initiated VFL

This clause specifies the preparation (including sample alignment), training, and interference procedures for AF-initiated VFL scenarios between AF and NWDAF(s) within a single PLMN.

6.2X.2.1 Preration procedures

This clause specifies the preparation including sample alignment for AF-initiated VFL scenarios.

Figure 6.2C.2.1-1: Preparation (including sample alignment) procedure for Vertical Federated Learning

- The AF as VFL server discovers and select candidate VFL clients(s) by the discovery procedure.

  - The AF as VFL server sends VFL preparation request to the candidate VFL client(s) with the ML Preparation Flag. This is done either directly using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service for a trusted AF, or indirectly via NEF using Nnef_VFLPreparation_Subscribe service for an untrusted AF, to check if the VFL client(s) can meet the ML Model training requirement (e.g. Analytics ID, ML Model interoperability information, Sample alignment requirement, Data Availability requirement, FL Availability time requirement (time span needed for the FL process), etc.). Sample alignment requirement includes  a list of samples targeted for VFL training as defined by the AF. Data Availability requirement includes a list of Event IDs of the local data necessary for training, ensuring different features for the same samples across each involved NF in the context of VFL feature alignment. It may also include the dataset statistical properties, the time window of the data samples, and the minimum number of data samples. The NEF is only involved for an untrusted AF and maps the external UE IDs (e.g. GPSIs) to the internal UE IDs (e.g. SUPIs).

Editor’s Note:	Whether to extend the existing ML Preparation Flag for reuse or introduce a new flag dedicated for VFL preparation is FFS.

Editor’s Note:	The specific service name for the untrusted AF case is to be determined during the normative phase.

- VFL client(s) checks if it can meet the ML Model training requirement and decides whether to join the VFL process based on operator policy and/or implementation. Example criteria used by VFL client(s) may be based on whether it can satisfy the sample alignment requirement, its data availability, and time availability, and ML Model Interoperability information.

  - VFL client(s) invokes directly Nnwdaf_MLModelTraining_Notify, Nnwdaf_MLModelTraining_Subscribe response, or Nnwdaf_MLModelTrainingInfo_Request response service operation for a trusted AF, or  indirectly via the NEF Nnef_VFLPreparation_Subscribe response service operation for an untrusted AF to indicate. This is to indicate to the VFL server whether it will join the VFL process. The response may include a reason if it cannot joint the VFL process.

- VFL server determines the final VFL client(s) to be involved in the VFL procedures based on the information received from step 6 of the previous discovery procedure and other information available from step 3.

END CHANGE