3GPP TSG-SA2 Meeting #164	S2-2408041

# Maastricht, Netherlands, 19 August - 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1150 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: Training and inference procedures for VFL among multiple NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following conclusion with NOTEs in TR 23.700-84 clause 8.2 needs to be addressed: P#2.1.5: 5GC shall support vertical federated learning (VFL), i.e. a federated learning technique without exchanging/sharing local data set or ML models, in the following scenarios: - VFL among NWDAFs in a single PLMN. P#2.4.1: Either the NWDAF or the AF can act as VFL server and initiate VFL training process with the VFL client(s). P#2.4.3: An identifier is allocated by a VFL server, which is used to correlate the participants during the VFL training and subsequent VFL inference processes and it is associated with the distributed ML Models in the VFL joint model training process. P#2.4.4: VFL Clients compute the intermediate results for their local ML models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server. P#2.4.6: An AF or NWDAF acting as VFL server aggregates intermediate results from VFL client(s), trains a local model, computes intermediate results based on its local ML model, and sends the intermediate results towards VFL clients involved in the joint VFL training process. P#2.4.7: VFL server may compute different intermediate training information (e.g. gradient information, loss information) for updating its own local model and the models of VFL clients during the VFL training process after processing the received intermediate results (that may include convergence reports), sends the updates to the VFL client(s), and the VFL server/client(s) update their local ML model based on the received information. The VFL server determines when the VFL training process terminates, then it will inform the VFL Clients that the training ends. NOTE 8: How the initial model is provided to VFL clients is to be discussed in normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.3 is updated to include VFL among multiple NWDAFs. New clause 6.2X is addded to address the above conclusion, including the training and inference procedures for VFL among multiple NWDAFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conclusions agreed in TR 23.700-84 (AIML_CN) KI#2 cannot be fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3, 6.2X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.3	Federated Learning (FL) among multiple NWDAFs

Federated learning among multiple NWDAFs is a machine learning technique in core network that trains an ML Model across multiple decentralized entities holding local data set, without exchanging/sharing local data set. This approach stands in contrast to centralized machine learning techniques where all the local datasets are uploaded to one server, thus allowing to address critical issues such as data privacy, data security, data access rights.

NOTE 1:	Horizontal Federated Learning (HFL) is supported among multiple NWDAFs, which means the local data set in different FL client NWDAFs have the same feature space for different samples (e.g. UE IDs).

NOTE 2: Vertical Federated Learning (VFL) may be also supported among multiple NWDAFs. In VFL, the local datasets in different FL client NWDAFs have different feature spaces for the same samples (e.g. UE IDs).

For Federated Learning supported by multiple NWDAFs containing MTLF, there is one NWDAF containing MTLF acting as FL server (called FL server NWDAF for short) and multiple NWDAFs containing MTLF acting as FL client (called FL client NWDAF for short), the main functionality includes:

FL server NWDAF:

-	discovers and selects FL client NWDAFs to participant in an FL procedure

-	requests FL client NWDAFs to do local model training and to report:

- Local model information for HFL.

- Intermedidate results (such as local model outputs and corresponding gradients) for VFL.

-	generates global ML Model by aggregating local model information from FL client NWDAFs for HFL.

-	aggregate intermediate results directly from FL client NWDAFs for VFL without additional training at the FL server NWDAF, or aggregate intermediate results based on local training outcomes at the FL server NWDAF along with the intermediate results from FL client NWDAFs.

-	sends the global ML Model back to FL client NWDAFs for HFL to perform an additional training iteration if needed.

-	send the gradients for each participcant back to FL client NWDAFs for VFL.

FL client NWDAF:

-	locally trains ML Model as tasked by the FL server NWDAF with the available local data set for HFL and/or VFL, which includes the data that may not be allowed to be shared with other FL client NWDAFs due to e.g. data privacy, data security, data access rights.

-	reports the trained local ML Model information to the FL server NWDAF for HFL, while reporting intermediate results to the FL server NWDAF for VFL.

-	receives the global ML Model or gradients from FL server NWDAF for HFL and/or VFL, and perform an additional training iteration if needed.

FL server NWDAF or FL client NWDAF register to NRF with their FL capability information as described in clause 5.2.

The NWDAF containing MTLF determines to train an ML Model either based on local configuration or when it receives a request from NWDAF containing AnLF . The NWDAF containing MTLF further determines whether the ML Model should be trained via FL mechanism based on Analytic ID, Service Area/DNAI or when data can not be obtained directly from data producer NF (e.g. due to data privacy, data security). The NWDAF containing AnLF is not aware whether the ML Model is trained based on FL or not.

If the NWDAF containing MTLF can act as an FL server for the ML Model training, then FL procedure is initiated by the NWDAF containing MTLF as FL server NWDAF directly.

If the NWDAF containing MTLF determines to train an ML Model based on local configuration and the FL mechanism is required, but the NWDAF containing MTLF can't act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model.

If the ML Model training is triggered by the request from NWDAF containing AnLF, the NWDAF containing MTLF determines the FL mechanism is required but it can not act as an FL server, the NWDAF containing MTLF should discover an FL server NWDAF as described in clause 5.2 and request the FL server NWDAF to provide the trained ML Model as described in clause 6.2C.2.2. The Notification Target Address and the Notification Correlation ID from the NWDAF containing AnLF is provided in the request message sent to the FL server NWDAF. The FL server NWDAF may determine to initiate FL procedure before providing the ML Model. The FL server NWDAF sends the ML Model information to the notification endpoint (e.g. the NWDAF containing AnLF) after the ML Model training success.

NOTE 2:	The security procedure on authorizating FL server to initiate FL procedure on the FL client(s) is described in Annex X, clause X.9 of TS 33.501 [49]. The security procedure authorizing an MTLF to request ML Models on behalf of an AnLF to another MTLF (e.g., FL server NWDAF) is described in Annex X, clause X.10 of TS 33.501 [49].

Before FL procedure is initiated by FL server NWDAF, appropriate FL client NWDAFs should be discovered by FL server NWDAF as described in clause 5.2.

When starting an FL procedure, the FL server NWDAF is to provide an initial model to each FL client NWDAF, and then each FL client NWDAF is to perform local model training using its local data set. The detailed procedure for FL among Multiple NWDAFs is described in clause 6.2C.

NEXT CHANGE

6.2X	Vertical Federated Learning

6.2X.1	General

This clause specifies how the training and/or inference processes by the vertical federated learning (VFL) technique can be performed in the following scenarios:

-	AF-initiated VFL between AF and NWDAF(s) within a single PLMN.

-	NWDAF-initiated VFL among NWDAFs or AF and NWDAF(s) within a single PLMN.

6.2X.2 NWDAF-initiated VFL among multiple NWDAFs

6.2X.2.1 General training procedure for VFL among multiple NWDAFs

This clause specifies the training procedures for NWDAF-initiated VFL scenarios among NWDAF within a single PLMN.

Figure 6.2X.2.1-1: General training procedure for NWDAF-initiated VFL among Multiple NWDAF

0.	The consumer (NWDAF containing AnLF) sends a subscription request to VFL server NWDAF to retrieve an ML Model, using Nnwdaf_MLModelProvision including Analytics ID and desired ML Model metric (e.g. ML Model Accuracy).

NOTE 1:	The ML Model Accuracy threshold can be used to indicate the target ML Model Accuracy of the training process and the VFL server NWDAF may stop the training process when the ML Model Accuracy threshold is achieved during the training process.

If the consumer (i.e. the NWDAF containing AnLF) provides the Time when the ML Model is needed, the VFL Server NWDAF can take this information into account to decide the maximum response time for its VFL Client NWDAF(s).

1.	VFL Server NWDAF selects NWDAF(s) containing MTLF (i.e. VFL Client NWDAF(s)) as described in the previous clauses for the discovery and preparation (including sample and feature alignment) procedures.

2.	VFL Server NWDAF sends a Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request to the selected NWDAF(s) containing MTLF (i.e. VFL Client NWDAF(s)), which participate in the Vertical Federated learning to perform the local model training and derive the local intermediate results based on the input parameter in the request from VFL Server NWDAF. The request includes interested UE IDs to indicate the target samples for training and feature profile to indicate which feature are used for training, the desired ML Model metric, and initial ML Model to configure the local model for the VFL client NWDAF(s), VFL model correlation ID used to correlate the training processes at the VFL client NWADF(s) during the training and subsequent inference processes, and also includes the maximum response time. The VFL Client NWDAF has to report the local intermediate results to the FL Server NWDAF before the maximum response time elapses.

3.	[Optional] Each VFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the VFL Client NWDAF has not local data available already.

4.	During Vertical Federated Learning training procedure, each VFL Client NWDAF trains the ML Model based on its local data to report the local intermediate results to the VFL Server NWDAF.

5.	Each VFL Client NWDAF sends Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTrainingInfo_Request response to the VFL Server NWDAF, including the local intermediate results and the VFL model correlation ID.

The local intermediate results, which are sent from the VFL Client NWDAF(s) to the VFL Server NWDAF during the FL training process, are the information for the VFL Server NWDAF to aggregate the local intermediate training results and generate the intermediate training information (e.g. gradient information, loss information) for updating its local model and local models at each VFL Client NWDAF.

If the VFL Client NWDAF is not able to complete the training of the local VFL Model within the maximum response time provided by the VFL Server NWDAF, the VFL Client NWDAF shall send the Delay Event Notification that include the delay event indication, an optional cause code (e.g. local ML Model training failure, more time necessary for local ML Model training) and the expected time to complete the training if available to the VFL Server NWDAF before the maximum response time elapses.

6.	The VFL Server NWDAF aggregates all the local intermediate training results to generate the intermediate training information (e.g. gradient information, loss information) based on the VFL model correlation ID and the label stored in the VFL server NWDAF for updating its local model and the local models at each VFL Client NWDAF.

If the VFL Server NWDAF provides the maximum response time for the VFL Client NWDAF(s) to provide the local intermediate results in step 2, the VFL Server NWDAF decides either to wait for the VFL Client NWDAF(s) which have not yet provided their local intermediate results within the new maximum response time or to aggregate only the retrieved local intermediate results to update its local model and the local models at each VFL Client NWDAF.. The VFL Server NWDAF makes this decision, considering the notification/response from the VFL Client NWDAF or, if the notification is not received, based on local configuration.

7.	[Optional] Based on the consumer request in step 0, the VFL Server NWDAF sends a Nnwdaf_MLModelProvision_Notify message to update the VFL training status to the consumer periodically or dynamically when some pre-determined status is achieved (e.g. the ML Model Accuracy threshold is achieved or training time expires).

8.	[Optional] The consumer decides whether the current model can fulfil the requirement, e.g. ML Model metric value is satisfactory for the consumer and determines to stop or continue the training process. The consumer re-invokes Nnwdaf_MLModelProvision_Subscribe service operation as used in step 0 to continue the training process or invokes Nnwdaf_MLModelProvision_Unsubscribe service operation to stop the training process.

9.	[Optional] Based on the subscription request sent from the consumer in step 8, the VFL Server NWDAF updates or terminates the current FL training process.

If the FL Server NWDAF received a request in step 6b to stop the Vertical Federated Learning Training process, steps 10 and 11 are skipped.

10.	If the VFL procedure continues, VFL Server NWDAF sends Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request that includes the the intermediate training information and VFL model correlation ID to selected FL Client NWDAF(s) for next round of Vertical Federated Training.

11.	Each VFL Client NWDAF updates its own ML Model based on the intermediate training information distributed by the VFL Server NWDAF at step 7.

NOTE 3:	The steps 3-8 should be repeated until the training termination condition (e.g. maximum number of iterations, or the result of loss function is lower than a threshold) is reached.

When the Vertical Federated Training procedure is complete, the VFL Server NWDAF requests the VFL client NWDAF(s) to terminate the VFL procedure by invoking Nnwdaf_MLModelTraining_Unsubscribe service with a cause code that the VFL process has finished. Then the VFL client NWDAF(s) terminate the local model training.

12.	After the training process is complete, the VFL Server NWDAF may send Nnwdaf_MLModelProvision_Notify that includes the model training completion indication to the consumer.

6.2X.2.2 General inference procedure for VFL among multiple NWDAFs

This clause specifies the inference procedures for NWDAF-initiated VFL scenarios among NWDAF within a single PLMN.

Figure 6.2X.2.2-1: General inference procedure for NWDAF-initiated VFL among Multiple NWDAF

0.	An analytics consumer sends a analytics subscription or request to VFL server NWDAF to retrieve analytics result, using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request including Analytics ID (e.g. Service Experience), Target of Analytics Reporting (e.g. UE IDs), Anlytics Filter Information (e.g. Application ID, S-NSSAI, DNN, Area of Interest, etc), Analytics Reporting Information (e.g. Analytics target period).

1.	Based on the requested analytics ID, the VFL Server NWDAF determines that VFL inference is required among multiple NWDAFs and then checks the associated VFL model correlation ID.

2.	The VFL server NWDAF sends the VFL inference request to the VFL Client NWDAFs, which were selected during the training procedure. The request includes the UE IDs used for inference, VFL model correlation ID to identify the associated ML Model(s) assigned to the VFL client NWADF(s) during the training.s

Editor’s Note: The details of the parameters included in the request are FFS.

3.	Each VFL Client NWDAF collects its local data by using the current mechanism in clause 6.2 if the VFL Client NWDAF has not local data available already.

4.	Based on the received VFL model correlation ID, each VFL Client NWDAF selects the local model and perform inference to generate local intermediate inference results.

5.	Each VFL Client NWDAF responds to the VFL Server NWDAF including local intermediate inference results and the VFL model correlation ID.

6.	Based on the VFL model correlation ID, the VFL Server NWDAF aggregates all the local intermediate inference results to generate consolidated inference results.

7.	The VFL Server NWDAF provides analytics result to the analytics consumer, using Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

END CHANGE