3GPP SA WG2 Meeting #164	 S2-2408052

# Maastricht, NL, 19-23 August 2024											                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1422 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6: Support of Standalone IMS Data Channel feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of features to support " Standalone IMS Data Channel Sessions " based on the conclusions for KI#6 in TR 23.700-77 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution add a new chapter to TS 23.228 based on the conclusions for KI#6 in TR 23.700-77. - Add a General description for IMS data channel. - Add a procedure to depict how to establish an IMS session with only standalone bootstrap DC for downloading application list and applications from DCSF; - Add a procedure to depict how to establish an IMS session with only standalone bootstrap DC with peer UE; - Add a procedure to depict how to establish an Combination of standalone bootstrap DC and application DC For the problem of when to alert the user on the MT UE during the standalone application DC establishment procedure, it is proposed to alert the user when the DC application has been downloaded by the MT UE. This is because the binding information (APP ID) in the SIP INVITE message may not be human-readable, and the user may not know what the DC application is used for or what permissions are required by the DC application. This information can only be obtained when the DC application is downloaded from the DCSF. It is better to show detailed information to the user so that the user can make the right decision regarding whether to accept the application call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed " Support of Standalone IMS Data Channel Sessions " is not in the TS 23.228. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | AC.7.X(new), AC.7.X.1(new) , AC.7.X.2(new) , AC.7.X.3(new) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text)* * *

AC.7.X	Standalone IMS Data Channel Sessions

AC.7.X.1 General

Standalone IMS data channel sessions are the IMS sessions only have data channel media without accompanying audio/video/messaging media.

The following standalone IMS data channel sessions shall be supported

-	Standalone bootstrap DC;

-	Combination of standalone bootstrap DC and application DC.

AC.7.X.2 Standalone boostrap DC

The following scenarios for IMS session with standalone bootstrap DC for downloading application list and applications from DCSF shall be supported:

A.	An IMS session between UE and DCSF with standalone bootstrap DC;

B.	An IMS session between two UEs with standalone bootstrap DC and further updated with application DC.

Figure AC.7.X.2-1 depicts a signalling flow diagram for scenario a.

AC.7.X.2-1: Standalone bootstrap DC set up procedure for scenario A

The steps in the call flow are as follows:

1.	Step 1 of clause AC.7.2.1 applies with the following differences:

-	The Request URI of the SIP INVITE message is a specific target URI, such as a specific PSI indicating standalone bootstrap DC.

-	The initial SDP offer of the SIP INVITE message contains only bootstrap data channel offers for UE#1 without any audio/video/messaging offers.

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure.

2-4.	Steps 2-4 of clause AC.7.2.1 apply.

5.	Step 5 of clause AC.7.2.1 applies with the following differences:

-	DCSF determines that UE#1 requests to establish a standalone bootstrap DC based on the value of the Called ID (i.e. the specific PSI) and the content of the SDP offer.

-	DCSF creates only the originating side media information.

6.	Step 6 of clause AC.7.2.1 applies with the following differences:

-	DCSF instructs the IMS AS to terminate the session.

7.	Step 7 of clause AC.7.2.1 applies.

8.	Step 8 of clause AC.7.2.1 applies with the following differences:

-	IMS AS instructs MF to allocate required data channel media resources for originating side only.

-	IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.2.1 apply.

11.	IMS AS terminates the IMS session based on the specific Request URI and sends 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The session with bootstrap data channel is established.

14.	The bootstrap data channels have been established between originating MF or MRF and UE#1. The UE#1 sends application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF or MRF.

15-16.	If the established bootstrap DC is not used for a period of time, UE#1 sends SIP BYE to release the bootstrap DC to save power.

Figure AC.7.X.2-2 depicts a signalling flow diagram for scenario B.

AC.7.X.2-2: Standalone bootstrap DC set up procedure for scenario B

The steps in the call flow are as follows:

1.	Step 1 of clause AC.7.2.1 applies with the following differences:

-	The initial SDP offer of the SIP INVITE message contains only bootstrap data channel offers for originating side, without any audio/video/messaging offers.

2-10.	Steps 2-10 of clause AC.7.2.1 apply.

11.	Steps 11-13 of clause AC.7.2.1 apply with the following differences:

-	The SIP INVITE message includes an updated SDP offer and an SDP early offer for bootstrap data channel for terminating side.

NOTE 3:	The SDP ealy offer is used for bootstrap DC because the bootstrap DC will be used to download DC application before 200 OK is received.

12-13.	UE#2 and terminating network returns a 183 response with an SDP answer and an SDP early answer for bootstrap data channel to originating network. According to the received SDP answer and SDP early answer, IMS AS may instruct MF or MRF to update data channel media resource information for UE#2.

14.	IMS AS sends the 183 response to UE#1, which includes SDP answer correspond to the SDP offer in step 1 and SDP early offer for bootstrap data channel for originating side.

15.	UE#1 sends a SIP PRACK message with an SDP early answer. IMS AS may instruct MF or MRF to update data channel media resource information for UE#1.

NOTE 4:	The content of the SDP ealy offer can be same as the SDP answer.

16.	UE#1 and UE#2 exchange SIP PRACK message and 200 OK for PRACK.

17-19. Steps 20-22 of clause AC.7.2.1 apply.

20.	The UE#1 can update the session to add application DC as specified in step 12-26 in clause AC.7.X.3.

NOTE 2:	To avoid the confusing for the user of UE#2, the UE#2 will not alert the user during the standalone bootstrap establishment procedure. The UE#2 will alert the user during the application DC establishment procedure, and the 200 OK from UE#2 is sent when the user of UE#2 accepte to use the DC application selected by UE#1.

AC.7.X.3 Combination of standalone bootstrap DC and application DC

An IMS session with a combination of standalone bootstrap DC and application DC is used to establish application DC for a downloaded application to the remote UE, which allows the remote UE to download the application if not yet downloaded after accepting the bootstrap DC, and update the session to use the application in the same session;

Figure AC.7.X.3-2 depicts a signalling flow diagram for the scenario of UE#2 does not download the DC application yet.

AC.7.X.3-1: Combination of Standalone Bootstrap DC and Application DC set up Signalling Procedure

The steps in the call flow are as follows:

1.	UE#1 sends the SIP INVITE request with an initial SDP offer to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP offer contains the bootstrap data channel and application data channel establishment request, and without any offers for audio/video/messaging media.

2.	DC routing decision and media source reservation is performed based on Step 2-10 of Figure AC.7.1-1 and Step 2-6 of Figure AC.7.2.1-1.

As specified in Figure AC.7.2.1-1, the DCSF determines that the added application data channel media descriptor of the SDP offer takes UE#2 as target endpoint and does not require anchoring on the local MF or MRF.

3.	IMS AS sends the SIP INVITE message to UE#2 via S-CSCF and terminating network, which includes the modified SDP offer and SDP early offer for bootstrap data channel.

NOTE 1:	The SDP ealy offer is used for bootstrap DC because the bootstrap DC will be used to download DC application before 200 OK is received.

4.	UE#2 returns a 183 response with the SDP answer and the SDP early answer for bootstrap data channel towards the originating network.

For the scenario of UE#2 has no corresponding DC Application stored, the UE#2 accepts the bootstrap data channel and set ports of application DC to zero in the SDP answer.

5.	The IMS AS sends the 183 response to UE#1 via the S-CSCF and originating P-CSCF.

6.	UE#1 sends the SIP PRACK message to the IMS AS via the P-CSCF and S-CSCF in the originating network. The SIP PRACK message contains an updated SDP offer for application data channel and the associated DC application binding information according to TS 26.114 [76]. The SIP PRACK message also contains an SDP early answer for bootstrap data channel.

7.	DC routing decision and media source reservation is performed based on Step 2-6 of Figure AC.7.2.1-1.

8.	IMS AS sends the SIP PRACK message with updated SDP offer for application data channel to the UE#2 via originating S-CSCF and the terminating network.

9.	After the bootstrap data channels have been established between originating MF or MRF and UE#2, the UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already.

10.	UE#2 sends 200 OK for PRACK with SDP answer for application data channel to the UE#1 via terminating network, IMS AS, and originating network.

11.	UE#1 and UE#2 exchange SIP UPDATE message and 200 OK for UPDATE.

12.	UE#2 starts to play ringtone for the user of UE#2 after the application data channel is established and the DC application has been downloaded.

NOTE2:	UE#2 may show the DC application information to the user based on the downloaded DC Application. Such as the human readable name, the detail introduction, the required permissions etc. The detail of what information should be show to the user is out of scope of this specification.

13.	UE#2 returns a 180 Ringing response to originating network.

14. UE#2 returns a 200 OK response for INVITE to originating IMS AS if the user of UE#2 accept to use the DC application requested by UE#1.

15. IMS AS responds to the MediaInstruction request. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1.

16. The DCSF stores the media resource information and responds to the Notify Request.

17. IMS AS sends the 200 OK response for INVITE to UE#1

18. UE#1 sends ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established, and UE#1 can start to send application data to UE#2 through the application data channel.

NOTE 3:	This procedure covers only the scenario of UE#1 selects a DC application to communicated with UE#2, the bootstrap DC is established only with DCSF of UE#1. If the UE#2 want to add another DC application of UE#2, another bootstrap DC with DCSF of UE#2 may also be established by UE#2 by re-Invite message.

* * * End of Changes * * *