3GPP TSG-WG SA2 Meeting #164	S2-2408064

# Maastricht, NL, 19th Aug – 23rd Aug, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3800 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction to Split MME architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Novamint, Sateliot, NEC, Intel, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions agreed in the TR 23.700-29 for KI#2 this CR proposes to introduce S&F features. This CR proposes the Annex to describe the S&F architectures using split MME option. |  |  |  |  |  |  |  |  |
| Summary of change: |  | S&F feature is implemented based on the conclusions reached for KI#2 in TR 23.700-29. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S&F features is not completely described. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X(new), X.1(new), X.2(new), X.2.1(new), X.2.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

Annex X (informative): 
Store and Forward Satellite operation

# X.1	Introduction

The S&F Satellite operation with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. The support of S&F Satellite operation is especially suited for the delivery of delay-tolerant/non-real-time IoT satellite services with NGSO satellites.

An example of "S&F Satellite operation" is illustrated in Figure X.1-1, in contrast to what could be considered the assumption for the "normal/default Satellite operation" with satellite access.

As shown in Figure X.1-1:

-	Under "normal/default Satellite operation" mode, signalling and data traffic exchange between a UE with satellite access and the remote ground network requires the service and feeder links to be active simultaneously, so that, at the time that the UE interacts over the service link with the satellite, there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network.

-	In contrast, under "S&F Satellite operation" mode, the end-to-end exchange of signalling/data traffic is now handled as a combination of two steps not concurrent in time (step A and B in Figure X.1-1). In step A, signalling/data exchange between the UE and the satellite takes place, without the satellite being simultaneously connected to the ground network (i.e. the satellite is able to operate the service link without an active feeder link connection). In step B, connectivity between the satellite and the ground network is established so that communication between the satellite and the ground network can take place. So, the satellite moves from being connected to the UE in step A to being connected to the ground network in step B.

| "Normal/default Satellite operation" mode |  |
| --- | --- |
| "S&F Satellite operation" mode |  |

Figure X.1-1: Illustration of "normal/default operation" and "S&F Satellite operation" modes with satellite access

## X.2	Example S&F Satellite operation Architectures

### X.2.1	Split MME architecture

In this architecture option:

1)	eNB is onboard the satellite

1)	MME is split into two functions:

a)	MME-onboard:	the MME part which is onboard the satellite. MME-onboard is in charge of (1) handling the S1 interface with the onboard eNB and (2) terminating the NAS protocol signalling from/to UEs via the onboard eNB.

b)	MME-ground:	the MME part which is on the ground network. MME-ground is in charge of handling the rest of interfaces towards other CN functions (S6a towards HSS, SGd towards SMS-GMSC/IWMSC /SMS Router, T6a towards SCEF, T6ai towards IWF-SCEF, S11 towards SGW). One MME-ground instance can be connected to one or multiple MME-onboard instances via a Ssf reference point (see Figure X.2.1). The Ssf reference point is outside the scope of 3GPP.

2)	Rest of the core network elements (e.g. HSS, SGW, PGW, SCEF/IWF-SCEF, SMS-GMSC/IWMSC/SMS Router) are on the ground to support SMS and CP CIoT services. For supporting S1-UP and UP CIoT services, part of the SGW and PGW function should be deployed on board the satellite. For SMS service, to support immediate delivery of delivery report (i.e. RP-ACK) to the UE,  SMSC function is deployed on board the satellite.

Figure X.2.1-1: “Split-MME” architecture for supporting S&F satellite operation for SMS and CP CIoT services

The split-MME architecture has below principles:

1)	The Ssf reference point is outside the scope of 3GPP in this release of specification.

2)	The MME-ground is an anchor node situated in ground network which has the UE context. MME-ground is responsible to synchronize the UE context with all the MME-onboard(s). The MME-ground together with the set of MME-onboard instances deployed in the set of satellites behaves jointly as a single MME entity. Each MME-onboard instance is associated with a different Satellite ID identifier. The Satellite IDs are based on the SIB information broadcasted by eNB. The list of Satellite IDs are valid in the registration area (i.e. TAI list), if available.

3)	The eNB and MME-onboard are onboard the satellite. Thus they can provide basic connection management procedures. The MME-onboard executes all the procedures with the UE which does not need interaction with other core network nodes on the ground when the service link is available.

4)	Whenever a procedure needs an interaction with a core network node in the ground then MME-onboard (stores it as feeder link is not available) and synchronizes with the MME-ground when feeder link is available, the MME-ground executes the procedure with the ground network nodes and syncs back the UE context with the MME-onboard or sends back the response message to the MME-onboard.

5)	The MO data is stored in the MME-onboard when the service link is available and the feeder link is unavailable, and transferred to the ground when the feeder link becomes available. The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. The MT data is stored in the MME-onboard when the feeder link is available and service link is unavailable, and transferred to the UE when service link becomes available. All types of data traffic (e.g. IP, NIDD and SMS) can be supported and transferred using the existing user plane and control plane procedures defined in EPS.

6)	For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC). For this, some or full SMSC function is expected to be collocated with MME function onboard the satellite.

7)	The MME-onboard store the timestamp of interaction with the UE (called as Request Time). This is provided to the MME-ground when feeder link is available. The MME-ground  may indicate to HSS the "Request Time", allowing the HSS to check that no other (e.g., terrestrial) MME has sent an Update Location Request after the "Request Time". Further interactions between MME-ground and HSS are described in clause 4.13.X.

8)	The Mobile Reachable Timer (MRT) is run at the MME-ground, each time UE gets in connected mode with any of the MME-onboard, the MME-onboard informs the MME-ground, then MME-ground restart the MRT timer. The mobile reachable timer value has to be sufficiently large at network so that it takes into account delay in receiving information from the MME-onboard after UE has come to connected mode( including case of PTAU procedure).

9)	If S&F data quota is reached, configured in MME-onboard based on local policy, the MME-onboard may force a connection release and provide wait timer to the UE as described in clause 4.13.X.

### X.2.2	Full CN onboard

<Will be removed after agreeing structure of the CR during revisions>